3GPP TSG-RAN WG3 Meeting #115-e	R3-22XXX

E-meeting, 21 Feb- 3 Mar 2022

Title: 	(TP for NR_Slice for TS 38.413) Supporting network slicing enhancement

Source: 	ZTE

Agenda item:	17.3

Document Type:	Other

1. Introduction

This contribution provides TP to update TS 38.413 for supporting network slicing.

It is noted in the TS 23.501, the NG-RAN shall attempt to enforce Target NSSAI when the IE received. While it is not correct in current description of BLCR that the enforcement in NG-RAN node is optional.

The NG-RAN shall attempt to find cells of TAs that can support all the S-NSSAIs in the Target S-NSSAIs, and if no such cell of a TA is available the RAN can attempt to select cells of TAs that best match the Target S-NSSAI. The NG-RAN shall attempt to ensure continuity of the PDU Sessions with activated User Plane associated with the S-NSSAIs in the Allowed NSSAI which are in the Target NSSAI. Also, the NG-RAN should attempt to ensure continuity of service for the S-NSSAIs of the Allowed NSSAI also available in the Target NSSAI, before prioritizing cells that are not supporting one or more of the S-NSSAI of the Allowed NSSAI also available in the Target NSSAI.

Proposal : To correct description for Target NSSAI in Initial Context Setup and Downlink NAS Transport procedures.

2. TP for TS 38.413

Start Change

8.3.1	Initial Context Setup

8.3.1.1	General

The purpose of the Initial Context Setup procedure is to establish the necessary overall initial UE context at the NG-RAN node, when required, including PDU session context, the Security Key, Mobility Restriction List, UE Radio Capability and UE Security Capabilities, etc. The AMF may initiate the Initial Context Setup procedure if a UE-associated logical NG-connection exists for the UE or if the AMF has received the RAN UE NGAP ID IE in an INITIAL UE MESSAGE message or if the NG-RAN node has already initiated a UE-associated logical NG-connection by sending an INITIAL UE MESSAGE message via another NG interface instance. The procedure uses UE-associated signalling.

For signalling only connections and if the UE Context Request IE is not received in the Initial UE Message, the AMF may be configured to trigger the procedure for all NAS procedures or on a per NAS procedure basis depending on operator’s configuration.

8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Initial context setup: successful operation

In case of the establishment of a PDU session the 5GC shall be prepared to receive user data before the INITIAL CONTEXT SETUP RESPONSE message has been received by the AMF. If no UE-associated logical NG-connection exists, the UE-associated logical NG-connection shall be established at reception of the INITIAL CONTEXT SETUP REQUEST message.

The INITIAL CONTEXT SETUP REQUEST message shall contain the Index to RAT/Frequency Selection Priority IE, if available in the AMF.

<Unchanged Text Omitted>

If the INITIAL CONTEXT SETUP REQUEST message contains the UE Radio Capability ID IE, the NG-RAN node shall, if supported, use it as specified in TS 23.501 [9] and TS 23.502 [10].

For each PDU session, if the PDU Session Expected UE Activity Behaviour IE is included in the INTIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall, if supported, handle this information as specified in TS 23.501 [9].

If the Target NSSAI Information IE is contained in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall ,if supported, use this information for redirection as specified in TS 23.501 [9].

<Unchanged Text Omitted>

Next of Change

8.6.2	Downlink NAS Transport

8.6.2.1	General

The Downlink NAS Transport procedure is used when the AMF only needs to send a NAS message transparently via the NG-RAN node to the UE, and a UE-associated logical NG-connection exists for the UE or the AMF has received the RAN UE NGAP ID IE in an INITIAL UE MESSAGE message or if the NG-RAN node has already initiated a UE-associated logical NG-connection by sending an INITIAL UE MESSAGE message via another NG interface instance.

8.6.2.2	Successful Operation

Figure 8.6.2.2-1: Downlink NAS transport

The AMF initiates the procedure by sending a DOWNLINK NAS TRANSPORT message to the NG-RAN node. If the UE-associated logical NG-connection is not established, the AMF shall allocate a unique AMF UE NGAP ID to be used for the UE and include that in the DOWNLINK NAS TRANSPORT message; by receiving the AMF UE NGAP ID IE in the DOWNLINK NAS TRANSPORT message, the NG-RAN node establishes the UE-associated logical NG-connection.

<Unchanged Text Omitted>

If the End Indication IE is included in the DOWNLINK NAS TRANSPORT message and set to "no further data", the NG-RAN node shall consider that besides the included NAS PDU in this message, there are no further NAS PDUs to be transmitted for this UE.

If the DOWNLINK NAS TRANSPORT message contains the UE Radio Capability ID IE, the NG-RAN node shall, if supported, use it as specified in TS 23.501 [9] and TS 23.502 [10].

If the Target NSSAI Information IE is contained in the DOWNLINK NAS TRANSPORT message, the NG-RAN node shall, if supported, use this information for redirection as specified in TS 23.501 [9].

<Unchanged Text Omitted>

End of Change