3GPP TSG-RAN WG3 #115-e		R3-222206

21th Feb – 3rd Mar 2022 E-Meeting

Source:	CATT

Title:	Discussion on NR QoE configuration details

Agenda Item:	15.2.1.2

Document for:	Discussion

- Introduction

In last RAN3 meeting, most of NR QoE configuration details related issues have been solved, and it seems only one FFS is left.

FFS: For RAN visible QoE metric reporting, there is no need to include slice ID.

This contribution will continue discussing open issues.

- Discussion

2.1 slice for RAN visible QoE

There is an issue on whether to include slice ID for RAN visible QoE metric reporting as below.

FFS: For RAN visible QoE metric reporting, there is no need to include slice ID.

In last RAN3 meeting, which ID shall be included in RAN visible QoE metric report has been discussed. We prefer PDU session ID to slice ID as different PDU session may have the same slice ID. Moreover, it is the common case for network improving Uu channel quality per DRB which is related to PDU session ID based on RAN visible QoE. So, we support to include PDU session ID in RAN visible QoE metric report.

Proposal 1: it is proposed to include PDU session ID instead of slice ID in RAN visible QoE metric report.

2.2 Overload handling

The open issue on whether there is a need to send pause/resume indication to MCT is as below:

- Whether a temporary stop and restart of QoE reporting should be indicated to OAM if such indication was sent to UE

There is a description in TS28.404 as below:

| Step 1 (M) | The RAN node sends a request to temporarily stop the reporting to the UEs that has started the QoE information collection. An indication about the temporary stop is sent to the management system. |
| --- | --- |
| Step 3 (M) | When the RAN overload situation is ended, the RAN node sends a request to restart the reporting to the UEs that has temporarily stopped the QoE information reporting. An indication about the restart is sent to the management system. |

Although TS28.404 is for LTE system, NR may be aligned with LTE. So, an indication about pause/resume may be needed to be sent to management system.

In last RAN3 meeting, some companies argue that it is not needed to inform OAM about a temporary stop every time because when UE is in RRC-IDLE or RRC-ACTIVE state, there is also a temporary stop occurs. In my opinion, it is the requirement raised by SA5 and we may clarify with SA5 about the scenarios when a temporary stop indication to management system is needed.

Proposal 2: There is a need to send the pause/resume indication to the MCE according to TS28.404. We need to clarify with SA5.

- Conclusion

According to the analysis in section 2, we have:

Proposal 1: it is proposed to include PDU session ID instead of slice ID in RAN visible QoE metric report.

Proposal 2: There is a need to send the pause/resume indication to the MCE according to TS28.404. We need to clarify with SA5.

- Reference

[1]