3GPP TSG-RAN WG3 #115-e                                                                            R3-222265

21th Feb -3rd Mar, Feburary, 2022

Agenda Item:	23.3

Source:	CMCC

Title:	Discussion on CP issue for SL relay

Document for:	Discussion, Decision

Introduction

In last RAN3 meeting, control plane procedure for L2 UE to Network had been discussed and achieve greatful progress, we list agreements and open issues here:

Agreements:

From protocol stack point of view, the termination point of Uu adaptation layer is located at gNB-DU

gNB-CU is responsible for the allocation of local ID of remote UE

Over F1, the remote UE is configured via the UE-associated F1AP messages for itself

The gNB-CU should send the updated local ID of remote UE to gNB-DU after initial access of remote UE

WA: the gNB-DU can include the gNB-DU F1AP UE ID of relay UE in the INITIAL UL RRC MESSAGE during the initial access of remote UE with local UE ID unique per relay UE.

The stage-2 TP can include the new flow charts for RRC establishment/reestablishment/resume for sidelink relay by considering the following aspects:

- Local ID allocation for remote UE via SUI of relay UE

- Configuration of relay UE Uu RLC CH for relaying remote UE’s SRB message before remote UE initial access

- INITIAL UL RRC MESSAGE & DL RRC MESSAGE enhancement (if any, related to Open issue 4)

- Remote UE context setup, including PC5 RLC channel related configuration, mapping configuration

- Relay UE context modification, including Uu/PC5 RLC channel related configuration

Note: the details can be addressed in phase 2 discussion

The UE context management procedures should be enhanced to include the Uu RLC channel and PC5 RLC channel related information, i.e.,

UE CONTEXT SETUP REQUEST message: Uu RLC channel to be setup list (for relay UE), PC5 RLC channel to be setup list (for relay UE and remote UE)

UE CONTEXT SETUP RESPONSE message: Uu RLC channel setup list (for relay UE), Uu RLC channel failed to setup list (for relay UE), PC5 RLC channel setup list (for relay UE and remote UE), PC5 RLC channel failed to setup list (for relay UE and remote UE)

UE CONTEXT MODIFICATION REQUEST message: Uu RLC channel to be setup/modified/release list (for relay UE), PC5 RLC channel to be setup/modified/release list (for relay UE and remote UE)

UE CONTEXT MODIFICATION RESPONSE message: Uu RLC channel setup/modified list (for relay UE), Uu RLC channel failed to setup/modified list (for relay UE), PC5 RLC channel setup/modified list (for relay UE and remote UE), PC5 RLC channel failed to setup/modified list (for relay UE and remote UE)

UE CONTEXT MODIFICATION REQUIRED message: Uu RLC channel required to be modified/release list (for relay UE), PC5 RLC channel required to be modified/release list (for relay UE and remote UE)

UE CONTEXT MODIFICATION CONFIRM message: Uu RLC channel modified list (for relay UE), PC5 RLC channel modified list (for relay UE and remote UE)

The additional information, e.g., QoS profile info (e.g., 5QI, priority, PDB, PER, etc), RLC mode, and control plane traffic type (FFS: SRB ID vs. priority), can be included when configuring PC5/Uu RLC CH from gNB-CU to gNB-DU.

gNB-CU determines the mapping configuration

F1AP message(s) are used to configure the mapping between DRB/SRB and Uu RLC Channel at the gNB-DU side

the UE associated F1AP message(s) of remote UE are used to configure the mapping between DRB/SRB and Uu RLC Channel at the gNB-DU

the mapping between DRB and Uu RLC Channel is configured at the granularity of GTP-U tunnel.

The following responsibility are defined for gNB-CU and gNB-DU, respectively, for sidelink relay:

gNB-CU’s responsibility:

- Local Remote UE ID allocation

- Remote UE and relay UE association and context maintenance

- Remote UE bearer mapping and multiplexing

- Relaying Uu/PC5 RLC channel management

- E2E QoS split management for relaying

- Dedicated thresholds for relay discovery

gNB-DU’s responsibility

- Uu adaptation layer (AL) support for CP/UP data

- Determine the RLC/MAC/PHY Configuration for the relaying Uu/PC5 RLC CHs of relay UE

- Dedicated resource pool for NR ProSe service (same as legacy)

Note: those aspects can be reflected by stage-3 TP.

The UE CONTEXT SETUP REQUEST message is enhanced to include local ID of remote UE for, e.g., inter-gNB-DU mobility

the INITIAL UL RRC MESSAGE can include the DU to CU RRC Container IE including the lower layer configurations of PC5 RLC channel and SRB1 PC5 RLC channel configuration for remote UE, FFS on how to deal with the mandatory IE, i.e., CellGroupConfig, in DU to CU RRC Container ID in this case.

Keep the original WA, i.e., the gNB-DU can include the gNB-DU F1AP UE ID of relay UE in the INITIAL UL RRC MESSAGE during the initial access of remote UE with local UE ID unique per relay UE ID.

The gNB-DU can include the local ID of remote UE in the INITIAL UL RRC MESSAGE during the initial access of remote UE

The control plane traffic type is indicated via SRB ID

Open issues:

FFS: the local ID of remote UE can be notified to gNB-DU before initial access of remote UE

Inclusion of Relay UE ID in INITIAL UL RRC MESSAGE?

Notification of local ID of remote UE before initial access?

Lower layer configuration for remote UE in INITIAL UL RRC MESSAGE TRANSFER message and indication of rejection to remote UE?

Impact caused by service continuity?

In this contribution, we will discuss L2 UE to network relay control plane procedure mentioned in open issues.

Discussion

Whether include Relay UE ID in INITIAL UL RRC MESSAGE

In last RAN3 meeting, we have the following WA.

WA: the gNB-DU can include the gNB-DU F1AP UE ID of relay UE in the INITIAL UL RRC MESSAGE during the initial access of remote UE with local UE ID unique per relay UE.

With the progress in RAN2, it is agreed that the size of local ID of remote UE is 8bits. That’s to say, gNB can guarantee the uniquencess of local ID of remote UE for one relay. So, that WA can be turned into agreement.

Proposal 1：Turning the WA into agreement.

WA: the gNB-DU can include the gNB-DU F1AP UE ID of relay UE in the INITIAL UL RRC MESSAGE during the initial access of remote UE with local UE ID unique per relay UE.

Notification of local ID of remote UE

As we agreed in last meeting, gNB-CU is responsible for the allocation of local ID of remote UE. The local ID of remote UE should be included in INITIAL UL RRC MESSAGE during initial access of remote UE, and gNB-CU sends the updated local ID of remote UE to gNB-DU after initial access of remote UE if needed.

The local ID of remote UE can be included in adaption layer header for SRB0 and other SRB/DRB transmission. So, relay UE should be notified the local ID of remote UE. However, it is still controversal for gNB-DU to get that information from gNB-CU.

Firstly, we identify the obvious motivation is make the mapping rule between relay UE, remote UE and local ID of remote UE, and the local ID is needed for gNB-DU to be included in the INITIAL UL RRC MESSAGE during initial access of remote UE. As mentioned by some companies, it is feasible for gNB-DU to get it from the SRAP header of SRAP PDU for SRB0 transmission. That seems acceptable for us. We just propose that gNB-CU notification is a straightforward way. Secondly, gNB configure SRB0 relaying uu RLC channel to relay UE during the RRC connection establishment procedure, as shown in step 5,6 and 7 in Fig.1. The local ID of remote UE is necessary for gNB-DU to map the SRB0 configuration to its subordicated remote UE. Furthurmore, if the local ID of remote UE is notified to gNB-DU before gNB-DU receive the SRB0 corresponding SRAP PDU, gNB-DU could check the verify the local ID in SRAP header. Considering the above aspects, the local ID of remote UE can be notified to gNB-DU before initial access of remote UE.

Proposal 2: Local ID of remote UE can be notified to gNB-DU before initial access of remote UE.

Fig.1 Remote UE Initial Access procedure

Lower layer configuration for remote UE in INITIAL UL RRC MESSAGE TRANSFER message

As discussed in CB for control plane, the remote UE should be configured with PC5 RLC channel for SRB1 transmission, which is realized by the SL-PHY-MAC-RLC-Config IE. So, SL-PHY-MAC-RLC-Config IE should be contained in INTIAL UL RRC MESSAGE TRANSFER message. Furthermore, admission control for remote UE on gNB-DU should be considered. For legacy procedure, the presence of DU to CU RRC Container IE is used to indicate the admission result. If the DU to CU RRC Container IE is not included in the INITIAL UL RRC MESSAGE TRANSFER, the gNB-CU should reject the UE under the assumption that the gNB-DU is not able to serve such UE. Similar mechanism can be reused, and to use presence of SL-PHY-MAC-RLC-Config IE to indicate the admission result for gNB-DU.

Proposal 3: SL-PHY-MAC-RLC-Config IE can be included in INITIAL UL RRC MESSAGE TRANSFER message.

Proposal 4: Presence of SL-PHY-MAC-RLC-Config IE can be used to indicate the admission result of gNB-DU.

Impact caused by service continuity

For path switch from direct to indirect, the relay UE information should be informed to remote UE with RRCReconfiguration message, which includes CellGroupConfig IE. So the gNB-CU should provide path switch configuration to gNB-DU, and gNB-DU includes that in CellGroupConfig container.

Proposal 5: Path switch configuration should be provided to gNB-DU for path switch from direct to indirect.

Conclusions

With the discussion above, the following proposals are achieved:

Proposal 1：Turning the WA into agreement.

WA: the gNB-DU can include the gNB-DU F1AP UE ID of relay UE in the INITIAL UL RRC MESSAGE during the initial access of remote UE with local UE ID unique per relay UE.

Proposal 2: Local ID of remote UE can be notified to gNB-DU before initial access of remote UE.

Proposal 3: SL-PHY-MAC-RLC-Config IE can be included in INITIAL UL RRC MESSAGE TRANSFER message.

Proposal 4: Presence of SL-PHY-MAC-RLC-Config IE can be used to indicate the admission result of gNB-DU.

Proposal 5: Path switch configuration should be provided to gNB-DU for path switch from direct to indirect.

Reference

- 3GPP RP-212819 " WID on NR sidelink relay".

- TR38.836, v17.0.0, Study on NR sidelink relay.