3GPP TSG-RAN WG3 #115-e                                                                               R3-222266

21th Feb -3rd Mar, Feburary, 2022

Agenda Item:	23.3

Source:	CMCC

Title:	 (TP for SL relay BLCR to 38.401) Control plane

Document for:	Discussion, Decision

- Introduction

In this contribution, we discuss the leftover issues on control plane, and attach TP for them.

- Discussion

Remote UE Initial Access procedure

For step 5, as we discussed and proposed in [1], local ID of remote UE can be notified to gNB-DU before initial access of remote UE.

For step 12 and 13, the SL-PHY-MAC-RLC-Config IE should be added in INITIAL UL RRC MESSAGE TRANSFER message to include PC5 RLC channel configuration at least for sidelink RLC for the initial access of remote UE.

For step 15, the relay UE establishes PC5 RLC channel for relaying of SRB1 over PC5 and establishes uu RLC channel for relaying of SRB1. The PC5 RLC channel and uu RLC channel can be configured with step 8 for relay UE, and PC5 RLC channel can be configured with step 14 for remote UE. So, the step 15 can not be performed earlier. Further more, we think step 15 should inclue remote UE behaviors.

For step 30, PC5 RLC channel and uu RLC channel for SRB2 and DRB of remote UE are prepared, which can be performed after the RRCReconfigurationComplete message. So, the step 30 can not be performed earlier.

Details for step 7,12,20,25 are not needed.

Remote UE RRC reestablishment procedure

Removing notes with same reasons as initial access procdure.

Remote UE RRC Inactive to other states

Removing notes with same reasons as initial access procdure.

Proposal 1: Approve the folowing TP.

- Conclusion

With the discussion above, the following proposal is achieved:

Proposal 1: Approve the folowing TP.

- Reference

[1] R3-222265, Discussion on CP issue for SL relay, CMCC, RAN3 #115-e

- Text Proposal for 38.401

8.x Overall procedures for L2 UE-to-Network Relay

8.x.1	Remote UE initial access

The signalling flow for remote UE Initial access is shown in Figure 8.x.1-1.

Figure 8.x.1-1: Remote UE Initial Access procedure

- The U2N Remote UE and the U2N Relay UE perform discovery procedure, and establish PC5 connection using NR ProSe procedure.

- The U2N Remote UE sends an RRCSetupRequest message to the U2N Relay UE via PC5 RLC Channel.

- The U2N Relay UE withholds the received RRC message and sends the SidelinkUEInformation message to the gNB-DU. Before that, if the U2N Relay UE is in RRC_IDLE/RRC_INACTIVE status, it should trigger the RRC establishment/resume procedure to enter RRC_CONNECTED status.

- The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the U2N Relay UE by encapsulating the SidelinkUEInformation message to gNB-CU, and gNB-CU allocates the local ID of U2N Remote UE.

- The gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message of the U2N Relay UE to gNB-DU, which contains the local ID of U2N remote UE. Such message may request the establishment of Uu RLC channel for SRB0.

- The gNB-DU sends the UE CONTEXT MODIFICATION RESPONSE message of the U2N Relay UE to gNB-CU

- The gNB-CU sends the DL RRC MESSAGE TRANSFER message of the U2N Relay UE to gNB-DU by encapsulating the RRCReconfiguration message, which contains the local ID allocated to the U2N Remote UE.

- The gNB-DU sends the RRCReconfiguration message to the U2N Relay UE to configure the local ID of the U2N Remote UE.

- The U2N Relay UE sends the RRCReconfigurationComplete message to gNB-DU.

- The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the U2N Relay UE by encapsulating the RRCReconfigurationComplete message to gNB-CU.

- After receiving the local ID of the U2N Remote UE, the U2N Relay UE sends the RRCSetupRequest message of the U2N Remote UE to gNB-DU. The local ID of the U2N Remote UE is conveyed in the SRAP header

- The gNB-DU sends the INITIAL UL RRC MESSAGE TRANSFER message to gNB-CU by encapsulating the RRCSetupRequest message of the U2N Remote UE. In addition, the local ID of the U2N Remote UE and SL-PHY-MAC-RLC-Config container which contains configuration of PC5 RLC channel is included in the INITIAL UL RRC MESSAGE TRANSFER message .

- The gNB-CU allocates a gNB-CU UE F1AP ID for the U2N Remote UE and generates a RRCSetup message towards the U2N Remote UE. The RRC message is encapsulated in the DL RRC MESSAGE TRANSFER message, and includes the configurations of PC5 RLC channel at least for the transmission of U2N Remote UE’s SRB.

- The gNB-DU sends the RRCSetup message to the U2N Remote UE via the U2N Relay UE.

- The gNB-CU and the U2N Relay UE perform relaying channel setup procedure. According to the configuration from gNB-CU, the U2N Relay establishes a PC5 RLC channel for relaying of SRB1 over PC5 and establishes Uu RLC channel for relaying of SRB1 towards gNB-DU.

- The U2N Remote UE sends the RRCSetupComplete message to the gNB-DU via the U2N Relay UE.

- The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

- The gNB-CU sends the INITIAL UE MESSAGE message to the AMF.

- The AMF sends the INITIAL CONTEXT SETUP REQUEST message to the gNB-CU.

- The gNB-CU sends the UE CONTEXT SETUP REQUEST message to establish the U2N Remote UE context in the gNB-DU.

- The gNB-DU sends the SecurityModeCommand message to the U2N Remote UE via U2N Relay UE.

- The gNB-DU sends the UE CONTEXT SETUP RESPONSE message of the U2N Remote UE to the gNB-CU, which contains the PC5 link configurations.

- The U2N Remote UE responds with the SecurityModeComplete message.

- The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

- The gNB-CU generates the RRCReconfiguration message and encapsulates it in the DL RRC MESSAGE TRANSFER message.

- The gNB-DU sends RRCReconfiguration message to the U2N Remote UE via the U2N Relay UE.

- The U2N Remote UE sends RRCReconfigurationComplete message to the gNB-DU via the U2N Relay UE.

- The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and send it to the gNB-CU.

- The gNB-CU sends the INITIAL CONTEXT SETUP RESPONSE message to the AMF.

- The gNB-CU establishes additional Uu RLC channels between the gNB-DU and the U2N Relay UE, and additional PC5 Uu RLC channels for the U2N Relay/Remote UE for the relay traffic of DRBs and SRBs belonging to the U2N Remote UE. Also, such step may configure the mapping between DRB/SRB and PC5/Uu RLC channel at the U2N Relay UE. This step may be performed earlier, e.g., right after step 11.

8.x.2	Remote UE RRC Reestablishment

The signalling flow for remote UE RRC Reestablishment is shown in Figure 8.x.2-1.

Figure 8.x.2-1: Remote UE RRC Reestablishment procedure

- The U2N Remote UE and the U2N Relay UE perform discovery procedure, and establish PC5 connection using NR ProSe procedure. This step may be omitted if PC5 connection was established.

- The U2N Remote UE sends an RRCSetupRequest message to the U2N Relay UE via PC5 RLC Channel.

3~10. The gNB-CU allocates the local ID of the U2N Remote UE if the U2N Relay UE does not have it. The details of those steps can be referred to Section 8.x.1.

- After receiving the local ID of the U2N Remote UE, the U2N Relay UE sends the RRCReestablishmentRequest message of the U2N Remote UE to gNB-DU.

- The gNB-DU sends the INITIAL UL RRC MESSAGE TRANSFER message to gNB-CU by encapsulating the RRCReestablishmentRequest message of the U2N Remote UE.

- The gNB-CU and U2N Relay UE perform relaying channel setup procedure. According to the configuration from gNB-CU, the U2N Relay UE establishes a PC5 RLC channel for relaying of SRB1 over PC5 and establishes Uu RLC channel for relaying of SRB1 towards gNB-DU. This step may be performed earlier, e.g., via steps 5~8.

14~23. The details of those steps can be referred to Steps 5~14 in Section 8.7. For L2 U2N relay, the RRC message(s) between the U2N Remote UE and the gNB-DU are relayed via the U2N Relay UE; Step18~19 may additionally perform the configurations of PC5 RLC channel(s)

- The gNB-CU establishes additional Uu RLC channels between the gNB-DU and the U2N Relay UE, and additional PC5 Uu RLC channels for the U2N Relay/Remote UE for the relay traffic of DRBs and SRBs belonging to the U2N Remote UE. Also, such step may configure the mapping between DRB/SRB and PC5/Uu RLC channel at the U2N Relay UE. This step may be performed earlier, e.g., right after step 11.

8.x.3	Remote UE RRC Inactive to other states

The signalling flow for remote UE from RRC Inactive to other states is shown in Figure 8.x.3-1.

Figure 8.x.3-1: Remote UE RRC Resume procedure

- The U2N Remote UE and the U2N Relay UE perform discovery procedure, and establish PC5 connection using NR Prose procedure. This step may be omitted if PC5 connection was established.

- The U2N Remote UE sends an RRCResumeRequest message to the U2N Relay UE via PC5 RLC Channel.

3~10. The gNB-CU allocates the local ID of the U2N Remote UE if the U2N Relay UE does not have it. The details of those steps can be referred to Section 8.x.1.

- After receiving the local ID of the U2N Remote UE, the U2N Relay UE sends the RRCResumeRequest message of the U2N Remote UE to gNB-DU.

- The gNB-DU sends the INITIAL UL RRC MESSAGE TRANSFER message to gNB-CU by encapsulating the RRCResumeRequest message of the U2N Remote UE.

- The gNB-CU and U2N Relay UE perform relaying channel setup procedure. According to the configuration from gNB-CU, the U2N Relay UE establishes a PC5 RLC channel for relaying of SRB1 over PC5 and establishes Uu RLC channel for relaying of SRB1 towards gNB-DU. This step may be performed earlier, e.g., via steps 5~8.

14~19. The details of those steps can be referred to Steps 5~10 in Section 8.6.2. For L2 U2N relay, the RRC message(s) between the U2N Remote UE and the gNB-DU are relayed via the U2N Relay UE; Step14~15 may additionally perform the configurations of PC5 RLC channel(s)

- The gNB-CU establishes additional Uu RLC channels between the gNB-DU and the U2N Relay UE, and additional PC5 Uu RLC channels for the U2N Relay/Remote UE for the relay traffic of DRBs and SRBs belonging to the U2N Remote UE. Also, such step may configure the mapping between DRB/SRB and PC5/Uu RLC channel at the U2N Relay UE. This step may be performed earilier, e.g., right after step 11.