3GPP TSG-RAN3 Meeting #115-e	R3-222279

Electronic meeting, 21 Feb – 3 Mar 2022

Agenda item:	15.2.2

Source: 	Samsung

Title: 	(TP for BL CR TS 38.423) Mobility support of NR QoE

Document for:	Other & Agreement

Introduction

This TP is to reflect below aspects,

- Change the IE type of QoE Measurement Status, we don’t think QoE Measurement Status (WA) is needed if it’s to reflect whether the QMC is configured or not, as the target gNB can know whether it’s configured or not by RRC Context. But we think the measurement status can reflect whether the QoE measurement is started or not for the alignment with MDT, so we propose to replace “ENUMERATED (ongoing, configured...) ” to “ENUMERATED (started, ended...)” in QoE Measurement Status IE.

- Introduce RVQoE Report Transfer message over XnAP, as we discussed in RVQoE discussion in R3-222280, the scenario of RVQoE reporting during or after handover exists, and we think the RVQoE report should be transferred to the gNB 1 from the gNB 2 for handover optimization.

- Replace “Trace Reference” to “Any available MDT” in CHOICE MDT Alignment Information IE. According to the discussion of the MDT-QoE alignment in R3-22228, we think the alignment of m-based MDT and s-based QoE can be supported, and it is the NG-RAN node to select the UE that meet the requirements for MDT, so the IE for m-based MDT should be any m-based MDT.

TP for BL CR to TS 38.423

<Start of change>

### 8.4.x	RVQoE Report Transfer

#### 8.4.x.1	General

The purpose of the RVQoE Report Transfer procedure is to transfer RVQoE report between NG-RAN nodes.

The procedure uses non UE-associated signalling.

#### 8.4.x.2	Successful Operation

Figure 8.4.x.2-1: RVQOE REPORT TRANSFER, successful operation

NG-RAN node1 initiates the procedure by sending the RVQOE REPORT TRANSFER message to NG-RAN node2. When receiving the message NG-RAN node2 shall use the RVQoE report included in the message for radio network optimization.

<Next change>

#### 9.1.3.X	RVQOE REPORT TRANSFER

This message is sent by NG-RAN node1 to NG-RAN node2 to transfer the RVQoE report triggered by NG-RAN node2, but received by NG-RAN node1.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI that configured the RVQoE (in NG-RAN node 2) | YES | ignore |
| MeasConfigAppLayerId | M |  | INTEGER(0..15) | As defined in TS 38.331 | YES | ignore |
| RAN visible QoE Report | M |  | OCTET STRING | As defined in TS 38.331[14] | YES | ignore |

<Next change>

### 9.2.3.b UE Application Layer Measurement Information

The IE defines the information about the QoE Measurement Collection (QMC) configuration.

Editor’s note: The presence of configuration container in the IE is FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Container for S-based Application Layer Measurement Configuration | O (WA) |  | OCTET STRING (SIZE(1..FFS)) | Contains the s-based QoE measurement configuration, see Annex L in [xx]. |
| QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405. It consists of MCC+MNC+QMC ID, where the MCC and MNC are coming with the trace activation request from the management system to identify one PLMN containing the management system, and QMC ID is a 3 bytes Octet String. |
| Measurement Configuration Application Layer ID | M |  |  | This IE indicates the identity of the application layer measurement configuration, as defined in TS 38.331 [10]. |
| Service Type | M |  | ENUMERATED (QMC for DASH streaming, QMC for MTSI, QMC for VR, ...) | This IE indicates the service type of QoE measurements. |
| QoE Measurement Status | M |  | ENUMERATED (start, end...) | Indicates whether the QoE measurement has been started or not. |
| CHOICE MDT Alignment Information | O |  |  | Indicates the MDT measurements with which alignment is required. |
| >S-based MDT |  |  |  |  |
| >>NG-RAN Trace ID | M |  | 9.2.3.97 | Indicates the MDT measurements with which alignment is required. |
| >M-based MDT |  |  |  |  |
| >>Any available MDT | M |  |  | Indicates NG-RAN node can select the UE for MDT measurement to align with QoE measurement if the UE satisfies the requirements of MDT |
| Measurement Collection Entity IP Address | O |  | Transport Layer Address 9.2.3.29 | The IP address of the entity receiving the QoE measurement report. |
| CHOICE Area Scope of QMC | O |  |  |  |
| >Cell based |  |  |  |  |
| >>Cell ID List for QMC |  | 1 .. <maxnoofCellIDforQMC> |  |  |
| >>>NR CGI | M |  | 9.2.2.7 |  |
| >TA based |  |  |  |  |
| >>TA List for QMC |  | 1 .. <maxnoofTAforQMC> |  |  |
| >>>TAC | M |  | 9.2.2.5 | The TAI is derived using the current serving PLMN. |
| >TAI based |  |  |  |  |
| >>TAI List for QMC |  | 1 .. <maxnoofTAforQMC> |  |  |
| >>>TAI | M |  | 9.2.3.20 |  |
| >PLMN based |  |  |  |  |
| >>PLMN List for QMC |  | 1 .. <maxnoofPLMNforQMC> |  |  |
| >>>PLMN Identity | M |  | 9.2.2.4 |  |
| S-NSSAI List | O | 0..1 |  |  |
| >S-NSSAI Item |  | 1 .. <maxnoofSNSSAIforQMC> |  |  |
| >>S-NSSAI | M |  | S-NSSAI 9.3.1.38 |  |

<End of change>