3GPP TSG-RAN3 Meeting #115-e	R3-222282

Electronic meeting, 21 Feb – 3 Mar 2022

Agenda item:	15.4

Source: 	Samsung

Title: 	(TP for BL CR to TS 38.401) Alignment of QoE and MDT

Document for:	Other & Agreement

Introduction

The TP in section 2 is to introduce the general procedure of the MDT alignment with QoE according to the discussion in R3-222281 (TP for BL CR to TS 38.473 and TS 38.463) Alignment of MDT and QoE.

TP for BL CR TS 38.401

<Start of change>

### 8.13.x	MDT alignment with QoE

The signalling flow for the MDT alignment with QoE in gNB-DU and gNB-CU-CP is shown in Figure 8.13.x-1

Figure 8.13.x-1	MDT alignment with QoE

- The UE sends a QMC start indication the gNB-CU-CP, and the QMC start indication includes the QoE reference and QoS flow ID associated with the started QMC.

2a. The gNB-CU-CP maps the QoS flow ID to DRB ID, if the MDT is not started, the gNB-CU-CP send the TRACE START message, including the QoE reference and the mapped DRB ID to gNB-CU-UP to activate MDT measurement, if the MDT is started already, the gNB-CU-CP send the BEAER CONTEXT MODIFICATION REQUEST message instead.

2b. The gNB-CU-CP maps the QoS flow ID to DRB ID, if the MDT is not started, the gNB-CU-CP send the TRACE START message, including the QoE reference and the mapped DRB ID to gNB-DU to activate MDT measurement, if the MDT is started already, the gNB-CU-CP send the UE CONTEXT MODIFICATION REQUEST message instead.

3a. The gNB-CU-UP performs the MDT measurement accordingly, and then send to MCE the MDT report including the QoE reference and the measurements relate to the indicated DRB received in step 2a.

3b. The gNB-DU performs the MDT measurement accordingly, and then send to MCE the MDT report including the QoE reference and the measurements relate to the indicated DRB received in step 2b.

4.  The MCE align the MDT measurements and the QoE measurements based the QoE reference and Trace ID.

<End of change>