3GPP TSG-RAN WG3 Meeting #115-e	R3-222294

21 Feb – 3 Mar 2022

Online

Agenda item:	14.2

Source:	Samsung

Title:	(TP to SCG BL CR TS38.401) Discussion on SCG activation and deactivation

Document for:	Discussion & Decision

1 Introduction

This paper provides the corresponding TP for TS38.401. Note that the words highlighted in blue are the added contents of these TPs while others are the message sequence change.

2 Discussions

As we discussed in [1], the following observations and proposals are provided.

Proposal 1: Add the general cause value and the specific cause value in F1AP as they have been added in Xn/X2AP spec.

Proposal 2: Introduce an indicator of ″SCG Activity″ for SCG (de)activation as the assisting information from DU.

Proposal 3: Enhance the UE context modification required procedure for CU-CP to obtain the assisting information from DU.

Observation 1: The legacy E1 inactivity procedure could not cover some basic SCG (de)activation scenarios without enhancement.

Proposal 4: Enhance the legacy E1 inactivity procedure for CU-CP to obtain the assisting information from CU-UP(s), since MN CU-CP needs the assisting information to make decision also.

Observation 2: The SCG status notification from CU-CP to CU-UP should be performed on both of MN and SN side.

Proposal 5: For SN addition with SCG (de)activation, SN-CU-CP notifies the SCG status to SN-CU-UP in Bearer Context Setup Request message. After MN gets the SN Addition Request Acknowledge message, MN-CU-CP notifies the SCG status to MN-CU-UP in Bearer Context Modification Request message.

Proposal 6: For MN initiated SN modification with SCG (de)activation, SN-CU-CP notifies the SCG status to SN-CU-UP in Bearer Context Modification Request message after SN-DU has modified the SCG status. After MN gets the SN Modification Request Acknowledge message, MN-CU-CP notifies the SCG status to MN-CU-UP in Bearer Context Modification Request message.

Proposal 7: For SN initiated SN modification with SCG (de)activation, SN-CU-CP notifies the SCG status to SN-CU-UP in Bearer Context Modification Request message after MN has confirmed the SCG status. After MN sends the SN Modification confirm message, MN-CU-CP notifies the SCG status to MN-CU-UP in Bearer Context Modification Request message.

References

[1] R3-222293_(TP to SCG BL CR of TS38.463) Discussion on open issues for SCG (de)activation.docx

[2] draft R3-221139 was 1033 CB # MRDC2_SCGActivation_Deactivation_phase 2_v2.doc

[3] RAN3_114-bis-e_Meeting_Report

Annex – TP to TS 38.401

--------------------------------Start of the First Change-----------------------------

8.4.x	SCG Deactivation and Activation

This clause gives the NR SCG deactivation and activation in EN-DC and MR-DC with NR SN given that the en-gNB and SgNB consists of a gNB-CU and gNB-DU(s).

Editor’s note: among the procedures listed in this section, when to perform the SCG status notification from CU-CP to CU-UP is FFS.

8.4.x.1	SN Addition with SCG Deactivation

Figure 8.4.x.1-1: SCG Deactivation in SN Addition procedure

1. The MN sends the SN addition Request towards the SN, indicates the request of SCG deactivation.

2. The SN-CU-CP sends BEARER CONTEXT SETUP REQUEST message to the SN-CU-UP to setup bearer context and notify the deactivated status of SCG.

3. The SN-CU-UP sends the BEARER CONTEXT SETUP RESPONSE message to the SN-CU-CP.

4. The SN-CU sends the UE CONTEXT SETUP REQUEST message to the SN-DU to setup UE context and indicate the request of SCG deactivation.

5. The SN-DU sends the UE CONTEXT SETUP RESPONSE message to the SN-CU.

6. The SN sends the SN addition Request Acknowledge towards the MN.

7. The MN-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message to the MN-CU-UP to notify the deactivated status of SCG.

8. The MN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the MN-CU-CP

8.4.x.2	SN Addition with SCG Activation

Figure 8.4.x.2-1: SCG Activation in SN Addition procedure

1. The MN sends the SN addition Request towards the SN, indicates the request of SCG activation.

2. The SN-CU-CP sends BEARER CONTEXT SETUP REQUEST message to the SN-CU-UP to setup bearer context and notify the activated status of SCG.

3. The SN-CU-UP sends the BEARER CONTEXT SETUP RESPONSE message to the SN-CU-CP.

4. The SN-CU sends the UE CONTEXT SETUP REQUEST message to the SN-DU to setup UE context and indicate the request of SCG activation.

5. The SN-DU sends the UE CONTEXT SETUP RESPONSE message to the SN-CU.

6. The SN sends the SN addition Request Acknowledge towards the MN.

7. The MN-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message to the MN-CU-UP to notify the activated status of SCG.

8. The MN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the MN-CU-CP

8.4.x.3	MN initiated SN Modification with SCG Deactivation

Figure 8.4.x.3-1: SCG Deactivation in MN initiated SN Modification procedure

1. SCG is activated.

2-5. The MN/SN CU-CP obtains the assisting information from MN-CU-UP/SN-CU-UP and SN-DU.

6. The MN sends the SN Modification Request towards the SN, indicates the request of SCG deactivation.

7. The SN-CU sends the UE CONTEXT MODIFICATION REQUEST message to the SN-DU to indicate the request of SCG deactivation.

8. The SN-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the SN-CU, indicates that the SCG is deactivated.

9. The SN-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP to notify the deactivated status of SCG.

10. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP

11. The SN sends the SN Modification Request Acknowledge towards the MN, indicates that the SCG is deactivated.

12. The MN-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message to the MN-CU-UP to notify the deactivated status of SCG.

13. The MN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the MN-CU-CP

8.4.x.4	MN initiated SN Modification with SCG Activation

Figure 8.4.x.4-1: SCG Activation in MN initiated SN Modification procedure

1. SCG is deactivated.

2-5. The MN/SN CU-CP obtains the assisting information from MN-CU-UP/SN-CU-UP and SN-DU

6. MN sends the SN Modification Request towards the SN, indicates the request of SCG activation.

7. The SN-CU sends the UE CONTEXT MODIFICATION REQUEST message to the SN-DU to indicate the request of SCG activation.

8. The SN-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the SN-CU, indicates that the SCG is activated.

9. The SN-CU-CP sends BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP to notify the activated status of SCG.

10. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP

11. The SN sends the SN Modification Request Acknowledge towards the MN, indicates that the SCG is activated.

12. The MN-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message to the MN-CU-UP to notify the activated status of SCG.

13. The MN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the MN-CU-CP

8.4.x.5	SN initiated SN Modification with SCG Deactivation

Figure 8.4.x.5-1: SCG Deactivation in SN initiated SN Modification procedure

1. SCG is activated.

2-5. The MN/SN CU-CP obtains the assisting information from MN-CU-UP/SN-CU-UP and SN-DU.

6. The SN-CU sends the UE CONTEXT MODIFICATION REQUEST message to the SN-DU to indicate the request of SCG deactivation.

7. The SN-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the SN-CU, indicates that the SCG is deactivated.

8. The SN sends the SN Modification Require towards the MN, indicates the request of SCG deactivation.

9. The MN sends the SN Modification Confirm towards the SN.

10. The SN-CU-CP sends BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP to notify the deactivated status of SCG.

11. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

12. The MN-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message to the MN-CU-UP to notify the deactivated status of SCG.

13. The MN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the MN-CU-CP

8.4.x.6	SN initiated SN Modification with SCG Activation

Figure 8.4.x.6-1: SCG Activation in SN initiated SN Modification procedure

1. SCG is deactivated.

2-5. The MN/SN CU-CP obtains the assisting information from MN-CU-UP/SN-CU-UP and SN-DU.

6. The SN-CU sends the UE CONTEXT MODIFICATION REQUEST message to the SN-DU to indicate the request of SCG activation.

7. The SN-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the SN-CU, indicates that the SCG is deactivated.

8. The SN sends the SN Modification Require towards the MN, indicates the request of SCG activation.

9. The MN sends the SN Modification Confirm towards the SN.

10. The SN-CU-CP sends BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP to notify the activated status of SCG.

11. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

12. The MN-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message to the MN-CU-UP to notify the activated status of SCG.

13. The MN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the MN-CU-CP

-------------------End of the Change-------------------