3GPP TSG-RAN WG3 Meeting #115 electronic	R3-222341

e-Meeting, 21th February – 3rd March 2022

Source:	vivo

Title:          Discussion on latency enhancement

Agenda Item:	19.3

Document for:	Discussion and Decision

- Introduction

In RAN2#116bis meeting, RAN2 discussed the pre-configured measurement gap and PRS processing window based on RAN1 LS [1][2] and made the following agreements. However, the overall procedures are not concluded, especially the activation/deactivation from LMF.

Pre-configured measurement gap

Agreements:

Proposal 5a:	A new UL MAC CE for positioning measurement gap activation and deactivation request is introduced.

Proposal 5b:	The new UL MAC CE for positioning measurement gap activation and deactivation request includes at least the ID of the pre-configured positioning measurement gap configuration for which the activation/deactivation is requested. Other parameter are FFS.

Proposal 5c (modified):	A new DL MAC CE for positioning measurement gap activation and deactivation command is introduced for positioning latency reduction.

Proposal 5d:	The new DL MAC CE for positioning measurement gap activation and deactivation command includes at least the ID of the pre-configured positioning measurement gap configuration which has been configured/activated by the gNB. Other parameter are FFS.

PRS processing window

Agreements:

Proposal 7:	The PRS processing window configuration is provided via RRCReconfiguration message. Whether PRS processing window configuration is provided per BWP or not is up to RAN1 to decide.

Proposal 8:	A new DL MAC CE for PRS Processing Window activation and deactivation command is introduced.

Proposal 9:	The new DL MAC CE for PRS Processing Window activation and deactivation command includes at least the ID of the pre-configured PRS Processing Window configuration, at least in the case when multiple PRS Processing Windows can be configured.

In this contribution, we further discuss the spec impact of pre-configured measurement gap and PRS processing window from RAN3 perspecitve.

- Discussion

RAN1 introduced the pre-configured measurement gap and PRS processing window to further reduce the latency of PRS measurement,

- The gNB may pre-configure the MG to the UE, and the LMF or UE may request to activate them via NRPPa message or MAC CE.

- The gNB may pre-configure the PRS processing window to the UE, and UE can process PRS and receive data (e.g., PDCCH/PDSCH/CSI-RS reception) in a configured PRS processing window after the LMF activate them via NRPPa message.

The related agreements made by RAN1 are as follows:

pre-configured measurement gap Agreement Support the following options (in the agreement made in RAN1#106-e) for a new mechanism of MG activation request for the purpose of positioning. - Option 2: by UE (via UCI or UL MAC CE) - Select only one of UCI and UL MAC CE in RAN1#106bis-e - Option 1: by LMF (via an NRPPa message) - Note: This is transparent to the UE Agreement Preconfiguration of MG(s) in RRC is supported from RAN1 perspective. - Each MG in the preconfiguration is associated with an ID - The information in the UL MAC CE for MG activation request by the UE can be one ID associated with the preconfiguration of the MG Send an LS to RAN2 and RAN3

PRS processing window Agreement The following options are supported subject to UE capability for priority handling of PRS when PRS measurement is outside MG. - Option 1: UE may indicates support of two priority states. - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 2: UE may indicate support of three priority states - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than PDCCH and URLLC PDSCH and higher priority than other PDSCH/CSI-RS - Note: The URLLC channel corresponds a dynamically scheduled PDSCH whose PUCCH resource for carrying ACK/NAK is marked as high-priority. - State 3: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 3: UE may indicate support of single priority state - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS Note: SSB is a separate issue. Agreement At least the following parameters for PRS processing window from the gNB to the UE are supported. - Starting slot - Periodicity - Duration/length - Cell and SCS information associated with the above parameters Discuss during the maintenance phase on the necessity of other parameters including but not limited to - Processing type (associated with the corresponding UE capability 1A/1B/2) - Band/CC-ID as needed depending on each scenario on which the PRS processing window is applied - The above cell and SCS information to determine where/when the PRS processing window is applied Note: Indication of processing type does not suggest UE indication of multiple capabilities among (1A/1B/2) is already supported, which is a separate discussion. Note: Some of the parameters above may not be mandatory for a PRS processing window Agreement The priority of PRS for UE supporting two priority states and three priority states can at least be indicated in RRC. Agreement For capability 1A as per working assumption made in RAN1#106-e, the DL signalings/channels in a per UE fashion (i.e. both across NR & LTE) inside the PRS processing window are dropped if the DL PRS is determined to be higher priority. For capability 1B as per working assumption made in RAN1#106-e, only the DL signalings/channels from a certain band inside the PRS processing window are dropped if the DL PRS is determined to be higher priority. Working assumption: Subject to UE capability, support PRS measurement outside the MG, within a PRS processing window, and UE measurement inside the active DL BWP with PRS having the same numerology as the active DL BWP. - Inside the PRS processing window, subject to the UE determining that DL PRS to be higher priority, support the following UE capabilities: - Capability 1: PRS prioritization over all other DL signals/channels in all symbols inside the window. - Cap. 1A: The DL signals/channels from all DL CCs (per UE) are affected. - Cap. 1B: Only the DL signals/channels from a certain band/CC are affected. - FFS: band or CC - Capability 2: PRS prioritization over other DL signals/channels only in the PRS symbols inside the window - A UE shall be able to declare a PRS processing capability outside MG. - FFS: Details of capability signalling (e.g., per UE or per band, etc.). Agreement PRS processing window request to the gNB by the LMF is supported from RAN1 perspective. - It is up to RAN3 to design the necessary information to be transferred in the NRPPa message. - Note: It is up to gNB to determine the usage of measurement gap or PRS processing window - Include it in the LS to RAN2 and RAN3. Agreement For PRS processing window configuration and indication, at least the following mechanism is supported - RRC (pre-)configuration for PRS processing window configuration and DL MAC CE activation for PRS processing window, respectively. Include it in the LS to RAN2 and request RAN2 to decide whether DL MAC CE is feasible for this indication.

The overall procedures were captured in [3] while not concluded, and there remain some open issues:

How does the gNB determine the pre-configured measurement gap?

The gNB is not aware of which UE will perform DL-PRS measurement, thus the pre-MG will not properly be allocated. In the RRC location measurement indication procedure, the UE sends some assistance data to assist the gNB to determine the corresponding matched gap configuration. Based on the same consideration, the LMF can send some essential information to the gNB to let the serving gNB knows in advance the time/frequency domain characteristics of PRS to be measured. And then the pre-configured MG configuration can be generated by gNB appropriately.

How does the LMF send the pre-configured measurement gap activation request?

The LMF is not aware of whether UE is configured with the pre-MG in the current procedure in [4]. In our view, at least the following options can be considered for LMF to activate the pre-MG.

- Option 1: The ID associated with the pre-MG, the precondition is that the pre-MG shall be sent to LMF after the pre-MG is sent to target UE.

- Option 2: UE associated assistance data in the LocationRequestInformation (i.e., positioning requirement, latency, Bandwidth that is needed to meet accuracy requirement)

- Option 3: UE associated assistance data similar to LocationMeasurementInfo (Rel-16 RRC MG request includes PRS time/frequency characteristics)

For Option 1 and 3, the LMF does not know the scheduling situation of UE and the configuration of the serving cell and active BWP, thus the pre-MG will not properly be activated.

Our preference is option 2 to send the essential information to the gNB to let the serving gNB determine the activation.

The coordination of pre-configured measurement gap and PRS processing window.

In our understanding, the PRS processing window is similar to the measurement gap. The main difference is that the UE still needs to receive the data transmission in the PRS processing window. Therefore, the detail schemes for the pre-configured gap are also applied to PRS priority. The LMF may send the common assistance data and activation request to the gNB and it is up to gNB implementation to determine the pre-configuration and activation of the measurement gap and/or PRS processing window.

Based on the above analysis, we suppose that the overall procedure is as below:

Figure 2 unified pre-configured MG/processing window procedure

1. The LMF sends the assistance information to gNB via NRPPa message to help with the pre-configured MG/PRS processing window configuration. The NRPPa message can be non-UE associated.

2-4. The gNB determines the pre-configured MG or/and PRS processing window configuration based on the assistance data and configures it to the UE. The pre-configuration is configured to the UE with potential positioning requirements and left to gNB implementation.

5. The Location Request arrives at the LMF. The LMF determines the PRS configuration and sends the PRS configuration to UE.

6. The LMF sends an NRPPa message to gNB including the PRS measurement characteristics. The NRPPa message is UE associated.

7. Based on the PRS configuration, the UE may send MAC CE to gNB to request activating the pre-configured MG or PRS processing window.

8. Based on Step 6 or Step 7, the gNB determines to configure a new measurement gap/PRS processing window or activate the pre-configured measurement gap/PRS processing window by the implementation.

9. The gNB then activates the MG/PRS processing window via DL MAC CE, or configures a new measurement gap/PRS processing window via RRC Reconfiguration message.

For the assistance information to determine the pre-configured MG in step 1, the gap parameters of pre-configuration need to try to align with the potential PRS configuration as possible, e.g., try to let potential PRS fall into gap duration. The LMF has each TRP’s PRS configuration information and determines the PRS configuration according to positioning QoS. The LMF can send the PRS configurations of related TRP to gNB in advance to assist the gNB to determine the pre-configuration parameters. The PRS configuration includes the time/frequency characteristics of PRS, i.e., periodicity/offset for time-domain per frequency layer.

In Step 6, the LMF can send the UE-associated PRS measurement characteristics and the QoS of positioning to the gNB to activate the pre-configured MG.

Based on the above procedure, we propose:

Proposal 1: Specify the NRPPa message including the assistance data from LMF to gNB to help with the configuration of pre-configured MG and PRS processing window, including:

- the time/frequency characteristics of PRS of related TRPs

Proposal 2: Specify the NRPPa message including the assistance data from LMF to gNB to help with the activation/deactivation of pre-configured MG and PRS processing window, including:

- the time/frequency characteristics of PRS of related TRPs

- the required QoS of positioning

- the suggested start time for measurement

- Conclusion

Proposal 1: Specify the NRPPa message including the assistance data from LMF to gNB to help with the configuration of pre-configured MG and PRS processing window, including:

- the time/frequency characteristics of PRS of related TRPs

Proposal 2: Specify the NRPPa message including the assistance data from LMF to gNB to help with the activation/deactivation of pre-configured MG and PRS processing window, including:

- the time/frequency characteristics of PRS of related TRPs

- the required QoS of positioning

- the suggested start time for measurement

- Reference

- R2-2200074	LS on latency improvement for PRS measurement with MG

- R2-2200089	LS on PRS processing window

- R2-2201815	Running 38.305 CR for Positioning WI on RAT dependent positioning methods