3GPP TSG-RAN WG3 #115-e                                                                              R3-222344

21 Feb - 3 Mar 2022

Online

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1679 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Semantic description update for the Name List of MDT M8 and M9 measurement configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Samsung, CATT, Lenovo, Motorola Mobility, China Unicom, China Telecom, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MDT_BT_WLAN-Core, TEI16 |  |  |  |  |  | Date: |  |  | 2022-2-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current specifications of Bluetooth/WLAN measurement configuration in S1AP/X2AP and RRC might result in the situation that no Name List is configured to UE, and UE would not report any Bluetooth/WLAN measurement results. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce additional description. For WLAN Measurement Configuration, adding a semantics description that WLAN Measurement Configuration Name List is present if WLAN Measurement Configuration IE is set to Setup. For Bluetooth Measurement Configuration, adding a semantics description that Bluetooth Measurement Configuration Name List is present if Bluetooth measurement Configuration IE is set to Setup. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has limited impact with the previous version of the specification. There is no ASN.1 impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MDT M8 M9 cannot be enforced in the UE when eNB receiving configuration without any Bluetooth/WLAN Measurement Configuration Name. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.134, 9.2.135 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.413 CR 1870 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Changes Start

9.2.134	Bluetooth Measurement Configuration

This IE defines the parameters for Bluetooth measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Bluetooth Measurement Configuration | M |  | ENUMERATED (Setup, …) |  |
| Bluetooth Measurement Configuration Name List |  | 0..1 |  | This IE is present if the Bluetooth Measurement Configuration IE is set to Setup. |
| >Bluetooth Measurement Configuration Name Item IEs |  | 1 .. <maxnoofBluetoothName> |  |  |
| >>Bluetooth Measurement Configuration Name | M |  | OCTET STRING (SIZE (1..248)) |  |
| BT RSSI | O |  | ENUMERATED (True, …) | In case of Immediate MDT, it corresponds to M8 measurement as defined in 37.320 [31]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofBluetoothname | Maximum no. of Bluetooth local name used for Bluetooth measurement collection, the maximum value is 4. |

9.2.135	WLAN Measurement Configuration

This IE defines the parameters for WLAN measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| WLAN Measurement Configuration | M |  | ENUMERATED (Setup, …) |  |
| WLAN Measurement Configuration Name List |  | 0..1 |  | This IE is present if the WLAN Measurement Configuration IE is set to Setup. |
| >WLAN Measurement Configuration Name Item IEs |  | 1 .. <maxnoofWLANName> |  |  |
| >>WLAN Measurement Configuration Name | M |  | OCTET STRING (SIZE (1..32)) |  |
| WLAN RSSI | O |  | ENUMERATED (True, …) | In case of Immediate MDT, it corresponds to M8 as defined in 37.320 [31]. |
| WLAN RTT | O |  | ENUMERATED (True, …) | For Immediate MDT, it corresponds to M9 as defined in 37.320 [31]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofWLANname | Maximum no. of WLAN SSID used for WLAN measurement collection, the maximum value is 4. |

End of  Change