3GPP TSG-RAN WG3 Meeting #115-e	R3-222384

# Online,  Feb 21st 2022 - March 3rd 2022

Agenda Item:	9.3.5.1.

Source:	Ericsson

Title:	Mobility Parameters Modification Range

Document for:	Discussion, Decision

# 1	Introduction

The Mobility Setting Change procedure enables an NG-RAN node to negotiate the handover trigger settings with a peer NG-RAN node controlling neighbouring cells. In the initial message of the procedure, two sets of Mobility Parameters are sent from NG-RAN node 1 to NG-RAN node 2, one in the NG-RAN node1 Mobility Parameters IE that refers to configuration change in NG-RAN node 1 cell, and a second in the NG-RAN node2 Proposed Mobility Parameters IE that refers to proposed configuration change in NG-RAN node 2 cell.

#### 9.1.3.22	MOBILITY CHANGE REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate adaptation of mobility parameters.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | reject |
| NG-RAN node2 Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | reject |
| NG-RAN node1 Mobility Parameters | O |  | Mobility Parameters Information 9.2.2.60 | Configuration change in NG-RAN node1 cell | YES | ignore |
| NG-RAN node2 Proposed Mobility Parameters | M |  | Mobility Parameters Information 9.2.2.60 | Proposed configuration change in NG-RAN node2 cell | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |

#### 9.2.2.60	Mobility Parameters Information

The Mobility Parameters Information IE contains the change of the Handover Trigger as compared to its current value. The Handover Trigger corresponds to the threshold at which a cell initialises the handover preparation procedure towards a specific neighbour cell. Positive value of the change means the handover is proposed to take place later.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Handover Trigger Change | M |  | INTEGER (-20 .. 20) | The actual value is IE value * 0.5 dB. |

In the Mobility Change Failure message, the NG-RAN node 2 can reply with a different set of parameters in the Mobility Parameters Modification Range IE, indicating the range of Handover Trigger Change values permitted by the NG-RAN node2 at the moment the MOBILITY CHANGE FAILURE message is sent.

#### 9.1.3.24	MOBILITY CHANGE FAILURE

This message is sent by the NG-RAN node2 to indicate to NG-RAN node1 that Proposed Mobility Parameters proposed by NG-RAN node1 were refused.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | ignore |
| NG-RAN node2 Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | ignore |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Mobility Parameters Modification Range | O |  | 9.2.2.61 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.2 |  | YES | ignore |

#### 9.2.2.61	Mobility Parameters Modification Range

The Mobility Parameters Modification Range IE contains the range of Handover Trigger Change values permitted by the NG-RAN node2 at the moment the MOBILITY CHANGE FAILURE message is sent.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Handover Trigger Change Lower Limit | M |  | INTEGER (-20 .. 20) | The actual value is IE value * 0.5 dB. |
| Handover Trigger Change Upper Limit | M |  | INTEGER (-20 .. 20) | The actual value is IE value * 0.5 dB. |

According to the specified parameters, the following use case is possible.

The figure above shows a case where NG-RAN node 2 receives from NG-RAN node 1 a proposal to change its handover troigger point of “+10”, namely of +5dB. The standard allows today that the proposal is rejected by NG-RAN node 2 and that NG-RAN node 2 can signal back to NG-RAN node 1 that the allowed change is [-20,..-10].

Is NG-RAN node 2 signals to NG-RAN node 1  that the Handover Trigger Change Lower Limit == -20 and the Handover Trigger Change Upper Limit == -10, this may be interpreted by NG-RAN node 1 as an indication that the current HO trigger point cannot be maintained by NG-RAN node 2 (as it is outside the range of allowed changes).

The latter seems incorrect, as a lower and upper limit to an handover trigger change should be “around” the current HO trigger point.

Proposal 1: Add a clarification in the semantic text for the Handover Trigger Change Lower Limit and the Handover Trigger Change Upper Limit.

Another possible inconsistency is  between the range of proposed values and the range of permitted values. If the NG-RAN node2 Proposed Mobility Parameters IE indicates a value of Handover Trigger Change of +10, this indicates a proposed change for NG-RAN node 2 cell of the Handover Trigger of +5 dB as compared to its current value. At the same time, the Mobility Parameters Modification Range IE may be sent with a value of Handover Trigger Change Upper Limit of +20 (corresponding to a maximum Handover Trigger Change of +10dB).

In this case, the NG-RAN node 2 should have replied with a Mobility Change Acknowledge message instead of a Mobility Change Failure message.

Examples of possible outcome according to current specification are shown in the figure below.

To have a consistent specification regarding the use of Mobility Change Acknowledge or Mobility Change Failure respectively as Successful Outcome and Unsuccessful Outcome of the Mobility Setting Change procedure, without any impact on ASN.1, we think it is enough to add a clarification in the procedural text or in the semantic description for the Mobility Parameters Modification Range IE. The purpose of the addition would be to clarify that the values of the Handover Trigger Change Lower Limit and Handover Trigger Change Upper Limit IEs included the Mobility Parameters Modification Range IE should be consistent with the value of the Handover Trigger Change IE included in the Mobility Parameters Information IE.

Proposal 2: Add a clarification in the procedural text for the Unsuccessful Operation of the Mobility Setting Change procedure.

# Conclusion

In this contribution we have proposed the following:

Proposal 1: Add a clarification in the semantic text for the Handover Trigger Change Lower Limit and the Handover Trigger Change Upper Limit.

Proposal 2: Add a clarification in the procedural text for the Unsuccessful Operation of the Mobility Setting Change procedure.

# Appendix – Drafted CR for XnAP

3GPP TSG-RAN WG3 Meeting # 115-e	R3-22xxxx

# Online,  Feb 21st 2022 - March 3rd 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | draftCR | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mobility Parameters Modification Range |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-02-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In a Mobility Setting Change procedure, a Mobility Change Failure message can be sent from NG-RAN node 2, even if the Mobility Parameters Modification Range IE contains a Handover Trigger Change Lower Limit or a Handover Trigger Change Upper Limit in the range of proposed parameters in the NG-RAN node2 Proposed Mobility Parameters IE. Clarify the range of possible values for Handover Trigger Change Lower Limit and Handover Trigger Change Upper Limit. Additionally, the range of values for the Handover Trigger Change Lower Limit and Handover Trigger Change Upper Limit includes values that are not applicable and that should not be used |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a clarification in the procedural text for the Unsuccessful Operation of the Mobility Setting Change procedure. Add a clarification in the semantic description for the validity of values for the Handover Trigger Change Lower Limit and Handover Trigger Change Upper Limit in in the Mobility Parameters Modification Range IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A unsuccessful operation can be triggered for Mobility Setting Change procedure even if Handover Trigger Change Lower Limit or Handover Trigger Change Upper Limit in the Mobility Parameters Modification Range IE are set to values within the range of parameters included in the NG-RAN node2 Proposed Mobility Parameters IE. Specification is unclear w.r.t. possible values for Handover Trigger Change Lower Limit and Handover Trigger Change Upper Limit. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.9.3, 9.2.2.61 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------------------------------------- First Change ----------------------------------------------

#### 8.4.9.3	Unsuccessful Operation

Figure 8.4.9.3-1: Mobility Settings Change, unsuccessful operation

If the requested parameter modification is refused by NG-RAN node2, or if NG-RAN node2 is not able to complete the procedure, NG-RAN node2 shall send the MOBILITY CHANGE FAILURE message with the Cause IE set to an appropriate value. NG-RAN node2 may include the Mobility Parameters Modification Range IE in the MOBILITY CHANGE FAILURE message, for example in cases when the proposed change is out of the permitted range.

If the requested parameter modification is refused by NG-RAN node2, the NG-RAN node2 should set a value for Handover Trigger Change Upper Limit in the Mobility Parameters Modification Range IE lower than the value of Handover Trigger Change included in the Mobility Parameters Information IE.

If the requested parameter modification is refused by NG-RAN node2, the NG-RAN node2 should set a value for Handover Trigger Change Lower Limit in the Mobility Parameters Modification Range IE higher than the value of Handover Trigger Change included in the Mobility Parameters Information IE.

---------------------------------------- Next Change ----------------------------------------------

#### 9.2.2.61	Mobility Parameters Modification Range

The Mobility Parameters Modification Range IE contains the range of Handover Trigger Change values permitted by the NG-RAN node2 at the moment the MOBILITY CHANGE FAILURE message is sent.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Handover Trigger Change Lower Limit | M |  | INTEGER (-20 .. 20) | The actual value is IE value * 0.5 dB. Values from +1 to +20 are not allowed. |
| Handover Trigger Change Upper Limit | M |  | INTEGER (-20 .. 20) | The actual value is IE value * 0.5 dB. Values from -20 to -1 are not allowed. |

---------------------------------------- End of Changes ----------------------------------------------