3GPP TSG-RAN WG3 #115-e	R3-222394

# 21 Feb– 03 Mar2021

Online

Agenda Item:	9.3.2

Source:	Ericsson

Title:	SoD on CB: # 5_DynamicACL

Document for:	Discussion, Decision

# Introduction

CB: # 5_DynamicACL

- Discuss available options for E1?

- Whether query functionality in the S-NODE MODIFICATION REQUEST is needed?

- For mobility from SA to ENDC/MRDC, target MN may add the source node IP address and its IP address for direct data forwarding into the SN addition request message for ACL functionality?

- Try to close this topic, and provide CRs if agreeable

(E/// - moderator)

Summary of offline disc R3-222394

Structure of the discussion:

First round comments to be provided by Friday the 25th at 12UTC

Second Round to focus on TPs. Second round comments to be provided by Tuesday the 1st at 13UTC

# For the Chairman’s Notes

Support Option 2: Carrying the source SN’s IP address in SN ADDITION REQUEST ACKNOWLEDGE message.

Include the IP address of both target MN node and source NG-RAN node in the Xn: S-Node Addition Request message

Include the IP address of both target MN node and source eNB node in the X2: SGNB ADDITION REQUEST

The source IP@ contained in the BEARER CONTEXT SETUP RESPONSE message corresponds to the source IP address used by the source gNB-CU-UP to forward traffic to a target NG-RAN node

Transfer of source IP address over F1-C and W1-C serves the purpose of dynamic ACL for MN-terminated SCG bearers and SN-terminated MCG bearers.

Agree to R3-222108

Agree to R3-222783

Agree to R3-222784

Agree to R3-222762

Agree to R3-222763

# Discussion

During the past RAN3 meetings many agreements were taken on dynamic ACL.

The latest agreements and open issues identified at R3-114bis-e are as follows:

Per Qos flow level source IP address is transferred for ACL enhancement.

The gNB-CU-UP signals the source IP address to be used for data forwarding to the CU-CP.

The ACL enhancement for indirect data forwarding for NG/S1 handover is pursued in Rel-17. The agreement will be revisited after the reply LS from SA2 is received.

Support of per PDU session/Per DRB level source IP address transfer or both of them needs further clarification given that per Qos flow level is agreed.

There is no Qos framework impact. Double check with SA2 is not needed.

Scenario is down prioritized according to operator’s requirement. May revisit after SA2 reply LS is received plus stage 2 clarification.

To introduce query functionality in the S-NODE MODIFICATION REQUEST procedure?

ACL in SA to/from EN-DC/MR-DC?

Try to close this topic in RAN3#115e

## Discussion on query functionality

The options available to obtain the source IP address from a source SN are described as follows:

- Option 1: Introducing a query function in MN-initiated SN modification request message.

- Option 2: Carrying the source SN’s IP address in SN ADDITION REQUEST ACKNOWLEDGE message.

- Option 3: Carrying the source SN’s IP address in S-NODE MODIFICATION REQUEST ACKNOWLEDGE message without the need for a query indication.

[1] acknowledges the issue of additional delays cause by option 1) and goes in favour of option 2).

[2] also goes in favour of option 2).

[3] proposes to choose between Option 2) and Option 3).

[4] goes in favour of Option 1) to cover cases where the MN decides to change the PDU session from SN terminated to MN terminated. Namely, a query procedure is used for MN to get the source IP of the SN for data forwarded by the SN to MN. In [4], Option 2 seems to be also acceptable.

Companies are invited to provide their view on which of the above options are preferred

| Company | Comment |
| --- | --- |
| Ericsson | Option 2. Option 2 can cover also the case where the MN decides to change the PDU session from SN terminated to MN terminated. In this case the MN already has the source IP the SN will use for data forwarding, received at SN ADDITION REQUEST ACKNOWLEDGE |
| Deutsche Telekom | Option 2. We share Ericsson’s view. |
| Huawei | Agree, option 2 seems the best one. |
| CATT | Option 1. When SN node is first added, the SN node does not know whether the SN terminated PDCU session would be move back to MN node or not. So, the SN has to inform MN of the IP address of all SN terminated PDU session which maybe not needed for most of the time. With that, we think option 2 is not an optimal solution. |
| ZTE | Flexible for option 2 or 3. |
| Samsung | Option 2. |
| Nokia | Option 2 |

Conclusions:

6 companies prefer Option 2. Of these companies one also supports Option 3.

1 company supports Option 1

In light of the above the following is proposed:

Support Option 2: Carrying the source SN’s IP address in SN ADDITION REQUEST ACKNOWLEDGE message.

## Discussion ACL in SA to/from EN-DC/MR-DC

The following options have been outlined:

- The solutions agreed at the last RAN3 meeting and the solutions presented during RAN3-114bis-e cover also cases of mobility from SA to/from EN-DC/MR-DC

- In order to address scenarios of mobility from SA to EN-DC/MR-DC, to include the IP address of  both target MN node and source NG-RAN node in the S-Node Addition Request message

In [1] option 1) is proposed

[3] and [4] both propose option 2)

Companies are invited to provide their view on the preferred option

| Company | Comments |
| --- | --- |
| Ericsson | Option 1. Data forwarding will happen either from the source NG-RAN or from the target MN, hence there is no need to signal two source IP addresses. By including a single source IP in the S-Node Addition Request, it is possible to signal either the source IP of the source NG-RAN or the source IP of the target MN. After further evaluation we have realized that Option 2 is the best option |
| Deutsche Telekom | Option 1. Only a single source IP address is needed to cover those mobility cases. |
| Huawei | Option1. Based on our analysis, there is no need to forward two IP addresses in those cases. |
| CATT | Option 2.Because option 1 could not work. For SA to DC handover, the problem is that the target MN does not know whether direct data forwarding applied or indirect data forwarding applied between target SN and source node. It is the target SN detect the availability of the direct data between source node and target SN based on the source node ID included in SN Addition Request message. So, the target MN does not know which IP address i.e. IP address of itself or source NG-RAN node, should be send to target SN when the target SN is added. The only way to make it work is target MN send both of the IP address to target SN and target SN decide which IP address should be added in the ACL list based on the direct data forwarding availability information between target SN and source node. |
| Huawei | After further checking internally, it seems that the target MN does not know whether direct data forwarding path is available or not between source node and target SN. Which means that when SN addition is going to send, the target MN cannot make a decision on which source IP addresses should be used for SN terminated bearers… Therefore, either we go for option 2 transferring 2 IP addresses to target SN or The target MN may send a SN modification request to update the correct IP address to SN when it received the direct data forwarding path available indication from target SN in SN addition response message. Either way is workable. The latter option seems better? [CATT]: Currently, it is not needed to include source IP address for the QoS flow to be modified. If we agree to initiate another SN modification request procedure after MN receives the SN addition response message, then further change on this message is needed. Besides, much more signalling is introduced during handover procedure if SN modification request message is used to update the correct IP address. So,with that,we think using one SgNB addition Request message to transfer the needed information is better. |
| ZTE | Option2. We share the similar view with CATT. The option1 can not handle the case when data forwarding occurs between target SN and source node. The current enhancement of the S-Node Addition Request is the straight forward way to handle this case. |
| Samsung | Option2. The MN cannot know whether direct data forwarding or indirect forwarding via MN when the MN sends SN Addition Request message. |
| Nokia | We share the understanding that option 1 will not work. Also, the option of triggering an extra SN modification request seems dubious for ACL because data forwarding is a close-to-real-time mechanism. So maybe there is no other solution than option 2. When coming to detailed stage 3 implementation, we will then also have to choose between that ACL allows either of the 2 signalled source addresses, or that it knows which one is for indirect DF (via target MN) and which one is for direct DF (directly from source NG-RAN node). |

Conclusion:

A majority of companies is in favour of Option 2. The following is proposed:

Support Option 2: include the IP address of  both target MN node and source NG-RAN node in the S-Node Addition Request message

## Clarification on Source IP interpretation

In [2] the following figure is presented:

Fig. 1: Interfaces and some termination points involved in inter-gNB handover with NR-DC in the source node. IP@ A is the source IP address used for data forwarding from the MN to the SN during the SN addition.

IP@ B is the source IP address used by the gNB-CU-UP to send data to the gNB-DU.

IP@ C is the source IP address used by the source NG-RAN node for data forwarding towards the target node for MN-terminated bearers.

IP@ D is the source IP address used by the source NG-RAN node for data forwarding towards the target node for SN-terminated bearers.

[2] goes on presenting three possible interpretations:

- Option 1: The S-gNB-CU-CP considers the provided IP@ B is also used as IP@ D, i.e. the source IP address the CU-UP has allocated for UP signalling towards the DU is also used as source IP address for data forwarding to target gNB in case of subsequent outgoing handover.

- Option 2: Separate signalling of source IP@ B and source IP@ D during the Bearer setup procedure.

- Option 3: The source IP@ contained in the BEARER CONTEXT SETUP RESPONSE message corresponds to the IP@ D, and the gNB-CU-CP is therefore not informed about IP@ B.

The proposal in [2] is to agree on option3, which can be summarised as follows:

Proposal 1: The source IP@ contained in the BEARER CONTEXT SETUP RESPONSE message corresponds to the source IP address used by the source gNB-CU-UP to forward traffic to a target NG-RAN node

The above proposal seems in line with the proposals in all submitted papers.

It is therefore proposed to agree to the proposal above.

Companies are invited to provide their comments if the above proposal is not agreeable

| Company | Comments |
| --- | --- |

Conclusion:

Agree to the following:

The source IP@ contained in the BEARER CONTEXT SETUP RESPONSE message corresponds to the source IP address used by the source gNB-CU-UP to forward traffic to a target NG-RAN node

[2] also places the following proposal:

Proposal 2: Transfer of source IP address over F1-C and W1-C serves the purpose of dynamic ACL for MN-terminated SCG bearers and SN-terminated MCG bearers.

Companies are invited to provide their views on Proposal 2

| Company | Comments |
| --- | --- |
| Ericsson | OK. It was already clarified during past meetings that transferring of source IP over F1 and W1 is due to cases where a direct UP tunnel can be established from MN-CU-UP to SN-DU and from SN-CU-UP to MN-DU |
| Deutsche Telekom | We are fine with support of Proposal 2. |
| Huawei | Agree. I assume that P2 was already covered by the CRs agreed last meeting. |
| CATT | OK |
| ZTE | OK |
| Samsung | OK |
| Nokia | OK (our proposal). We understand it will be formulated as an agreement that "transfer of source IP over F1 and W1 is due to cases where a direct UP tunnel can be established from MN-CU-UP to SN-DU and from SN-CU-UP to MN-DU". Because such agreement covers more than one interface (either F1-C and Xn-U, or W1-C and Xn-U) it is not straightforward to document it in stage 3 (and we couldn't find such documentation in the agreed stage 3 CRs?), but it should be simple to do it in stage 2, i.e. TS 38.401. So suggest to include TP to TS 38.401 based on above text for handling in second round (Nokia volunteers to draft one). |

Conclusions:

All companies agree to the proposal. It is proposed to agree to the following:

Transfer of source IP address over F1-C and W1-C serves the purpose of dynamic ACL for MN-terminated SCG bearers and SN-terminated MCG bearers.

# Conclusion, Recommendations [if needed]

If needed

# References

R3-221706, Discussion on ACL remaining issues (Huawei, Deutsche Telekom, Ericsson, China Telecom)

R3-221870, Avoiding delayed HO in deployments supporting dynamic ACL for data forwarding (Nokia, Nokia Shanghai Bell)

R3-222175, Discussion on ACL issues (ZTE Corporation)

R3-222005, Discussion on ACL for handover from SA to DC scenario (CATT)