3GPP TSG-RAN WG3#115-e                                             	                     R3-222457

E-Meeting, 21 February – 03 March 2022

Agenda item:	9.3.5.1

Source:	Nokia, Nokia Shanghai Bell

Title: 	Response to R3-222186 and R3-222237

Document for:	Approval

# Discussion

In Tdoc R3-222186 the following is stated:

At first UPF sends one or several end marks without QFI for a PDU session to show there is no more data packets for any QoS flow in the PDU session. But in this case when NG-RAN nodes receives one or several end marks with a QFI added, it is misleading for NG-RAN to understand the intention from UPF.

It is not clear in this case the QFI in the end marks means only the corresponding QoS flow has no more data packets or means all QoS flows in the PDU session have no more data packets. The behavior is not specified in standard but happen in the real network.

And in tdoc R3-222237 the followjng is stated:

We think that for intra-5GS handover, there is no restrictions on whether the GTP end markers are transmitted together with the QFI or not, as long as it is pretty clear that the GTP-U end marker packets are per PDU session.

In addition, it is already allowed to embed the QFI into the end marker for the 5GC to EPC inter-system handover as follows.

<Excerpted from section 9.3.2.5 Data Forwarding for the User Plane of TS 38.300> - Handling of end marker packets: - The source NG-RAN node receives one or several end marker packets per PDU session from the UPF. When there are no more data packets to be forwarded for QoS flows mapped to an E-RAB, the source NG-RAN node sends one or several end markers including one QFI (by means of the PDU Session User Plane protocol TS 38.415 [30]) of those QoS flows mapped to that E-RAB and sends the end marker packets to the UPF over the PDU session tunnel. From the included QFI in the end markers and its mapping to an EPS bearer ID, the UPF knows which EPS bearer tunnel it needs to forward the end-markers to the SGW. The QFI is removed in the end marker packets sent to the SGW.

Then technically speaking, for intra-system handover, it is also allowed to carry the QFI in the end markers, which is already supported in TS 38.415. Hence there is no need to restrict the scenario. On the other hand, if it is to be restricted, this may have negative impact on the on-site and real deployments, and also lead to more IoT test at this late stage.

However, it was very clear from the beginning when we designed the intra-5GS and inter-system handover that we decided to avoid impacts on the PSA UPF and therefore have decided that end marker packets with QFI will only affect forwarding UPFs and only in the inter-system case.

Looking at specifications this is very clear in several places that the requirement to include the PDU Session Container GTP-extension header with QFI is only for inter-system handover and only over data forwarding tunnels:

In TS 29.281, this is very clear from sections 7.3.2.3:

#### 7.3.2.3	End Marker in 5GS

The End Marker message(s) shall also be sent after sending the last G-PDU that needs to be sent on a GTP-U tunnel or after receiving an End Marker Indication as specified in 3GPP TS 23.501 [28] and 3GPP TS 23.502 [29]. End marker packets sent over data forward tunnels in 5GS, for data forwarding between 5GS and EPS, shall be sent with the PDU Session Container extension header including the Qos Flow Identifier of one of the QoS flows mapped to the same E-RAB.

In TS 29.281, this is very clear from sections 5.2.2.7:

#### 5.2.2.7	PDU Session Container

This extension header shall be transmitted in a G-PDU over the N3 and N9 user plane interfaces, between NG-RAN and UPF, or between two UPFs. It shall also be transmitted in End Marker packets over data forwarding tunnels in 5GS, for data forwarding between 5GS and EPS. The PDU Session Container has a variable length and its content is specified in 3GPP TS 38.415 [31].

|  |  |  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets |  |  |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 |  | 1 |
| 1 |  |  | 0xn |  |  |  |  |  |  |  |  |
| 2-(4n -1) |  |  |  | PDU Session Container |  |  |  |  |  |  |  |  |
| 4n |  |  |  | Next Extension Header Type (NOTE) |  |  |  |  |  |  |  |  |

NOTE:	The value of this field is '0' if no other Extension header follows.

Figure 5.2.2.7-1: PDU Session Container Extension Header

TS 29.244 section 5.17.3

Then in TS 29.244 it is clear from section 5.17.3 that the actions requested by SMF to configure the PDU Session Container only involves the forwarding UPFs and not the PSA UPF:

Indirect data forwarding during handovers between 5GS and EPS is supported as follows (see 3GPP TS 38.300 [42]):

-	For 5G to 4G handover, the source NG-RAN node sends one or several end markers including one QFI of those QoS flows mapped to the same E-RAB and sends the end marker packets to the UPF over the PDU session tunnel. UPF removes the QFI and maps to an appropriate E-RAB tunnel towards SGW.

To forward data (G-PDUs and End Marker packets) during a 5GS to EPS handover, the SMF shall:

-	provision one PDR per E-RAB (that supports data forwarding for at least one QoS flow), with the list of QFIs that are mapped to the E-RAB;

-	request the UPF to remove the GTP-U PDU Session Container extension header (including the QFI) from the data by including the GTP-U Extension Header Deletion field set to 'PDU Session Container' in the Outer Header Removal IE of the PDR(s);

-	associate to each PDR a FAR to forward the data to the GTP-U tunnel of the corresponding E-RAB, i.e. with an Outer Header Creation IE containing the F-TEID of the (forwarding) SGW for the corresponding forwarding GTP-U tunnel.

Therefore, receiving the end marker with the PDU Session container extension and QFI is something that is not expected by a source gNB and which it will treat as an error. This is true for both intra-5GS and inter-system handovers. We do not want to extend all the principles quoted above to add yet another option for the PSA UPF into a deep frozen release.

# Conclusion and Proposal

It is very clear from the 3GPP specifications and from the intention when designing end markers for intra-5GS and inter-system handover that a PSA UPF is not expected to include QFI extensions into the end marker packets. We should not change the specifications at this late stage giving any hint that this additional option would be allowed for a receiving gNB.

Proposal 1: Not agree the CRs in R3-222186/ R3-222187.

# References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO, Inc.

- TS 29.281, GPRS Tunnelling protocol User Plane (GTPv1-U)

- TS 29.244, Interface between Control Plane and User Plane nodes, stage 3