3GPP TSG-RAN3 #115-e	R3-222606

21st February –3rd March, 2022

Online

Agenda Item:	23.4

Source:	CATT

Title:	(TP for TS 38.413 on SL relay): Impacts on RAN of AN Release of L2 U2N Relay UE

Document for:	other

- Introduction

In this contribution, one cross-layer issue for L2 U2N relay will be discussed that is the RAN impact of AN release of relay UE.

- Discussion

According to the description in [1], the AN release procedure is as below:

Figure-1 AN Release procedure

The initiation of AN release may be due to:

- (R)AN-initiated with cause e.g. O&M Intervention, Unspecified Failure, (R)AN (e.g. Radio) Link Failure, User Inactivity, Inter-System Redirection, request for establishment of QoS Flow for IMS voice, Release due to UE generated signalling connection release, mobility restriction, Release Assistance Information (RAI) from the UE, etc.; or

- AMF-initiated with cause e.g. Unspecified Failure, etc.

Regarding to the (R)AN-initiated AN release, the corresponding description in [2] is as below:

8.3.2 UE Context Release Request (NG-RAN node initiated) 8.3.2.1 General The purpose of the UE Context Release Request procedure is to enable the NG-RAN node to request the AMF to release the UE-associated logical NG-connection due to NG-RAN node generated reasons. The procedure uses UE-associated signalling. 8.3.2.2 Successful Operation Figure 8.3.2.2-1: UE context release request The NG-RAN node controlling a UE-associated logical NG-connection initiates the procedure by sending a UE CONTEXT RELEASE REQUEST message towards the affected AMF. The UE CONTEXT RELEASE REQUEST message shall indicate the appropriate cause value, e.g., "TXnRELOCOverall Expiry", "Redirection", for the requested UE-associated logical NG-connection release. …… 9.2.2.4 UE CONTEXT RELEASE REQUEST This message is sent by the NG-RAN node to request the release of the UE-associated logical NG-connection over the NG interface. Direction: NG-RAN node AMF IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; ignore. AMF UE NGAP ID; M; 9.3.3.1; YES; reject. RAN UE NGAP ID; M; 9.3.3.2; YES; reject. PDU Session Resource List; 0..1; YES; reject. >PDU Session Resource Item; 1..<maxnoofPDUSessions>; -. >>PDU Session ID; M; 9.3.1.50; -. Cause; M; 9.3.1.2; YES; ignore. …… 9.3.1.2 Cause The purpose of the Cause IE is to indicate the reason for a particular event for the NGAP protocol. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CHOICE Cause Group; M. >Radio Network Layer. >>Radio Network Layer Cause; M; …… User inactivity, ……. User inactivity; The action is requested due to user inactivity on all PDU sessions, e.g., NG is requested to be released in order to optimise the radio resources.. 8.3.3.2 Successful Operation Figure 8.3.3.2-1: UE context release: successful operation …… Upon reception of the UE CONTEXT RELEASE COMMAND message, the NG-RAN node shall release all related signalling and user data transport resources and reply with the UE CONTEXT RELEASE COMPLETE message.

Based on the above description in [2], it is obvious that:

Observation 1: For a UE, when there is no data transmission/reception requirement on all its PDU sessions, NG-RAN can initiate the UE context release procedure. Once receiving the UE context release command from AMF, NG-RAN will release all related signalling and user data transport resources of the UE.

For L2 U2N relay UE, the UP protocol stack is as below. According to the UP protocol, for L2 U2N relay, the relay UE’s PDU session(s) is different from the remote UE’s PDU session(s).

Figure-2 UP protocol stack of L2 U2N relay

Observation 2: For L2 U2N relay, the PDU sessions of relay UE and remote UE are different.

Based on the above observation 1 and observation 2, the issue is that if relay UE is only used for remote UE data forwarding and relay UE itself has no data transmission/reception requirement on its own all PDU sessions, according to the current TS 38.413, whether the (R)AN-initiated AN release should be initiated? Based on the definition of user inactivity in TS 38.413, it is unclear whether the all PDU sessions refers all PDU sessions of relay UE itself or all PDU sessions including relay UE and its connected remote UEs.

| User inactivity | The action is requested due to user inactivity on all PDU sessions, e.g., NG is requested to be released in order to optimise the radio resources. |
| --- | --- |

If “all the PDU session” refers to all the PDU sessions including relay UE itself and its connected remote UE, (R)AN-initiated AN release will not be initiated at least one remote UE has data transmission/reception requirement on any PDU session.

If “all the PDU session” refers to all the PDU sessions of relay UE itself, (R)AN-initiated AN release should not be initiated if relay UE is only used for remote UE’s data forwarding. Otherwise, the RRC connection of relay UE will be lost which will impact the remote UE’s data forwarding.

In our view, it had better clarify that in case of L2 U2N relay, all PDU sessions in the definition of user inactivity should include the PDU sessions of both relay UE and its served remote UE.

Proposal 1: For L2 U2N relay UE, only when user inactivity on all the PDU sessions of the relay UE itself and its served remote UE(s) happens, the NG-RAN can initiate the UE CONTEXT RELEASE procedure for it.

- Conclusion

Observation 1: For a UE, when there is no data transmission/reception requirement on all its PDU sessions, NG-RAN can initiate the UE context release procedure. Once receiving the UE context release command from AMF, NG-RAN will release all related signalling and user data transport resources of the UE.

Observation 2: For L2 U2N relay, the PDU sessions of relay UE and remote UE are different.

Proposal 1: For L2 U2N relay UE, only when user inactivity on all the PDU sessions of the relay UE itself and its served remote UE(s) happens, the NG-RAN can initiate the UE CONTEXT RELEASE REQUEST procedure for it.

- Reference

[1] TS23.502, Procedures for the 5G System (5GS).

[2] TS38.413, NG Application Protocol (NGAP).

- Text Proposal for 38.413

9.3.1.2	Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the NGAP protocol.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent for radio network layer cause when none of the specified cause values applies. |
| TXnRELOCOverall expiry | The timer guarding the handover that takes place over Xn has abnormally expired. |
| Successful handover | Successful handover. |
| Release due to NG-RAN generated reason | Release is initiated due to NG-RAN generated reason. |
| Release due to 5GC generated reason | Release is initiated due to 5GC generated reason. |
| Handover cancelled | The reason for the action is cancellation of Handover. |
| Partial handover | Provides a reason for the handover cancellation. The HANDOVER COMMAND message from AMF contained PDU Session Resource to Release List IE or QoS flow to Release List and the source NG-RAN node estimated service continuity for the UE would be better by not proceeding with handover towards this particular target NG-RAN node. |
| Handover failure in target 5GC/ NG-RAN node or target system | The handover failed due to a failure in target 5GC/NG-RAN node or target system. |
| Handover target not allowed | Handover to the indicated target cell is not allowed for the UE in question. |
| TNGRELOCoverall expiry | The reason for the action is expiry of timer TNGRELOCoverall. |
| TNGRELOCprep expiry | Handover Preparation procedure is cancelled when timer TNGRELOCprep expires. |
| Cell not available | The concerned cell is not available. |
| Unknown target ID | Handover rejected because the target ID is not known to the AMF. |
| No radio resources available in target cell | Load on target cell is too high. |
| Unknown local UE NGAP ID | The action failed because the receiving node does not recognise the local UE NGAP ID. |
| Inconsistent remote UE NGAP ID | The action failed because the receiving node considers that the received remote UE NGAP ID is inconsistent. |
| Handover desirable for radio reasons | The reason for requesting handover is radio related. |
| Time critical handover | Handover is requested for time critical reason i.e., this cause value is reserved to represent all critical cases where the connection is likely to be dropped if handover is not performed. |
| Resource optimisation handover | The reason for requesting handover is to improve the load distribution with the neighbour cells. |
| Reduce load in serving cell | Load on serving cell needs to be reduced. When applied to handover preparation, it indicates the handover is triggered due to load balancing. |
| User inactivity | The action is requested due to user inactivity on all PDU sessions, e.g., NG is requested to be released in order to optimise the radio resources. For L2 U2N Relay UE, this action is requested due to user inactivity on all PDU sessions of L2 U2N Relay UE and its served remote UE(s). |
| Radio connection with UE lost | The action is requested due to losing the radio connection to the UE. |
| Radio resources not available | No requested radio resources are available. |
| Invalid QoS combination | The action was failed because of invalid QoS combination. |
| Failure in the radio interface procedure | Radio interface procedure has failed. |
| Interaction with other procedure | The action is due to an ongoing interaction with another procedure. |
| Unknown PDU Session ID | The action failed because the PDU Session ID is unknown in the NG-RAN node. |
| Unknown QoS Flow ID | The action failed because the QoS Flow ID is unknown in the NG-RAN node. |
| Multiple PDU Session ID instances | The action failed because multiple instance of the same PDU Session had been provided to/from the NG-RAN node. |
| Multiple QoS Flow ID instances | The action failed because multiple instances of the same QoS flow had been provided to the NG-RAN node. |
| Encryption and/or integrity protection algorithms not supported | The NG-RAN node is unable to support any of the encryption and/or integrity protection algorithms supported by the UE. |
| NG intra-system handover triggered | The action is due to a NG intra-system handover that has been triggered. |
| NG inter-system handover triggered | The action is due to a NG inter-system handover that has been triggered. |
| Xn handover triggered | The action is due to an Xn handover that has been triggered. |
| Not supported 5QI value | The QoS flow setup failed because the requested 5QI is not supported. |
| UE context transfer | The action is due to a UE resumes from the NG-RAN node different from the one which sent the UE into RRC_INACTIVE state. |
| IMS voice EPS fallback or RAT fallback triggered | The setup of QoS flow is failed due to EPS fallback or RAT fallback for IMS voice using handover or redirection. |
| UP integrity protection not possible | The PDU session cannot be accepted according to the required user plane integrity protection policy. |
| UP confidentiality protection not possible | The PDU session cannot be accepted according to the required user plane confidentiality protection policy. |
| Slice(s) not supported | Slice(s) not supported. |
| UE in RRC_INACTIVE state not reachable | The action is requested due to RAN paging failure. |
| Redirection | The release is requested due to inter-system redirection or intra-system redirection. |
| Resources not available for the slice(s) | The requested resources are not available for the slice(s). |
| UE maximum integrity protected data rate reason | The request is not accepted in order to comply with the maximum data rate for integrity protection supported by the UE. |
| Release due to CN-detected mobility | The context release is requested by the AMF because the UE is already served by another CN node (same or different system), or another NG interface of the same CN node. |
| N26 interface not available | The action failed due to a temporary failure of the N26 interface. |
| Release due to pre-emption | Release is initiated due to pre-emption. |
| Multiple Location Reporting Reference ID Instances | The action failed because multiple areas of interest are set with the same Location Reporting Reference ID. |
| RSN not available for the UP | The redundant user plane resources indicated by RSN are not available. |
| NPN access denied | Access was denied, or release is requested, for NPN reasons. |
| CAG only access denied | Access was denied because the cell is a non-CAG cell and UE is only allowed to access CAG cells. |
| Insufficient UE Capabilities | The procedure can’t proceed due to insufficient UE capabilities. |