3GPP TSG-RAN WG3 Meeting #115-e	R3-222657

21 February – 3 March 2022

Agenda item:	19.2.2

Source: 	Ericsson, CATT

Title: 		(TP for NRPPa BL CR on Positioning) Completion of RRC_INACTIVE assistance data for positioning

Document for:	Discussion and Approval

# 1. Introduction

The following proposals are agreed from CB#1903 Positioning with RRC_INACTIVE:

Assistance information

Proposal 1, UE reporting information in POSITIONING INFORMATION REQUEST message follows LPP format to help gNB to configure CG-SDT resource.

Proposal 2, agree the TP R3-222657 in revision of R3-221948 (Ericsson) and new TP R3-2 R3-222658 (Ericsson) to capture the agreements on UE reporting information for NRPPa and F1AP.

The LPP is encoding is as follows:

PeriodicalReportingCriteria ::=		SEQUENCE {

reportingAmount						ENUMERATED {

ra1, ra2, ra4, ra8, ra16, ra32,

ra64, ra-Infinity

} DEFAULT ra-Infinity,

reportingInterval					ENUMERATED {

noPeriodicalReporting, ri0-25,

ri0-5, ri1, ri2, ri4, ri8, ri16, ri32, ri64

}

}

# 5. Annex 1: TP to NRPPa BL CR

<<START OF CHANGES>>

### 9.2.x8	UE Reporting Information

This IE contains the UE Reporting Information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reporting Amount | M |  | INTEGER (0..64) | Value 0 represents an infinite number of periodic reporting |
| Reporting Interval | M |  | ENUMERATED (none, 0.25, 0.5, 1, 2, 4, 8, 16, 32, 64) |  |

<<NEXT CHANGE>>

UEReportingInformation::= SEQUENCE {

reportingAmount				INTEGER (0..64),,

reportingInterval			ENUMERATED {none, quarter, half, one, two, four, eight, sixteen, thirty-two, sixty-four, ...},

iE-extensions			ProtocolExtensionContainer { { UEReportingInformation-ExtIEs } }	OPTIONAL,

...

}

<<END OF CHANGE>>