3GPP TSG-RAN3 Meeting #115-e	R3-222751

Electronic meeting, 21 Feb – 3 Mar 2022

Agenda item:	15.3

Source: 	Samsung

Title: 	(TP for BL CR to TS 38.473) RAN visible QoE

Document for:	Discussion & Agreement

Introduction

The TP is to capture the discussion in CB#QoE1 and CB#QoE5

Annex 1 (TP for BL CR to TS 38.473)

<Start of change>

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message |  | Successful Outcome |  | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Response message |  | Response message |  |
| Reset | RESET |  | RESET ACKNOWLEDGE |  |  |  |
| F1 Setup | F1 SETUP REQUEST |  | F1 SETUP RESPONSE |  | F1 SETUP FAILURE |  |
| gNB-DU Configuration Update | GNB-DU CONFIGURATION UPDATE |  | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-DU CONFIGURATION UPDATE FAILURE |  |
| gNB-CU Configuration Update | GNB-CU CONFIGURATION UPDATE |  | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU CONFIGURATION UPDATE FAILURE |  |
| UE Context Setup | UE CONTEXT SETUP REQUEST |  | UE CONTEXT SETUP RESPONSE |  | UE CONTEXT SETUP FAILURE |  |
| UE Context Release (gNB-CU initiated) | UE CONTEXT RELEASE COMMAND |  | UE CONTEXT RELEASE COMPLETE |  |  |  |
| UE Context Modification (gNB-CU initiated) | UE CONTEXT MODIFICATION REQUEST |  | UE CONTEXT MODIFICATION RESPONSE |  | UE CONTEXT MODIFICATION FAILURE |  |
| UE Context Modification Required (gNB-DU initiated) | UE CONTEXT MODIFICATION REQUIRED |  | UE CONTEXT MODIFICATION CONFIRM |  | UE CONTEXT MODIFICATION REFUSE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST |  | WRITE-REPLACE WARNING RESPONSE |  |  |  |
| PWS Cancel | PWS CANCEL REQUEST |  | PWS CANCEL RESPONSE |  |  |  |
| gNB-DU Resource Coordination | GNB-DU RESOURCE COORDINATION REQUEST |  | GNB-DU RESOURCE COORDINATION RESPONSE |  |  |  |
| F1 Removal | F1 REMOVAL REQUEST |  | F1 REMOVAL RESPONSE |  | F1 REMOVAL FAILURE |  |
| BAP Mapping Configuration |  | BAP MAPPING CONFIGURATION |  | BAP MAPPING CONFIGURATION ACKNOWLEDGE |  | BAP MAPPING CONFIGURATION FAILURE |
| GNB-DU Resource Configuration |  | GNB-DU RESOURCE CONFIGURATION |  | GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE |  | GNB-DU RESOURCE CONFIGURATION FAILURE |
| IAB TNL Address Allocation |  | IAB TNL ADDRESS REQUEST |  | IAB TNL ADDRESS RESPONSE |  | IAB TNL ADDRESS FAILURE |
| IAB UP Configuration Update |  | IAB UP CONFIGURATION UPDATE REQUEST |  | IAB UP CONFIGURATION UPDATE RESPONSE |  | IAB UP CONFIGURATION UPDATE FAILURE |
| Resource Status Reporting Initiation |  | RESOURCE STATUS REQUEST |  | RESOURCE STATUS RESPONSE |  | RESOURCE STATUS FAILURE |
| Positioning Measurement |  | POSITIONING MEASUREMENT REQUEST |  | POSITIONING MEASUREMENT RESPONSE |  | POSITIONING MEASUREMENT FAILURE |
| Positioning Information Exchange |  | POSITIONING INFORMATION REQUEST |  | POSITIONING INFORMATION RESPONSE |  | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange |  | TRP INFORMATION REQUEST |  | TRP INFORMATION RESPONSE |  | TRP INFORMATION FAILURE |
| Positioning Activation |  | POSITIONING ACTIVATION REQUEST |  | POSITIONING ACTIVATION RESPONSE |  | POSITIONING ACTIVATION FAILURE |
| E-CID Measurement Initiation |  | E-CID MEASUREMENT INITIATION REQUEST |  | E-CID MEASUREMENT INITIATION RESPONSE |  | E-CID MEASUREMENT INITIATION FAILURE |

Table 2: Class 2 procedures

| Elementary Procedure | Message |  |
| --- | --- | --- |
| Error Indication | ERROR INDICATION |  |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |  |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |  |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |  |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |  |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |  |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |  |
| Paging | PAGING |  |
| Notify | NOTIFY |  |
| PWS Restart Indication | PWS RESTART INDICATION |  |
| PWS Failure Indication | PWS FAILURE INDICATION |  |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |  |
| RRC Delivery Report | RRC DELIVERY REPORT |  |
| Network Access Rate Reduction | NETWORK ACCESS RATE REDUCTION |  |
| Trace Start |  | TRACE START |
| Deactivate Trace |  | DEACTIVATE TRACE |
| DU-CU Radio Information Transfer | DU-CU RADIO INFORMATION TRANSFER |  |
| CU-DU Radio Information Transfer | CU-DU RADIO INFORMATION TRANSFER |  |
| Resource Status Reporting | RESOURCE STATUS UPDATE |  |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |  |
| Reference Time Information Reporting Control |  | REFERENCE TIME INFORMATION REPORTING CONTROL |
| Reference Time Information Report |  | REFERENCE TIME INFORMATION REPORT |
| Access Success |  | ACCESS SUCCESS |
| Cell Traffic Trace | CELL TRAFFIC TRACE |  |
| Positioning Assistance Information Control | POSITIONING ASSISTANCE INFORMATION CONTROL |  |
| Positioning Assistance Information Feedback | POSITIONING ASSISTANCE INFORMATION FEEDBACK |  |
| Positioning Measurement Report | POSITIONING MEASUREMENT REPORT |  |
| Positioning Measurement Abort | POSITIONING MEASUREMENT ABORT |  |
| Positioning Measurement Failure Indication | POSITIONING MEASUREMENT FAILURE INDICATION |  |
| Positioning Measurement Update | POSITIONING MEASUREMENT UPDATE |  |
| Positioning Deactivation | POSITIONING DEACTIVATION |  |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |  |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |  |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |  |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |  |
| QoE Information Transfer | QOE INFORMATION TRANSFER |  |

<Next change>

## 8.X	QoE Information Transfer procedures

### 8.X.1	QoE Information Transfer

#### 8.X.1.1	General

The purpose of the QoE Information Transfer procedure is to transfer RAN visible QoE information from the gNB-CU to the gNB-DU. The procedure uses UE-associated signalling.

#### 8.X.1.2	Successful operation

Figure 8.X.1.2-1: QoE Information Transfer procedures.

The gNB-CU initiates the procedure by sending the QOE INFORMATION TRANSFER message to the gNB-DU.

If the QoE Information List IE is included in QOE INFORMATION TRANSFER message, the gNB-DU shall, if supported, take it into account according to TS 38.300 [6].

#### 8.X.1.3	Abnormal Conditions

Not applicable.

<Next change>

### 9.2.X	QoE Information Transfer messages

#### 9.2.X.1	QOE INFORMATION TRANSFER

This message is sent by a gNB-CU to a gNB-DU, to indicate RAN visible QoE information.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| QoE Information List |  |  |  |  |  |  |
| >QoE Information Item |  |  | 1..<maxnoofQoEInformation> |  |  |  |
| >>QoE Metrics | O |  | FFS |  | - | - |
| >DRBID List |  |  | 9.3.1.x |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoEInformation | Maximum no. of QoE information for one UE, the maximum value is16. |

<Next change>

#### 9.3.1.x	DRB ID List

This IE provides DRB information associated with QoE Information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DRB ID List |  |  | 1 .. <maxnoofDRBs> |  | – |  |
| >DRB ID | M |  | 9.3.1.8 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |

<Next change>

#### 9.3.1.y	QoE Metrics

This IE provides RAN visible QoE Metrics for gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| >>Buffer Level | O |  | OCTET STRING | as defined in TS 38.331 [8]. | – |  |
| >>Playout Delay | O |  | OCTET STRING | as defined in TS 38.331 [8]. | – |  |

<end of change>