3GPP TSG-RAN WG3 Meeting #115-e	R3-222765

# Online, Feb 21st – Mar 3rd 2022

Agenda Item:	10.2.4

Source:	Ericsson

Title:	(TP for SON for TS 38.300) Inter-System Load Balancing

Document for:	Discussion, Decision

# TP for SON for TS 38.300

//////////////////////////////////////// Change Start ////////////////////////////////////////////////

### 15.5.1	Support for Mobility Load Balancing

#### 15.5.1.1	General

The objective of mobility load balancing is to distribute load evenly among cells and among areas of cells, or to transfer part of the traffic from congested cell or from congested areas of cells, or to offload users from one cell, cell area, carrier or RAT to achieve network energy saving. This can be done by means of optimization of cell reselection/handover parameters and handover actions. The automation of such optimisation can provide high quality user experience, while simultaneously improving the system capacity and also to minimize human intervention in the network management and optimization tasks.

Intra-RAT , intra-system inter-RAT and inter-system load balancing scenarios are supported.

In general, support for mobility load balancing consists of one or more of following functions:

-	Load reporting for intra-RAT and intra-system inter-RAT load balancing;

-	Load balancing action based on handovers;

-	Adapting handover and/or reselection configuration;

-	Load reporting for inter-system load balancing

#### 15.5.1.2	Load reporting for intra-RAT and intra-system inter-RAT load balancing

The load reporting function is executed by exchanging load information over the Xn/X2/F1/E1 interfaces.

The following load related information should be supported which consists of,

-	Radio resource usage (per-cell and per SSB area PRB usage: DL/UL GBR PRB usage, DL/UL non-GBR PRB usage, DL/UL total PRB usage, and DL/UL scheduling PDCCH CCE usage) ; PRB usage for slice(s): DL/UL GBR PRB usage, DL/UL non-GBR PRB usage, and DL/UL Total PRB allocation);

-	TNL capacity indicator (UL/DL TNL offered capacity and available capacity);

-	Cell Capacity Class value (UL/DL relative capacity indicator);

-	Capacity value (per cell, per SSB area and per slice: UL/DL available capacity);

-	HW capacity indicator (offered throughput and available throughput over E1, percentage utilisation over F1);

-	RRC connections (number of RRC connections, and available RRC Connection Capacity);

-	Number of active UEs.

To achieve load reporting function, Resource Status Reporting Initiation & Resource Status Reporting procedures are used.

//////////////////////////////////////// Next Change ////////////////////////////////////////////////

#### 15.5.1.x	Load reporting for inter-system load balancing

The load reporting function for inter-system load balancing is executed by exchanging load information between NG-RAN and E-UTRAN. Both event-triggered and periodic inter-system load reporting are supported. Event-triggered inter-system load reports are sent when the reporting node detects crossing of cell load thresholds.

The following load related information should be supported:

-	Cell Capacity Class value (UL/DL relative capacity indicator);

-	Capacity value (per cell: UL/DL available capacity);

-	RRC connections (number of RRC connections, and available RRC Connection Capacity);

-	Number of active UEs.

-	PRB usage (per cell: UL/DL)

NGAP procedures used for inter-system load balancing are Uplink RAN Configuration Transfer and Downlink RAN Configuration Transfer.

S1AP procedures used for inter-system load balancing are eNB Configuration Transfer and MME Configuration Transfer.

//////////////////////////////////////// End of Changes ////////////////////////////////////////////////