3GPP TSG-RAN WG3 #115-e	R3-222434
Online, 21 Feb - 3 Mar 2022

Agenda Item:	19.2.2

Source:	Samsung (moderator)

Title:	CB: # 1902_Pos_RRC_INACTIVE

Document for:	Discussion

Introduction

CB: # 1903_Pos_RRC_INACTIVE

- Need to generalize the UE Reporting Information into a LMF Assistance Information? If yes, what should be included in it?

- Assuming the UE Reporting Information remains as defined so far, what information to include in it and how to encode it?

- E.g. should it include validity area, validity time, area based, motion based etc.?

- Any agreements that can be taken for different use cases of anchor relocation? Should any use case be postponed?

- Converge on details on the mechanism for SRS reservation/releasing over F1. Need to check with RAN2?

- Should the Xn: Retrieve UE Context procedure be enhanced? E.g. addition of location event indications, NRPPa Transaction ID, History SRS configuration etc

- Capture agreements and provide TPs

(Samsung - moderator)

The plan is the following:

- Phase 1, try to achieve agreements, the deadline for phase 1 is the end of Thursday Feb 24th

- Phase 2, try to converge the agreement and TP work.

For the Chair’s Notes

### Assistance information

Proposal 1, UE reporting information in POSITIONING INFORMATION REQUEST message follows LPP format to help gNB to configure CG-SDT resource.

Proposal 2, agree the NRPPa TP R3-222657 (Ericsson, CATT) in revision of R3-221948  and F1AP TP R3-222658 (Ericsson, CATT).

### SRS reservation

Proposal 3, RAN3 agrees to include positioning context reservation indication in RRC CONTEXTT RELEASE COMMAND message over F1AP.

Proposal 4, agree the F1AP TP R3-222671 (Samsung, CATT) in revision of R3-222287.

Proposal 5, releasing the SRS configuration both in gNB and UE side at the same time should be ensured in order to avoid UL interference.

Proposal 6, RAN3 agrees to reuse positioning deactivation to release SRS configuration when UE resumes to a new cell/gNB, i.e. the anchor gNB receives Retrieve UE Context Request from other gNBs.

### Positioning context related

Proposal 7, RAN3 agrees to include NRPPa Transaction ID in RETRIEVE UE CONTEXT RESPONSE message as positioning information, to make sure LMF understand the message from the same NRPPa transaction.

Proposal 8, agree the XnAP TP R3-222670 (CATT, Samsung) in revision of R3-221893.

### In case of w/o anchor relocation

Proposal 9, the RRC_INACTIVE positioning in case of SDT w/o anchor relocation can be discussed in R18.

Round-2 Discussion

### Assistance information

Proposal 1, UE reporting information in POSITIONING INFORMATION REQUEST message follows LPP format to help gNB to configure CG-SDT resource.

Proposal 2, agree the TP R3-22XXX in revision of R3-221948 (Ericsson) and new TP R3-22XXXX (Ericsson) to capture the agreements on UE reporting information for NRPPa and F1AP.

### SRS reservation

Proposal 3, RAN3 agrees to include positioning context reservation indication in RRC CONTEXTT RELEASE COMMAND message.

Proposal 4, agree the TP R3-22xxxx in revision of R3-222287 (Samsung) to capture the agreements on SRS reservation.

Proposal 5, releasing the SRS configuration both in gNB and UE side at the same time should be ensured in order to avoid UL interference.

Proposal 6, RAN3 agrees to reuse positioning deactivation to release SRS configuration when UE resumes to a new cell/gNB, i.e. the anchor gNB receives Retrieve UE Context Request from other gNBs.

### Positioning context related

Proposal 7, RAN3 agrees to include NRPPa Transaction ID in RETRIEVE UE CONTEXT RESPONSE message as positioning information, to make sure LMF understand the message from the same NRPPa transaction.

Proposal 8, agree the TP R3-22XXXX in revision of R3-221893 (CATT) to capture the agreements on NRPPa Transaction ID.

| Company | Comment |
| --- | --- |
| Ericsson | ### Assistance Information: Do we need a mirror F1AP tp for signalling the UE reporting information to the gNB-DU? This assistance information is for RRM purpose with INACTIVE state, which is at gNB-CU. ### Positioning context related As stated, there is a mistake in NRPPa where Transaction IDs are everywhere. Transaction IDs are “commonly agreed” by RAN3 to be only used for non-UE associated messages. We don’t think that building on a mistake is the right thing to do. If an ID is needed to correlate with the Positioning Information Update message, why not sending the RAN UE NGAP ID as signalled in the UL/DL UE ASSOCIATED NRPPA TRANSPORT message? |
| Nokia | P1-P6: OK P7: Seems OK. Regarding the usage of Transaction ID, we note the following specification text in NRPPa: - 9.2.4, “NRPPa Transaction ID IE is used to associate all the messages belonging to the same procedure” - 5.2, “at any instance in time one protocol peer may have more than one ongoing NRPPa procedure” Based on the above, our understanding is that NRPPa Transaction ID is only useful for Class 1 procedures. It is needed for non-UE associated Class 1 procedures (as highlighted by Ericsson), but seems “needed” also for UE-associated Class 1 procedures since parallel NRPPa procedures are allowed (although there may not be any real/practical scenario in current release where transaction ID would be absolutely necessary given the existence also of e.g. Measurement ID). But at least based on current definitions, it seems we should include Transaction ID in all Class 1 procedures (both UE-associated and non-UE associated). |
| HW | P1-P7 ok. Since the transaction ID is already included in the existing NRPPa Positioning Information exchange procedure, it would be needed to be exchanged. |
| CATT | All of the proposals are ok with us. |
| Samsung | Agree all the proposals |

Moderator’s summary:

With further clarification, all companies agree the proposals.

### Assistance information for deciding the UE state

Question 1, do companies agree to introduce below assistance information to help gNB to decide the RRC state of the positioning UE?

- "Reporting Amount", "Reporting Interval", and "Duration" dependent on the configured event type and as specified in TS 24.080

- “Suggested State”

| Company | Comment |
| --- | --- |
| Ericsson | Disagree, this was not agreed by RAN2 and LMF does not know UE RRC state and will conflict with gNB/AMF implementation. This will create a paradox in the specification that we are not ready to accept. Also as said, UE can send UL LCS/LPP messages via SDT in inactive, and for LMF the UE is in CM_CONECTED state |
| Nokia | We are not convinced that additional assistance information is needed, beyond what is agreed in P1-P2. Also, UE’s RRC state should be transparent to LMF. |
| HW | The suggested state information is not related to LMF knowing RRC state, but to let the gNB to know wether the serving gNB can configure the UE into RRC Inactive, ie, go with Rel-16 way (always keep UE in connected) or can go with Rel-17 way (can release UE into inactive when apprioporate). In other words, the suggested state information is used to assist gNB to make decision when configuring UE sending SRS, ie, whether use RRCReconfiguration or RRCRelease message to configure SRS. Maybe the name of the information can be updated if it can be considered by companies. |
| CATT | We should not introduce the suggested RRC state from LMF to gNB. Currently, the RRC State is transparent to LMF, and it should be decided by gNB itself. |
| Samsung | The assistance information seems not that essential for INACIVE positioning UE, the gNB can decide the UE state based on current information. |

Moderator’s summary:

Majority companies are not convinced that additional assistance information from LMF is needed.

### In case of without anchor relocation

In the first round, one company proposed to enhance the partial UE context retrieve procedures in case of SDT without anchor relocation, two companies showed their concerns on the impact of SDT progress. The moderator understands the concerns, but since the stage 2 work of SDT is stable, the moderator suggests to further discuss the potential positioning impact or scenarios based on the SDT stage 2 in the 2nd round, at least we can make the understanding clearer, even the support of without anchor relocation may be postponed to R18.

Positioning scenarios to be considered in case of without anchor relocation.

The following is the current SDT stage 2 status of BL CR in R3-221562

XX.3	RA-based SDT without UE context relocation

Editor’s Note: Further discussion is needed for the overall procedure for RACH based SDT without anchor relocation.

UE

Receiving gNB

Last Serving gNB

AMF

UPF

(

s

)

1

.

RRCResumeRequest

+

UL data and

/

or UL SDT signaling

2

.

RETRIEVE UE CONTEXT REQUEST

(

SDT indicator

)

Subsequent SDT

UL data

0

.

UE in RRC

_

INACTIVE

CM

-

CONNECTED

6

.

RETRIEVE UE CONTEXT FAILURE

(

RRCrelease message

)

SDT

UL data

or UL NAS PDU

Establish

SDT

RLC entity

3

.

Decides to keep

UE context

SDT D

L data

Keep

PDCP

entity

4

.

FFS

:

PARTIAL UE CONTEXT RETRIEVAL

(

RLC config

,

UL PDCP TNL addr

)

5

.

FFS

:

PARTIAL UE CONTEXT RETRIEVAL ACKNOWLEDGE

(

CG TNL addr

)

8

.

UE in RRC

_

INACTIVE

CM

-

CONNECTED

7

.

RRC release

(

suspend indication

)

Figure XX.3-1. RA-based SDT without UE context relocation

According to the current stage 2 figure above and the discussion in the 1st round, the moderator summarized below two scenarios for positioning during SDT without anchor relocation.

Scenario 1, deferred positioning case, the NRPPa transactions may be triggered after the LMF receives the LCS event report, the blue part below relates to positioning.

Scenario 2, normal positioning case, the NRPPa message arrives before RRC resume.

Question 2, do companies agree the above two scenarios when positioning happens during SDT without anchor relocation?

- If scenario 1 is agreed, do companies agree option 1 in Q6?

- If scenario 2 is agreed, do companies agrees the enhancement proposed by HW in Q6 or VIVO’s proposal in Q8 (which can also be applied to the scenario of with anchor relocation)?

| Company | Comment |
| --- | --- |
| Ericsson | We are following the SDT discussions and we disagree that the stage 2 design is stable, in fact changes are still being discussed… Just to clarify that Ericsson is not against of support of SDT w/o anchor relocation, but companies have the right for more time to check on the proposals with an updated understanding of the concept(s) based on a stabilized SDT design, and not a shaky one. We propose to postpone the discussion to R18 and that the WI completion is not impacted by non support of SDT w/o anchor relocation. It is fine if moderator prefers to capture some general understanding in this CB which can be used as inputs for R18 discussion. |
| Nokia | We are fine to postpone discussion of “without anchor relocation” to R18. |
| HW | To clarify, For scenario 1, our intention is to transfer pos context after the event report and Positioning Information Request messge, ie, use the Partial Retrieve UE Context procedure to transfer the pos context only, as follows: UE Receiving gNB Last Serving gNB AMF/LMF UPF ( s ) 1 . RRCResumeRequest + UL data and / or UL SDT signaling 2 . RETRIEVE UE CONTEXT REQUEST ( SDT indicator ) 0 . UE in RRC _ INACTIVE CM - CONNECTED 6 . RETRIEVE UE CONTEXT FAILURE ( RRCrelease message ) SDT UL data or UL NAS PDU (Event Report) Establish SDT RLC entity 3 . Decides to keep UE context Keep PDCP entity 4 . FFS : PARTIAL UE CONTEXT RETRIEVAL ( RLC config , UL PDCP TNL addr ) 5 . FFS : PARTIAL UE CONTEXT RETRIEVAL ACKNOWLEDGE ( CG TNL addr ) 8 . UE in RRC _ INACTIVE CM - CONNECTED 7 . RRC release ( suspend indication ) . PARTIAL UE CONTEXT RETRIEVAL (Pos Information) , . PARTIAL UE CONTEXT RETRIEVAL ACK (SRS) POSITIONING INFORMATIN REQUEST The anchor and serving gNB has no idea about what is included in the NAS message so there will no positioning related context transfer during event report. For Scenario 2, as clarified in previous response, which is not included in the summary: Firstly we don’t think the failure message of a class 1 procedure can be sent from a different node. Then regarding the way of sending failure upon receiving the RRC Context Release message: we thinks this can work, we don’t think the AMF would discard the NRPPa message. The AMF still have the UE context and every information to transfer the NRPPa message. The AMF would have no idea what information is included in the NRPPa, and can transfer any NRPPa message based on the routing ID etc, when the LCS session is ongoing in the AMF. For scenario 2, we also support the solution proposed in Q6. |
| CATT | We are fine to postpone the discussion to R18. |
| Samsung | Ok to postpone the discussion to R18 |

Moderator’s summary:

4 companies prefer to postpone the discussion the case of SDT w/o anchor relocation, 1 company provided the further clarification. It seems that companies have no commom understanding on the scenerios, but all the companies are fine to further discuss this in R18.

Proposal 9, the RRC_INACTIVE positioning in case of SDT w/o anchor relocation can be discussed in R18.

Round-1 Discussion

Assistance information from LMF to gNB

Information for CG-SDT configuration

RAN3 114bis-e meeting, it was agreed to include UE reporting information in POSITIONING INFORMATION REQUEST message. The leftover issues are

- Q1, whether to consider area event and motion event in UE reporting information

- Q1bis, how to define the IE type of Reporting Amount/Interval IEs

[2], Qualcomm:  Q1: Yes, Q1bis: use LCS format

Proposal 3:	The "LMF assistance information" from the LMF to the gNB should include information on the UE configured event type.

Proposal 4:	The "LMF assistance information" from the LMF to the gNB should include the reporting information "Reporting Amount", "Reporting Interval", and "Duration" dependent on the configured event type and as specified in TS 24.080 [4].

[3], Huawei  Q1: Yes, Q1bis: use LCS format

Proposal 1: Use reporting Duration and Interval from TS24.080 in UE Reporting Information IE.

[5], Ericsson  Q1: No, Q1bis: use LPP format

Proposal 1: The encoding of the UE Reporting Information IE follows LPP format

[7], Samsung  Q1: No

Proposal 1: RAN3 agrees not to consider area based and motion based in UE reporting information in R17.

According to the discussions in the contributions [2], [3], [5] and [7], it is the moderator’s understanding that all companies acknowledge that UE reporting information can be used for gNB to allocate proper CG-SDT resources for a positioning UE, but companies have different views on the usage scenario of UE reporting information, i.e. whether it’s for LCS message or LPP message.

Regarding other proposals like the name of the IE and the descriptions proposed by [2], they can be revisited after the 1st round discussion. Please note that the references for LCS message and LPP message are in the Annex if companies want to further check the detail IEs.

### Q1, which message do you think the UE reporting information is for? If it’s for LCS message, should the “duration” IE be introduced for area event and motion event in UE reporting information IE?

| Company | LCS message or LPP message | Comment |
| --- | --- | --- |
| Samsung | LPP message | We share similar view with the argument in [5]. The SDT is for temporary transmission, but the deferred location event may be for the next hours or days, UE may be not in RRC_INACTIVE state at that moment, it seems the UE reporting information is useless in that case. For the LPP messages which will be transmitted after each location event if positioning is needed, the LMF can use this UE reporting information to indicate the gNB that there will be subsequent positioning procedures and please prepare the resources accordingly. If there’s no LPP messages for one location event, the LMF will not send this information. Besides, after checking the LCS specs, we haven’t found any proper IE to indicate the actual reporting time for area event and motion event. The “duration” is maximum duration of event reporting by a UE(refer to Annex A), but it’s not the actual reporting time, which should depend on the UE’s movement behavior and criteria sent by the network, so we don’t think this duration IE is a good reference for CG-SDT configuration. |
| HW | Depend on the CG periodicity | We have consensus on the fact that the UE reporting information is used for configure CG. Then whether the CG is used for event report or LPP measurement report. We think technically both might be possible and it depend on the periodicity of the CG, which is still under discussion in SDT WI. A short periodicity is definitely more suitable for LPP measurement report and a long periodicity also work for periodic event report. So we previously had the preference of LPP periodicity but we think we should have a look the periodicity of the CG. |
| CATT | LPP message | Share the view with SS. |
| Ericsson | LPP message | SDT is regarded as a temporary situation for the UE, so the LPP periodicity is enough to carry this purpose (we do not need days…). Also, our understanding is that RAN2 has thoroughly discussed the proposals in [2] on LMF assistance information and were rejected… |
| Qualcomm | LCS message | LPP periodic reporting can only be used between UE and LMF. This is not supported in the CN for providing periodic services to an LCS client. Therefore, in practice, if periodic or triggered reporting is desired (i.e., by an LCS client) only the LCS reporting criteria are applicable. How useful these criteria are for CG is a separate question. We think this information provided to a gNB is primarily (or even only) useful to assist a gNB to transition the UE to RRC_INACTIVE. We can't see how this can be reliably used for CG, since there is too much uncertainty in when the UE reports (i.e., the UE will not report exactly on e.g., slot or frame boundaries, but e.g., +/- delta seconds). |
| vivo | LCS message | In our understanding, the information can be used to assist the gNB to decide whether to turn the UE to RRC_INACTIVE. If the gNB decides to transition the UE to RRC_INACTIVE, the information can be further used to help with the SDT configuration. How to interpret the information is up to gNB implementation and the LMF does not need to convert the original interval. One way to compromise is to specify the maximum duration, i.e. the interval * amount. |
| CMCC | LPP message | Share same view with Samsung. |

Moderator’s summary:

7 companies commented, 4 support LPP message and 2 support LCS message, one company think this depends on the periodicity of CG-SDT, after checking the RAN2 status, the T3XXX(newSDTTimer) is more fit for LPP message, which can be seems as the periodicity of CG-SDT. On the other hand, the argument for LCS message is to use the information to decide the RRC state, which is not aligned with our previous agreement that the information is for gNB to decide the CG-resource. Regarding whether to consider the assistance information about LCS message to help gNB for deciding the RRC state can be discussed in the 2nd round.

Proposal 1, UE reporting information in POSITIONING INFORMATION REQUEST message follows LPP format to help gNB configure CG-resource.

Proposal 2, agree the TP R3-22XXX in revision of R3-221948 (Ericssion) and new TP R3-22XXXX (Ericssion) to capture the agreements on UE reporting information for NRPPa and F1AP.

For 2nd round discussion

Whether introduce other assistance information to help gNB to decide the RRC state of the positioning UE?

Other information or scenario of assistance information.

Below are some other proposals related to assistance information from LMF in [2], [3] and [5].

[2], Qualcomm: assistance information can be used for gNB to decide UE state

Proposal 2:	The "LMF assistance information" from an LMF to the gNB from Proposal 1 should comprise all information which can assist the gNB to decide on whether to transit the UE to RRC_INACTIVE state or not.

[3], Huawei  Suggested State from LMF to gNB

Proposal 2: LMF to include the assistance information (eg. Suggested State) in Positioning Information Request message to suggest the gNB-CU to release the UE into RRC Inactive state.

[5], Ericsson  Validity area and validity timer for pre-configured SRS

Proposal 2: Introduce a validity area and validity timer for the positioning related configuration in RRC_INACTIVE in the UE Reporting Information IE.

Based on the above proposals, please answer the following question.

### Q2, do companies agree below enhancements about the assistance information from LMF to gNB?

- 1) Add descriptions like “when the UE reporting information in included, the serving NG-RAN node can take into account to decide the UE state”

- 2) include suggested state in Positioning information request from LMF.

- 3) include Validity area and validity timer information in UE reporting information.

| Company | Comment |
| --- | --- |
| Samsung | For 1), we think UE reporting information can be used for gNB to decide UE state, so we agree to add the related descriptions as suggested by [2]. For 2), more clarification is needed, such as does LMF have any advantage or any additional information to suggest the UE state, comparing to the serving gNB, so that this suggested state from LMF is necessary. For 3), RAN2 had already precluded the pre-configured SRS in the last meeting, so there’s no need to introduce the validity area and validity timer. |
| HW | - We think assistance information is for CG configuration. We should not extend the usage. Last meeting we have agreed to remove the corresponding description. - Note that UE in inactive positioning has different capabilities from that in Connected, which may lead to different accuracy/latency/power consumption. The suggestion of stage is more like the LMF to choose the R16 positioning (Connected state) or R17 positioning (Inactive state) to achieve the positioning with different KPI, such as accuracy, latency, power consumption. If there is not such kind of suggestion, the serving gNB would always go with R17 way to try to release the UE into Inactive. - Agree with Samsung. no need. |
| CATT | For 1), the intension to introduce the assistance info from LMF to gNB is to assist gNB to decide UE state. For 2), we do not see why and how the LMF suggest the RRC state. It’s totally decided by the gNB. For 3), Share the view with SS and HW, it’s not needed. |
| Ericsson | 1) Disagree. As Huawei said, this is uniquely for CG configuration as discussed during last e-meeting. 2) RRC state is not exposed to LMF, that was RAN2 agreement. Also, it could be that the UE is already in RRC Inactive…any UL LCS or LPP msg can be carried using SDT; so from LMF perspective it cannot for sure know what is the UE RRC state. 3) Ok to follow the majority. |
| Qualcomm | We consider (1) and (3) being indirect assistance for assisting the gNB on RRC state. However, a direct request would be preferred, and therefore, we support (2). (1) and (3) belong together, because (3) is also reporting information. So our 1st preference is (2). 2nd preference is (1) and (3). |
| vivo | 1) Agree. No stage 3 impact and it’s up to gNB implementation. 2) Disagree. The LMF is not aware of the service status of the UE. 3) Disagree. Same view as SS. |
| CMCC | 1) Disagree. The intention of introducing the assistance info from LMF is CG configuration, not for deciding the UE state. 2) Disagree. LMF is not aware of the RRC state. We are wondering how the LMF Can provide the suggested state. 3) Disagree. Same view with Samsung. |

Moderator’s summary:

No consensus, but regarding whether other assistance information needed for deciding RRC state can be further discussed in the 2nd round, please the proponent companies provide further clarification to solve the concerns from other companies in the first round.

SRS Reservation and release

SRS reservation

In RAN3 114bis-e meeting, it was agreed to Send an indication (e.g. Reserve SRS) IE from gNB-CU to gNB-DU over F1AP to reserve the SRS configuration in DU for RRC_INACTIVE UE positioning, FFS on the details. From the contributions, there’re two options on the messages to include the SRS reservation indication.

- option 1 POSITIOING INFOMRAITION REQUEST message

- option 2 RRC CONTEXT RELEASE COMMAND message.

[3], Huawei  option 1 and option 2

Proposal 4: Include an indication (e.g. Reserve SRS) either in POSITIOING INFOMRAITION REQUEST message, or RRC CONTEXT RELEASE COMMAND message.

[5], Ericsson  option 2

Proposal 3: The gNB-CU sends an indication Reserve SRS for positioning IE to gNB-DU for reservation of the UL SRS positioning resources in RRC INACTIVE state using the F1 UE CONTEXT RELEASE COMMAND message. FFS on the procedural text and IE encoding pending on RAN3 progress on CG-SDT in AI 24.3

[7], Samsung  option 2

Proposal 6 RAN3 agrees include the positioning context reservation indication in UE CONTEXT RELEASE COMMAND message.

### Q3, which option do companies agree to support the reservation of the SRS resource on gNB-DU?

- Option 1, include positioning context reservation indication in POSITIOING INFOMRAITION REQUEST message

- Option 2, include positioning context reservation indication in RRC CONTEXTT RELEASE COMMAND message

| Company | Option 1/2 | Comment |
| --- | --- | --- |
| Samsung | Option 2 | We think it’s better to follow the SDT mechanism, RRC Context Release command is the best choice, including the indication in any other messages before the final release may lead to useless information. |
| HW | Either | We have the preference of using the positioning procedure. We are OK if majority selecting Option 2. |
| CATT | Option 2 | Better to align with SDT. |
| Ericsson | Option 2 | Both can work, but we expect that before implementing the F1 Positioning functions, a split gNB will support the mechanism for UE context reservation via the UE CONTEXT RELEASE COMMAND message. This reduces the implementation efforts for F1 development engineers. |
| CMCC | Either | No strong view. |

Moderator’s summary:

All companies agree option 2.

Proposal 3, RAN3 agree to include positioning context reservation indication in RRC CONTEXTT RELEASE COMMAND message.

Proposal 4, agree the TP R3-22xxxx in revision of R3-222287 (Samsung) to capture the agreements.

SRS release

Regarding how to release the reserved SRS, majority of companies prefer no specification impact, but as analyzed in [7], reusing positioning deactivation procedure is only valid for the scenario when UE resumes to a new cell, both UE and gNB can release the SRS resources accordingly. But for the scenario when the positioning is end but UE is still within the anchor cell, if the gNB release the SRS resource via positioning deactivation, how the UE get the SRS deactivation is still on RAN2’s discussion. But no matter how, as argued in [3], to avoid UL interference, the SRS configuration should be released at the same time both in gNB and UE.

[3], Huawei  reuse Positioning Deactivation

Proposal 5: Do not define SRS release condition at the gNB-DU; use the Positioning Deactivation to release the SRS.

[4], CATT  reuse Positioning Deactivation

Proposal 3: Positioning Deactivation procedure could be reused to release the SRS resources in the gNB-DU, no further enhancement is necessary.

[5], CMCC  network implementation

Proposal 3: Release occasion is based on network implementation of gNB-DU and do not introduce any pre-configured conditions or new releasing the positioning context IE over F1 interface.

[7], Samsung  check RAN2’s progress

Proposal 7: RAN3 checks RAN2’s progress on how to release the SRS configuration both in gNB and UE during the RAN3 115e meeting.

Based on the above discussion, the moderator formulates below proposals.

### Q4, do companies agree with below proposals on how to release the reserved SRS?

- Proposal 1, releasing the SRS configuration both in gNB and UE side at the same time should be ensured in order to avoid UL inerference.

- Proposal 2, RAN3 agrees to reuse positioning deactivation to release SRS configuration when UE resumes to a new cell/gNB, i.e. the anchor gNB receives Retrieve UE Context Request from other gNBs.

| Company | Comment |
| --- | --- |
| Samsung | Yes. |
| HW | Yes. |
| CATT | Yes |
| Ericsson | Yes |
| Qualcomm | Yes |
| vivo | Yes with comments, RAN2 already agreed that the SRSp configuration is released autonomously when the UE sends RRCResumeRequest to a cell other than the cell where it is released to RRC_INACTIVE state. |
| CMCC | Yes |

Moderator’s summary:

All companies agree the proposals. Although these proposals have no spec impact, they’re needed for RAN3 to make sure we have the same understanding, no enhancements are needed at current stage.

Proposal 5, releasing the SRS configuration both in gNB and UE side at the same time should be ensured in order to avoid UL interference.

Proposal 6, RAN3 agrees to reuse positioning deactivation to release SRS configuration when UE resumes to a new cell/gNB, i.e. the anchor gNB receives Retrieve UE Context Request from other gNBs.

Positioning impact on Xn

Positioning context related

RAN3 114bis-e meeting, RAN3 agreed to Include Routing ID and Requested SRS transmission characteristics in RETRIEVE UE CONTEXT RESPONSE message over XnAP, other information is FFS. In contribution [7] and [10], it is proposed to include NRPPa Transaction ID in the RETRIEVE UE CONTEXT RESPONSE message so that the new serving gNB can send include the NRPPa Transaction ID in the NRPPa messages sent to LMF to let the LMF knows which Transaction this message belongs. In addition, [7] also proposed to include the History SRS configuration which can help the new serving gNB allocate a more proper SRS resources. In addition, contribution [6] proposed to include an explicit IE for indicating location event is introduced within Retrieve UE Context Request message.

### Q5, do companies agree with below enchantments for UE context procedures?

- 1), include an explicit IE for indicating location event within RETRIEVE UE CONTEXT RESPONSE REQUEST message.

- 2), include NRPPa Transaction ID in RETRIEVE UE CONTEXT RESPONSE message, to make sure LMF knows the same transaction (i.e. the same positioning UE).

- 3), include History SRS in RETRIEVE UE CONTEXT RESPONSE message, to help gNB configure SRS.

| Company | Comment |
| --- | --- |
| Samsung | We see benefits on the bullet 1), but it may be discussed in RAN2 firstly, maybe we can check RAN2’s progress during the meeting and further discuss this in the 2nd round. Bullet 2) is definitely needed, otherwise the LMF cannot know the message from the new serving gNB belongs to which transaction, the positioning procedure cannot go on without this information. Bullet 3) is beneficial. The Requested SRS transmission characteristics from LMF are kind of recommendations, the serving gNB may not configure the SRS the same as recommended from LMF according to the actual radio conditions of the UE, so we feel like the SRS configuration (if any) of the last serving cell would be more close to the real situation of the UE. |
| HW | - No. The event report is encapsulated in the NAS message. The serving gNB has no chance to know what information is included in the NAS message during the SDT procedure. It is impossible. - OK for transaction ID. - No need. The SRS is requested by the LMF, rather than the old gNB. So the Requested SRS transmission characteristics IE is enough. More importantly, the SRS may be not available in the old gNB, because the old gNB may not be engaged in a UL positioning before. Normally the SRS is encapsulated in the CellGroupConfig IE and is transparent to the CU. The SRS is only available at the CU if there was a UL positioning or UL+DL positioning at the old gNB. |
| CATT | - No, same view with HW. - Ok for transaction ID. - History SRS configuration is not essential for the target gNB. |
| Ericsson | Disagree on all. 1) same view as Hw. 2) Hope we are not mixing between Transaction IDs and Measurement IDs. A Transaction ID is a pure protocol-level concept that has nothing to do with measurements or the semantics of what is carried. Whenever a new procedure instance is used, the transaction ID in that AP will be different. Also, our understanding is that Transaction IDs are only used for non UE associated procedures and not for Class 2 (despite that some Class 2 procedures in NRPPa have it, which was a mistake...) 3) the new gNB will re-configure the UE SRS transmission anew, depending on LMF’s request and the gNB’s local radio conditions; it can totally disregard this history info. This is not needed. |
| vivo | 1) No, the indication is implicitly informed by the Requested SRS transmission characteristics 2) Yes, to associate with a specific transaction. 3) No strong view, seems beneficial if the new serving cell can configure the same SRS, then the LMF does not update the SRS configuration to other related gNBs. |
| CMCC | 1) Our intention is to introduce a Location Event Indication IE assisting serving gNB retrieving the positioning context from anchor gNB. If the majority of views do not want the enhancement, we are also OK. 2) OK. 3) The history SRS configuration can not help gNB configure SRS since the local environment for new serving gNB is totally different from the old gNB. |

Moderator’s summary:

5 out of 6 companies agree to include NRPPa Transaction ID in the positioning context, 1 company argued that NRPPa Transaction ID is not for UE associated NRPPa messages, but the argument is not aligned with the current specification.

Proposal 7, RAN3 agrees to include NRPPa Transaction ID in RETRIEVE UE CONTEXT RESPONSE message as positioning information, to make sure LMF understand the message from the same NRPPa transaction.

Proposal 8, agree the TP in revision of R3-221893 (CATT) to capture the agreements on NRPPa Transaction ID.

In case of without anchor relocation

In the last RAN3 114bis-e meeting, RAN3 had below open issue to be further discussed.

The case of without anchor relocation for RRC_INACTIVE UE positioning. (e.g. identify the issues or possible enhancement) based on the progress of SDT WI.

And the following are the related proposals from companies.

[3], Huawei  postpone the discussion

Proposal 3: Postpone the discussion of supporting the case of without anchor relocation for the UL and UL+DL positoining, either in Rel-18, or at least after the SDT discussion is complete.

[4], CATT  introduce NRPPa container over Xn

Proposal 1: RAN3 is request to discuss which option to go on handling of the UE specific DL NRPPa message during SDT or in normal inactive state.

-	Option 1: the anchor fails the NRPPa procedure with proper cause value and relocates the UE context to the new gNB.

-	Option 2: the anchor gNB forwards the received NRPPa PDU and corresponding routing ID to the new gNB, the new gNB handles the NRPPa message and responses the LMF accordingly.

Proposal 2: Discuss and agree the TP for Xn, if decide to transfer the received DL NRPPa message in the Xn (option 2).

[5], CMCC  fail the positioning procedure in case of without anchor relocation

Proposal 2: Support to introduce the NRPPa failure message from anchor gNB to LMF if without anchor relocation.

[7], Samsung  introduce NRPPa PDU transfer message over Xn

Proposal 3: RAN3 agrees to introduce a new Xn message (e.g. NRPPa Transfer) including NRPPa PDU to deliver the NRPPa message between the new serving cell and LMF in case of SDT without anchor relocation.

There’re four companies discussed this open issue. Although two companies showed their concerns on the impact that may depend on SDT and the limited time left for R17 in [3] and [5], two companies had already proposed solutions for the case without anchor relocation. It’s the moderator’s understanding that SDT is only responsible for the delivery of the messages over SRB, the transmission of NRPPa during on-going SDT should be discussed in Positioning WI. Since we already have solutions on the table, it’s no harm to have a try, and below figure is for the reference.

### Q6, which option can be used to support positioning information exchange in case of without anchor relocation?

- Option 1, introduce NRPPa PDU over Xn

- Option 2, send NRPPa failure message in case of without anchor relocation

| Company | Comment |
| --- | --- |
| Samsung | Option 1. If option 2 is used, it means that the RRC INACTIVE positioning cannot be supported. |
| HW | If we complete this in R17, we strongly have the preference of include explicit IE in the newly defined “Partial UE Context Retrieve procedure” in SDT WI. Note that the SRS configuration has to be explicitly included in the response message, so we should use explicit IEs. We can simply include the previously defined “Positioning Information” IEs: 9.1.1.xx PARTIAL UE CONTEXT RETRIEVE REQUEST This message is sent by the old NG-RAN node to transfer part of the UE Context to the new NG-RAN node. Direction: old NG-RAN node new NG-RAN node. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3.1; YES; reject. New NG-RAN node UE XnAP ID reference; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the new NG-RAN node.; YES; ignore. Old NG-RAN node UE XnAP ID reference; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the old NG-RAN node.; YES; ignore. Partial UE Context Information for SDT; O; 9.2.3.y; YES; ignore. Positioning Information; O; 9.2.3.x. 9.1.1.yy PARTIAL UE CONTEXT RETRIEVE RESPONSE This message is sent by the new NG-RAN node to acknowledge the retrieval of part of the UE context from the old NG-RAN node. This message is also used to provide data forwarding related information for NR SDT or positioning. Direction: new NG-RAN node old NG-RAN node. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3.1; YES; reject. New NG-RAN node UE XnAP ID reference; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the new NG-RAN node; YES; ignore. Old NG-RAN node UE XnAP ID reference; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the old NG-RAN node; YES; ignore. SDT Data Forwarding DRB List; 0..1; YES; ignore. >SDT Data Forwarding DRB Item; 1..<maxnoofDRBs>. >>DRB ID; M; 9.2.3.33. >>DL TNL Information; O; UP Transport Layer Information 9.2.3.30. SRS configuration; O. Range bound; Explanation. maxnoofDRBs; Maximum no. of DRBs. Value is 32.. |
| CATT | We do not expect to proceed with the DL NRPPa as the figure shown above. In [4], we think the handling of UE specific DL NRPPa message received in the anchor during SDT could be aligned with non-SDT handling during SDT. Anchor gNB could release the UE and re-initiate another RRC Resume procedure for the DL NRPPa handling, then anchor gNB may relocate the UE context together with the NRPPa PDU, the new gNB becomes the new anchor and handles the NRPPa PDU accordingly. Another way is just to go for the option 1 in Rel-17, just like the handling of DL NAS PDU in RRC_INACTIVE. Which means option 1 is feasible, and we should not say without NRPPa-PDU transfer over Xn, the Positioning in Inactive is not supported. As the ongoing SDT procedure is being used for positioning report in Inactive, we understand the DL NRPPa message is not tightly related to the Inactive positioning, which means the DL NRPPa message will not always come during SDT. Maybe we could consider the option 1 in Rel-17. |
| Ericsson | We must not add “specification impacts” on top of something that is currently being specified (and that is already quite controversial in the corresponding WI). In our opinion, this is not a correct way of working in 3GPP (e.g., how would TPs be handled if we impact procedures in the BL CR of another WI? How would stage 2/3 modifications be handled across different WIs??). Since this is the last e-meeting of the Release, we strongly suggest taking the case of positioning with SDT without anchor relocation for Rel-18, once the SDT XnAP design has been stabilized. |
| CMCC | In Rel-17, considered the limited time, we prefer to take no enhancement over Xn in the case of without anchor relocation for RRC_INACTIVE UE positioning using SDT. We prefer option 2 which is a simple way of sending NRPPa failure message to LMF. |

Moderator’s summary:

2 companies support option 1, one company support option 2, one company proposed to enhance partial UE context retrieve message in SDT, 2 companies have concerns that the impact depends on SDT progress. Since we still have another round, let’s discuss it based on the progress of SDT to see how far we can go, which may also help the discussion in R18.

As analyzed in [3] and [7], if the NRPPa message relates to UL positioning, the SRS configuration should be transmitted from the serving gNB to the anchor gNB to assemble the RRC release message to include the SRS configuration.

### Q7, do companies agree to include SRS configuration as an explicit IE over XnAP so that the anchor gNB can assemble the RRC Release message?

| Company | Comment |
| --- | --- |
| Samsung | Yes |
| Huawei | Yes if we complete this in R17. See comments in Q6. |
| CATT | It seems this is the special case of Q5, where the DL NRPPa message received in the anchor gNB is SRS related. For this case, If we try to do something in Rel-17, maybe we could consider HW’s proposal as been provided in the comments to Q6. |
| Ericsson | How can the Retrieve UE Context Failure be triggered at step 11? This is not a Class 2 procedure? The stage 2 framework requires more analysis. We strongly suggest to consider positioning in SDT w/o anchor relocation in R18. |
| CMCC | Suggest to postpone in Rel-18. |

Moderator’s summary:

Let’s discuss HW’s proposal in 2nd round.

NRPPa failure message

In contribution [10], it is proposed to let the new serving gNB send the NRPPa failure message including the new cause value to LMF to trigger the retransmission of NRPPa message

### Q8, do companies agree with the above proposal?

| Company | Comment |
| --- | --- |
| Samsung | Our understanding is that the anchor gNB that receives the NRPPa request should send the failure message to LMF, after the path switch, the AMF will notify the LMF UE moves to a new serving gNB, the LMF can retransmit the NRPPa request message to the new serving gNB. |
| HW | We don’t think the failure message of a class 1 procedure can be sent from a different node. |
| CATT | Actually, the [10] discussed how to handle the DL NRPPa for a UE in RRC Inactive, not for SDT case. Handling of NAS-PDU in Inactive could be taken as reference, i.e. anchor gNB could send the failure message towards the LMF. After Path switch, LMG may re-initiate the DL NRPPa procedure towards the new serving gNB. |
| Ericsson | This needs more thoughts and discussion. We strongly suggest to consider positioning in SDT w/o anchor relocation in R18. |
| vivo | We also think it’s straightforward to let the anchor gNB send the failure message to LMF. However, the timing shall be clarified: 1. If sent by the last serving gNB upon receiving Retrieve UE Context Req message, the path switch may have not complete yet and the NRPPa message is retransmitted to the anchor gNB. To address the issue, a delay timer may associate with the failure message. 2. If sent by the last serving gNB upon receiving the RRC Context Release message, the AMF shall discard the failure message and LMF will not retransmit the NRPPa message. The behavior of AMF shall be clarified that it will not discard the failure message even the path switch happens. |
| CMCC | As Samsung mentioned, anchor gNB that receives the NRPPa request should send the failure message to LMF. The clarification proposed by vivo need to be further confirmed by all companies. |

Moderator’s summary:

This can be further discussed in 2nd round.

Others

There’s an LS [1] on the DRX cycle used in PRS measurement from RAN4, but there’s no discussion about the LS in the contributions. If companies have any view or way forward on the LS, please provide your comment.

| Company | Comment |
| --- | --- |
| Qualcomm | This means that the UE response time can be different in different RRC states, and therefore an LMF would need to take this into account when requesting location information from a UE. |

Moderator’s summary:

Seems no specification impact in RAN3, the moderator suggests to note the LS.

Conclusion

References

| [1] | R3-221659 | LS on DRX cycle used in PRS measurement in RRC_INACTIVE state (RAN4) | LS in |
| --- | --- | --- | --- |
| [2] | R3-221747 | (TP for NRPPa baseline) LMF Assistance Information to support positioning in RRC_INACTIVE state (Qualcomm Incorporated) | Other |
| [3] | R3-221881 | (TP for POS BL CR for TS 38.455, TS 38.473) on RRC_INACTIVE positioning (Huawei) | other |
| [4] | R3-221893 | (TP for Positioning BL CR 38.423) Positioning support in Inactive (CATT) | other |
| [5] | R3-221948 | (TP for NRPPa BL CR on Positioning) Completion of RRC_INACTIVE assistance data for positioning (Ericsson) | other |
| [6] | R3-222257 | Discussion on RRC INACTIVE State Positioning (CMCC) | Discussion |
| [7] | R3-222285 | Positioning in RRC inactive state (Samsung) | discussion |
| [8] | R3-222286 | (TP for BL CR TS38.423) RRC Inactive positioning (Samsung) | other |
| [9] | R3-222287 | (TP for BL CR TS38.473) RRC Inactive positioning (Samsung) | other |
| [10] | R3-222342 | Discussion on RRC_INACTIVE state Positioning (VIVO TECH GmbH) | Discussion |

Annex A (LCS message, TS24.008, TS 29.002)

LCS-PeriodicTriggeredInvokeArg	::= SEQUENCE {

referenceNumber	[0]	LCS-ReferenceNumber,

h-gmlc-address	[1]	GSN-Address,

qoS		[2] LCS-QoS		OPTIONAL,

reportingPLMNList	[3]	ReportingPLMNList	OPTIONAL,

periodicLocation	[4]	PeriodicLocation	OPTIONAL,

areaEventReporting	[5]	AreaEventReporting	OPTIONAL,

motionEventReporting	[6]	MotionEventReporting	OPTIONAL,

PeriodicLocation	::= SEQUENCE {

periodicLDRInfo	[0]	PeriodicLDRInfo,

... }

PeriodicLDRInfo ::= SEQUENCE {

reportingAmount	ReportingAmount,

reportingInterval	ReportingInterval,

...}

-- reportingInterval x reportingAmount shall not exceed 8639999 (99 days, 23 hours,

-- 59 minutes and 59 seconds) for compatibility with OMA MLP and RLP

ReportingAmount ::= INTEGER (1..maxReportingAmount)

maxReportingAmount INTEGER ::= 8639999

ReportingInterval ::= INTEGER (1..maxReportingInterval)

-- ReportingInterval is in seconds

maxReportingInterval INTEGER ::= 8639999

AreaEventReporting	::=	SEQUENCE {

deferredLocationEventType	[0] DeferredLocationEventType,

areaList		[1]	AreaList,

occurrenceInfo	[2]	OccurrenceInfo	OPTIONAL,

intervalTime	[3]	IntervalTime	OPTIONAL,

maximumInterval	[4]	MaximumInterval	OPTIONAL,

samplingInterval	[5] SamplingInterval	OPTIONAL,

duration		[6]	Duration	OPTIONAL,

locationInfo	[7]	LocationInfo	OPTIONAL,

... }

OccurrenceInfo ::= ENUMERATED {

oneTimeEvent	(0),

multipleTimeEvent	(1),

...}

IntervalTime ::= INTEGER (1..32767)

-- minimum interval time between area reports in seconds

MaximumInterval ::= INTEGER (1..86400)

-- maximum time interval between consecutive event reports in seconds.

SamplingInterval ::= INTEGER (1..3600)

-- maximum time interval between consecutive evaluations by a UE of a trigger event,

-- in seconds.

Duration ::= INTEGER (1..8640000)

-- maximum duration of event reporting by a UE, in seconds

- Annex B (LPP message, TS 37.355)

#### CommonIEsRequestLocationInformation

The CommonIEsRequestLocationInformation carries common IEs for a Request Location Information LPP message Type.

-- ASN1START

CommonIEsRequestLocationInformation ::= SEQUENCE {

locationInformationType		LocationInformationType,

triggeredReporting			TriggeredReportingCriteria	OPTIONAL,	-- Cond ECID

periodicalReporting			PeriodicalReportingCriteria OPTIONAL,	-- Need ON

additionalInformation		AdditionalInformation		OPTIONAL,	-- Need ON

qos							QoS							OPTIONAL,	-- Need ON

environment					Environment					OPTIONAL,	-- Need ON

locationCoordinateTypes		LocationCoordinateTypes		OPTIONAL,	-- Need ON

velocityTypes				VelocityTypes				OPTIONAL,	-- Need ON

...,

[[

messageSizeLimitNB-r14	MessageSizeLimitNB-r14		OPTIONAL	-- Need ON

]],

[[

segmentationInfo-r14	SegmentationInfo-r14		OPTIONAL	-- Need ON

]]

}

...

PeriodicalReportingCriteria ::=		SEQUENCE {

reportingAmount						ENUMERATED {

ra1, ra2, ra4, ra8, ra16, ra32,

ra64, ra-Infinity

} DEFAULT ra-Infinity,

reportingInterval					ENUMERATED {

noPeriodicalReporting, ri0-25,

ri0-5, ri1, ri2, ri4, ri8, ri16, ri32, ri64

}

}