3GPP TSG-RAN WG3 #115-e                                                                         R3-222486

21 Feb - 3 Mar 2022

Online

Agenda item:	11.3

Source: 	ZTE, Ericsson

Title: 	(TP for RedCap BL CR 38.470) RedCap Paging

Document for:	Discussion and Approval

1. Introduction

In order to support NR eDRX for RedCap UE, the CU needs to provide NR eDRX information to DU, and this document updates the corresponding stage 2 text for TS38.470 BLCR[1].

2. References

[1] R3-221533, BL CR to 38.470 Support for Redcap Ues, ZTE, Nokia, Nokia Shanghai Bell, Ericsson, Samsung, CATT, Qualcomm Incorporated, Radisys, Reliance JIO

- Text Proposal for TS38.470 BLCR

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BH	Backhaul

DRB	Data Radio Bearers

F1-U	F1 User plane interface

F1-C	F1 Control plane interface

F1AP	F1 Application Protocol

GTP-U	GPRS Tunnelling Protocol

IAB	Integrated Access and Backhaul

IP	Internet Protocol

NR-MIB	NR-Master Information Block

O&M	Operation and Maintenance

PA	Paging Area

PF	Paging Frame

PO	Paging Occasion

PH	Paging Hyperframes

eDRX	extended Discontinuous Reception

QoS	Quality of Service

RIM	Remote Interference Management

RLC	Radio Link Control

RRC	Radio Resource Control

SCTP	Stream Control Transmission Protocol

SRB	Signalling Radio Bearers

SIB1	System Information Block 1

SIB10	System Information Block 10

SIB12	System Information Block 12

SIB13	System Information Block 13

SIB14	System Information Block 14

SL	Sidelink

TNL	Transport Network Layer

V2X	Vehicle-to-Everything

<<<<<<<<<<<<<<<<<<<< NEXT OF CHANGES >>>>>>>>>>>>>>>>>>>>

5.2.5	Paging function

The gNB-DU is responsible for transmitting the paging information according to the scheduling parameters provided.

The gNB-CU provides paging information to enable the gNB-DU to calculate the exact PH, if the eDRX is configured, and PO and PF. The gNB-CU determines the PA. The gNB-DU consolidates all the paging records for a particular PH,  PO, PF and PA, and encodes the final RRC message and broadcasts the paging message on the respective PH, PO, PF in the PA.

<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>