3GPP TSG-RAN WG3 Meeting #115-e	R3-222453

Online, 21 February – 03 March 2022

Agenda item:	21.2

Source:	Nokia (moderator)

Title:	Summary of offline: time synchronisation enhancements

Document for:	Discussion and Decision

# 1	Introduction

This paper summarizes the following email discussion:

CB: # NRIIOT2_PDC

- The Uu time synchronization error budget is introduced over F1 interfaces?

- Introduce a new Measurement and report procedure to support TA-based and/or RTT-based PDC?

- The UE-associated signalling is needed over F1AP to provide gNB Rx-Tx time difference measurement (RTT-based PDC) and/or TA measurement (TA-based PDC) from gNB-DU to gNB-C?

- Whether the gNB-CU needs to provide the TRS/PRS and SRS configuration information to gNB-DU in RTT measurement

- During the handover, what kinds of time synchronization assistance information should the source gNB notify the target gNB, e.g., Uncertainty, Time Information Type, TSN distribution, Periodicity…?

- Capture agreements and provide TPs if agreeable

(Nok - moderator)

Summary of offline disc R3-222453

# 2	For the Chair’s Notes

R3-222561 rev of R3-221874 is agreed (TP to TS 38.473)

R3-222581 is endorsed (CR to TS 38.470)

# 3	Discussion (Round 1)

Please provide your Round 1 views (5 questions) by 11:00 UTC Wednesday February 23rd, so that comments may be taken into account during the online discussion later that day.

## 3.1	Propagation Delay Compensation

RAN3 received an LS from RAN2 in [1], listing their agreements on Propagation Delay Compensation (PDC). In this 1st round of discussion, it is proposed to focus on the following RAN3 aspects:

a)	Signalling framework over F1AP for RTT-based PDC and TA-based PDC

b)	Whether PD pre-compensation is performed by the gNB-CU or by the gNB-DU

Also, it is proposed to first focus on unicast delivery of RTI without any ongoing NR positioning (e.g. E-CID positioning using “NR TADV” has not been initiated by the LMF). Potential specification impacts of broadcast delivery of pre-compensated RTI and of concurrent NR positioning procedures can be handled in 2nd round discussion.

Overview of related papers:

Company views on the high level PDC signalling framework are summarized in Table 1 below.

| Company | RTT-based PDC at gNB/UE | TA-based PDC at gNB |
| --- | --- | --- |
| Nokia [2] | New UE-associated Class 1 request (CU-initiated) New UE-associated Class 2 report (DU-initiated) | Same as RTT-based PDC |
| ZTE [4][5] | New IE in UE Context Setup/Modification New UE-associated Class 2 report (DU-initiated) | ? |
| Huawei [7][8] | New UE-associated Class 1 request (CU-initiated) New UE-associated Class 2 report (DU-initiated) | New IE in RTI Reporting Control / RTI Report |
| Ericsson [10] | ? | New UE-associated Class 1 request (CU-initiated) |
| CATT [11] | New UE-associated Class 1 request (CU-initiated) New UE-associated Class 2 report (DU-initiated) | Same as RTT-based PDC |

Table 1: Company views on PDC signalling framework

Company views on which node performs PD pre-compensation are summarized in Table 2 below.

| Company | RTT-based PDC at gNB/UE | TA-based PDC at gNB |
| --- | --- | --- |
| Nokia [2] | CU performs PD pre-compensation | CU performs PD pre-compensation |
| ZTE [4][5] | CU performs PD pre-compensation | ? |
| Docomo [6] | CU performs PD pre-compensation | CU performs PD pre-compensation (unicast) DU performs PD pre-compensation (broadcast/SIB9) |
| Huawei [7][8] | CU performs PD pre-compensation | DU performs PD pre-compensation |
| Ericsson [10] | ? | CU performs PD pre-compensation |
| CATT [11] | CU performs PD pre-compensation | DU performs PD pre-compensation (slight preference) |

Table 2: Company views on node that performs PD pre-compensation

Moderator’s observations for RTT-based PDC:

-	5 of 5 companies propose that CU performs PD pre-compensation.

-	4 of 4 companies propose to introduce a new UE-associated Class 2 procedure to enable DU to report gNB Rx-Tx time difference to CU.

-	3 of 4 companies propose to also introduce a new UE-associated Class 1 procedure to enable CU to request (either “on-demand” or “periodic”) gNB Rx-Tx time difference from DU. The remaining 1 company proposes to reuse the UE Context Setup/Modification procedure for the same purpose.

Based on the above observations, it seems there is (near-)consensus on the following:

Proposal #1:	For RTT-based PDC, PD pre-compensation is performed by CU.

Proposal #2:	To support RTT-based PDC, introduce a new UE-associated class 1 procedure (CU-initiated) and a new UE-associated class 2 procedure (DU-initiated) to enable reporting of gNB Rx-Tx time difference from DU to CU.

Question 1: Can Proposals #1 & #2 be agreed for RTT-based PDC?

NOTE: If proposal 2 is agreeable, then details of the new procedures can be discussed in the 2nd round.

| Company | Comments |
| --- | --- |
| Huawei | Agree the two proposals. And another reason why a new UE-associated procedure is needed instead of reusing the positioning message or E-CID measurement, is that the POSITIONING MEASUREMENT REQUEST/ RESPONSE carrying the gNB Rx-Tx Time Diff are non-UE associated messages and E-CID measurement will also be involved with LMF besides the UE and gNB. |
| ZTE | Yes for proposal 1 For proposal #2, similar as that in UU interface that RRCReconfiguration includes the measurement configuration and the measurement results are included in a UE specific message MeasurementReport, we prefer in F1AP to introduce a new IE in existing signalling(UE Context Setup /Modification Request) to configure the measurement control information, and a new UE-associated class 2 procedure (DU-initiated) to enable reporting of gNB Rx-Tx time difference from DU to CU. In this manner, a DL message can be saved and can decrease the measurement configuration delay. |
| CATT | Agree to the two proposals. Use the new message is more clear. |
| Samsung | We support two proposals. |
| Nokia | Yes, for both proposals. |
| Ericsson | Proposal 1 is fine. Proposal 2, about the DU to CU reporting, we could also consider to report a group of UEs in one message. |
| Qualcomm | Yes for both |
| Moderator Summary: - 7 companies responded - For Proposal 1, all 7 companies agree. - For Proposal 2, 6 companies agree to introduce a new UE-associated class 1 procedure and 6 companies agree to introduce a new UE-associated class 2 procedure. However, 1 company prefers to reuse the UE Context Setup/Modification procedure rather than a new class 1 procedure, while another company proposes to consider a new non-UE associated class 2 procedure (which would be needed “to report a group of UEs in one message”?) rather than a new class 2 procedure Proposed conclusion: - Agree to Proposal 1 (consensus). - Agree to Proposal 2, since 6 of 7 companies agree to introduce a new UE-associated class 1 procedure (CU-initiated), and 6 of 7 companies agree to introduce a new UE-associated class 2 procedure (DU-initiated). |  |

Moderator’s observations for TA-based PDC:

-	3 of 5 companies propose that CU performs PD pre-compensation, while the remaining 2 companies propose that DU performs PD pre-compensation.

-	3 of 4 companies propose to introduce a new UE-associated Class 1 procedure to enable CU to request (either “on-demand” or “periodic”) TA-based PDC information from DU. The remaining 1 company proposes to enhance the existing non-UE associated Reference Time Information Reporting Control and Reference Time Information Report procedures to carry UE-specific information.

-	2 companies additionally indicate that common procedures can be used for both TA-based and RTT-based PDC.

Based on the above observations, companies are split whether PD pre-compensation is performed by CU or DU, i.e. whether DU reports NR Timing Advance or compensated RTI. Regardless of this, all but one company proposes to introduce new UE-associated procedures for TA-based PDC, while the one company that proposes to reuse non-UE associated procedures indicated at last RAN3 meeting that new UE-associated F1AP procedure is acceptable (see [13] question #6).

Therefore, the following is proposed:

Proposal #3:	To support TA-based PDC, introduce a new UE-associated class 1 procedure (CU-initiated) and a new UE-associated class 2 procedure (DU-initiated) to enable reporting of “TA-based PDC information” (details FFS).

Issue-1:	Further discuss whether the “TA-based PDC information” is:

a)	NR Timing Advance (meaning PD pre-compensation is performed by CU) or

b)	compensated RTI (meaning PD pre-compensation is performed by DU).

Question 2: Can Proposal #3 be agreed for TA-based PDC? If so, can TA-based PDC and RTT-based PDC use common procedures (i.e. the new procedures of Proposal #2 and #3 can be the same)?

| Company | Comments |
| --- | --- |
| Huawei | We agree this proposal 3 under conditions that we don’t intend to combine the new TA-based PDC with the existing non-UE associated Reference Time Information Reporting Control procedure. For example, we don’t want to specify that the CP has to calculate the final PDC value after acquiring the node level Time Reference Information from the REFERENCE TIME INFORMATION REPORT message, and acquiring the UE level TA-based PDC value from another new procedure. If this can be met, we are also fine to have a common UE-associated procedure for TA-based and RTT-based PDC. |
| ZTE | Yes, we agree with Proposal 3. But based on the comments of Q1, we do not agree that the new procedures of Proposal2 and Proposal3 are the same. |
| CATT | Agree the proposal 3. The new message can be used for both TA and RTT |
| Samsung | Yes, we agree with Proposal 3. And we prefer common procedure for TA-based PDC and RTT-based PDC. |
| Nokia | Yes, for proposal 3. We prefer common F1AP procedures for TA-based and RTT-based PDC. |
| Ericsson | Ok with Proposal 3. In addition, we think the existing E-CID, which is UE based, can also be considered, if such has started. |
| Qualcomm | Ok with proposal 3. In addition, we don’t see a strong reason not to attempt common procedure(s) for TA and RTT. |
| Moderator Summary: - 7 companies responded. - 6 companies agree with Proposal 3, while 1 company can agree with Proposal 3 with a caveat that we don’t combine the new TA-based PDC with the existing non-UE associated Reference Time Information Reporting Control procedure. Proposed conclusion: - Agree to proposal 3. The caveat from Huawei can be taken into account during the 2nd round when discussing further details of the new procedures. |  |

Question 3: For TA-based PDC, what does the gNB-DU report to the gNB-CU: (a) NR Timing Advance or (b) compensated RTI?

| Company | Comments |
| --- | --- |
| Huawei | We think the key problem is not which node (CU/DU) performs the PDC. For unicast message, anyway, it is the CU that generates compensated Reference Time Information to the UE, based on the information sent from the DU. So the key problem is about the exact information and its granularity of the information from the DU to CU. - For a), as discussed in R3-221967, the TA indicating granularity is 16∙64∙Tc/2μ , (about 520ns) which may cause an additional sync error. - For b), the granularity is in terms of 10ns (by referring to the Reference Time broadcast in SIB9 in RRC specification), which is more accurate. So b) is our preference. |
| ZTE | We think PD pre-compensation in gNB can only be performed by CU for unicast case, and RRC signalling is encoded in CU, so (a) is a good choice. |
| CATT | We prefer b). Share with Huawei. |
| Samsung | We’re fine with (a) to be aligned with RTT-based PDC, i.e. CU performs PD pre-compensation. |
| Nokia | We prefer (a). Regarding whether there is any difference in accuracy/granularity between (a) and (b), the comments from Huawei assume that TA would be signalled over F1AP with same granularity as the signalling over Uu (i.e. granularity of 521ns, 260ns, 130ns, 65ns, 33ns for µ = 0, 1, 2, 3 and SCS 15kHz, 30kHz, 60kHz and 120kHz respectively). However, we believe that TA with finer granularity than 16∙64∙Tc/2μ could be signalled over F1AP. For example, Rx-Tx measurement report with k=5 gives 32ns (64Tc) resolution for TA, so the Rx-Tx granularity could be allowed for reporting TA over F1AP. Then, there would be no meaningful difference in accuracy between (a) and (b). For example: INTEGER (0..62500) in units of 64Tc (where max value is roughly aligned with NTA range) |
| Ericsson | a). It is more logical for CU to perform compensation. |
| Qualcomm | Prefer a). Also Nokia’s suggestion should be considered. |
| Moderator Summary: - 7 companies responded - 5 companies prefer (a) while 2 companies prefer (b). Proposed conclusion: - Further discuss online. Most companies prefer (a), and perhaps the suggested encoding to use units of 64Tc might address the concerns of the 2 companies the prefer (b)? |  |

## 3.2	Uu time synchronisation error budget over F1

Overview of related papers:

-	2 companies (ZTE in [4][5] and CATT in [12]) propose to introduce a “cell level” Uu Time Synchronisation Error Budget over F1AP in the REFERENCE TIME INFORMATION REPORTING CONTROL message. The “cell level” value is e.g. the most stringent value among the UE-specific values provided over NGAP from the CN.

-	1 company (Nokia in [2]) believes that a “cell level” Uu Time Synchronisation Error Budget is useful for the DU to derive SIB9 periodicity for broadcast-based RTI delivery due to UE clock drift. However, it may be sufficient in Rel-17 to assume that DU is configured via OAM with an appropriate SIB9 periodicity value.

-	1 company (Huawei in [8]) believes that Uu Time Synchronisation Error Budget is only useful for PDC decision by the CU, and therefore is not needed by the DU.

Moderator’s Summary:

Companies are split whether DU needs to know the Uu Time Synchronisation Error Budget. Among companies who do see the need, there are differing views on why (e.g. accuracy of unicast RTI delivery or periodicity of broadcast RTI delivery).

Issue-2:	Further discuss whether:

a)	CU provides a “cell level” Uu Time Synchronisation Error Budget (e.g. the most stringent value among the UE-specific values provided over NGAP from the CN) in the REFERENCE TIME INFORMATION REPORTING CONTROL message; or

b)	No need to signal any Uu Synchronization Error Budget over F1 in Release 17.

Question 4: Should Uu Time Synchronisation Error Budget (cell level, e.g. most stringent value among the UE-specific values provided over NGAP from the CN) be included in the REFERENCE TIME INFORMATION REPORTING CONTROL message?

| Company | Comments |
| --- | --- |
| Huawei | We don’t see the need, as discussed in R3-221968 |
| ZTE | Yes. |
| CATT | If the DU perform the PDC, this information is needed in DU |
| Samsung | We’re fine with b) in Rel.17. |
| Nokia | We can accept (b). |
| Ericsson | b) would be fine in order to conclude at this meeting. |
| Qualcomm | The use cases still seem marginal, so prefer b). In any case if it is a clear omission, and a very clear new argument can be made, this can always be revisited either as a correction or in rel-18. |
| Moderator Summary: - 7 companies responded - 2 companies support including the Uu Synchronization Error Budget in REFERENCE TIME INFORMATION REPORTING CONTROL message, while 5 companies indicate that there is no need to signal any Uu Synchronization Error Budget over F1 in Release 17. Proposed conclusion: - No consensus to include the Uu Synchronization Error Budget over F1 in Release 17. |  |

## 3.3	Time synchronization: XnAP open issues

The following are the relevant agreements / open issues already captured in the Chair’s minutes:

Introduce the Time Synchronisation Assistance Information IE as an optional UE-level parameter in

-	XnAP (HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE)

It is FFS on whether assistance information (e.g., UE TSN timing reference, referenceTimeInfo delivery periodicity, timestamp) should be delivered during HO.

Overview of RAN3 papers:

-	Nokia [3]: The Time Synchronization Assistance Information IE already agreed to be exchanged over XnAP is enough for the target gNB to determine the delivery configuration of RTI to the UE in the new cell. Therefore, no additional time synchronisation assistance information is needed over XnAP in Release 17.

-	Huawei [7]: The RTI configuration in the source gNB can be used by the target gNB to determine the suitable time sync delivery mode and delivery period after the handover completion. At least the RTI periodicity is needed.

-	Ericsson [9]: It is beneficial for the target gNB to know the UE’s TSN time reference information used in the source gNB as early as possible during NG and Xn handover. Therefore, the TSN Time Reference Information (Uncertainty, Time Information Type, TSN distribution, and Periodicity) should be included in the HANDOVER REQUEST message.

Moderator’s Summary:

To assist the target gNB in deciding the RTI delivery configuration, two companies believe it is useful for the source gNB to provide the RTI Periodicity in the HANDOVER REQUEST message. In addition, one company believes that RTI Uncertainty, Time Information Type, and TSN Distribution are also useful.

This topic has been discussed for several meetings without any change to company opinions. Since this is the last meeting of the work item, the moderator suggests to first focus on RTI periodicity which has support from at least 2 companies.

Issue-3:	Further discuss whether to enable source gNB to provide RTI periodicity to the target gNB in the HANDOVER REQUEST message.

Question 5: In case of handover, should RTI periodicity be included in the HANDOVER REQUEST message?

| Company | Comments |
| --- | --- |
| Huawei | Yes. This can help the target gNB to determine the delivery period after the handover completion, based on the information from the source gNB. This can be seen as reference information to be considered by the target gNB. |
| ZTE | No. Based on the RAN2 specification, the UE provides only the referenceTimeInfoPreference to the gNB and it can already be delivered to the target gNB. The target gNB can obtain all the RTI related info. urthermore, RAN2 has not agreed to introduce RTI periodicity. |
| CATT | No |
| Samsung | No. We have the similar view as Nokia in [3]. |
| Nokia | No. |
| Ericsson | Yes. Other information like uncertainty, time type, which is not part of the Time Synchronization Assistance Information, might be useful to have. |
| Qualcomm | Neutral. Use cases are quite slim but would not oppose this if it helped to move forward. However that does not seem to be the case. |
| Moderator Summary: - 7 companies responded. - 2 companies support including RTI periodicity in HANDOVER REQUEST, while 5 companies do not. - 1 company additionally supports including other information like uncertainty and time type. Proposed conclusion: - No consensus to include RTI periodicity (or any other additional assistance information) in the HANDOVER REQUEST message. |  |

# 4	Discussion (Round 2)

Please provide your Round 2 views (5 questions) by 13:00 UTC Monday February 28th, so that TPs can be revised/finalized.

The following agreements were captured in the Chair’s minutes after Round 1:

For RTT-based PDC, PD pre-compensation is performed by the gNB-CU.

Introduce a new UE-associated class 1 procedure (CU-initiated) and a new UE-associated class 2 procedure (DU-initiated) to support both RTT-based PDC and TA-based PDC.

For RTT-based PDC, gNB-DU reports gNB Rx-Tx time difference to gNB-CU.

For TA-based PDC, gNB-DU reports NR Timing Advance (meaning PD pre-compensation is performed by the gNB-CU) to gNB-CU.

In this 2nd round of discussion, it is proposed to focus on the following:

a)	Finalizing the F1AP TP for RTT-based PDC and TA-based PDC (draft to be provided by Nokia)

b)	Agreeing on a TS 38.470 BL CR capturing the new F1AP procedures (draft to be provided by Huawei)

c)	If E-CID positioning procedure is ongoing, whether it can be used for TA-based PDC [10]

d)	Possible impacts of SIB9 delivery of pre-compensated RTI [6].

## 4.1	TP for TS 38.473

A TP for TS 38.473 capturing the PDC-related agreements is provided in the CB folder, merging inputs from several companies as follows:

-	The F1AP TP in [2] was used as baseline since it included the new class 1 and class 2 procedures supporting both TA-based PDC and RTT-based PDC.

-	The following deltas were then merged from [8] and [10] (highlighted in yellow and tentatively marked as FFS):

1)	add RAN UE PDC Measurement ID IE in the PDC MEASUREMENT INITIATION REQUEST, PDC MEASUREMENT INITIATION RESPONSE, and PDC MEASUREMENT REPORT messages (as proposed in [10]).

2)	align the values of the PDC Measurement Periodicity IE with the values of the E-CID Measurement Periodicity IE (as proposed in [8] & [10])

3)	encode the NR Timing Advance IE as INTEGER type in units of [64*] ns (as proposed in Round 1 Question #3). The range is 0ms to 2ms (roughly the same range as NTA)

4)	add Time Stamp IE in the PDC Measured Results Item IE (as proposed in [8]).

Question 1: Are the above changes #1-4 agreeable (YES/NO)?

| Company | Comments |
| --- | --- |
| Nokia | We are OK with changes #1-3. However, we are not clear why #4 (Time Stamp IE) is needed by the CU. |
| Huawei | 1), this is introduced for multiple measurement configuration for a single UE. Not see much need now. But we are fine to have it for future reasons; 2), ok, and we can refer to R3-221188 with the new agreed value? 3), we may slight prefer to have finer granularity. For 15khz, [64*] ns is about 32 ns, we may suggest to use [8*] if with consensus. Another thing is that the IE name can be updated to e.g., “NR PDC Timing Advance” since the “Value Timing Advance NR” is used in R3-221627. 4) this is needed, since the Time Stamp IE is used to ensure that the Rx-Tx time difference is measured in the same slot. |
| Samsung | 1) The usage of multiple measurement configuration for PDC is unclear. 2) Fine 3) Fine 4) We don’t see the usage yet. We’d rather wait for RRC signalling design in RAN2. If the Time Stamp info is included in UE report, the Time Stamp IE in the PDC Measured Results Item IE might be useful as Huawei comments. |
| ZTE | - Ok - The values of the PDC Measurement Periodicity IE depends on the UE Holdover time performance requirements for 5G System. In ts 22.261, the following performance requirements are provided: Table 7.8-1: Timing resiliency performance requirements for 5G System Use case; Holdover time (note 3); Sync target; Sync accuracy; Service area; Mobility; Remarks. Power grid (time synchronization device); >5 s; UTC (note 1); <250 ns to1000 ns (note2); < 20 km2; low; When 5G sync modem is integrated into PTP grandmaster solution (with 24h holdover capability at sub-stations). So we think the maximal value of 5s seems enough, but the minimal value should be smaller, e.g. 60ms. Thus, the value range can be ENUMERATED (60ms, 120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, …) (FFS) - Ok - Time Stamp IE suggested to be Reference SFN IE with value range of INTEGER (0..1023) And based on the TS 38.133, the value range of gNB Rx-Tx Time Difference IE can be INTEGER (0..1970049, …) with unit of Tc. |
| Nokia2 | The motivation behind the proposed encoding of gNB Rx-Tx Time Difference from ZTE above is unclear (assumes resolution of Tc, i.e. k=0). Why not reuse the existing gNB Rx-Tx Time Difference IE? |
| CATT | - ok, but same concern as SS and HW. It may not be useful now - ok - ok - no idea |
| Ericsson | Fine with 1 to 3. Do not see need for 4. |
| Moderator Summary: - 6 companies responded - For RAN UE PDC Measurement ID IE, 3 companies were fully OK with the proposal, while 3 companies expressed doubts about the usefulness in this release since the R17 use cases do not require multiple measurement configuration for a single UE. But of the 3 companies that expressed doubts, 2 indicated that they are nevertheless OK to have the IE for future proofness. - For the periodicity values, 5 companies indicated they were OK with the proposal (aligned with E-CID) while 1 company indicated that one smaller value (60ms) would be useful while values longer than 5120ms are not needed. From moderator’s perspective, further “refinement” of the periodicity values could be done during the correction phase if needed (e.g. to align the gNB Rx-Tx time difference periodicity values with RAN2’s decision on UE Rx-Tx time difference periodicity). - For NR Timing Advance, 5 companies indicated they were OK with the proposal while 1 company expressed preference for smaller granularity and renaming to “NR PDC Timing Advance” (to distinguish from an existing NR Timing Advance that has different encoding). From moderator’s perspective, renaming seems fine but further “refinement” of the granularity could be done during the correction phase if needed. - For Time Stamp, 1 company indicated they were OK with the proposal while 5 companies did not see the need. Proposed conclusion: - Add RAN UE PDC Measurement ID IE in the PDC MEASUREMENT INITIATION REQUEST, PDC MEASUREMENT INITIATION RESPONSE, and PDC MEASUREMENT REPORT messages. - Align the values of the PDC Measurement Periodicity IE with the values of the E-CID Measurement Periodicity IE. - Encode the NR PDC Timing Advance IE as INTEGER type in units of [64*] ns. The range is 0ms to 2ms. - No need for a Time Stamp IE in the PDC Measured Results Item IE. - Note: Further “refinement” of the periodicity values and the NR PDC Timing Advance granularity may be needed during the correction phase. |  |

Question 2: Please provide any other comments related to the draft F1AP TP. If needed, feel free to directly edit the draft TP.

| Company | Comments |
| --- | --- |
| Huawei | In the PDC MEASUREMENT INITIATION REQUEST message, the TA and RTT based Quantities are provided. We understand that these two quantities will not be used at the same time for the UE (based on RAN2 agreements). Then we can add the semantic descriptions that in this release, only a single quantity is allowed. Another alternative is to have a choice structure. Anyway, no strong view whether to have this clarification. |
| Samsung | Agree with Huawei’s comment. |
| ZTE | Agree with Huawei’s comment. And RAN2 already has agreement that TA and RTT based PDC will not be used simultaneously for the same UE. |
| Nokia | It is true that TA and RTT based PDC are not to be used simultaneously, and this is presumably captured somewhere by RAN2. However, we don’t see a need to include such restriction in Stage 3 – the CU will obviously not request something that it does not need, and it seems overspecification to state in semantics description what CU can’t do. Or stated another way – the place to capture the RAN2 agreement (i.e. something CU shouldn’t need to do) is not in a semantics description of F1AP. |
| CATT | Agree with Huawei. We may specify it as choice structure |
| Ericsson | Small editorial comments on the F1AP TP, revised in the CB folder. |
| Nokia2 | If it is desired to capture somehow that TA and RTT based PDC will not be used simultaneously for the same UE, one option (following an example in NGAP that was used for maxnoofCAGSperCell) is to indicate in the description of maxnoMeasPDC that “maximum is 1 in this release” (which can then be increased in a future release if needed). |
| Moderator Summary: - 4 companies commented that TA-based and RTT-based PDC cannot be used at the same time for a particular UE and indicated a preference to capture this somehow (e.g. in semantics description or by using a CHOICE structure). Proposed conclusion: - Add “Maximum is 1 in this release” to the description of maxnoMeasPDC. |  |

## 4.2	CR for TS 38.470

With the addition of new procedures, an update of TS 38.470 seems needed (draft to be provided by Huawei).

Question 3: Please provide any comments related to the draft 38.470 CR. If needed, feel free to directly edit the draft CR.

| Company | Comments |
| --- | --- |
| Huawei | Ok. Comments are appreciated. |
| Samsung | We’re fine with the draft 38.470 CR. By the way, the Reference Time Information Reporting procedure (Rel.16) is not included in TS 38.470. Don’t we need to add a section for the procedure in TS 38.470? [Huawei2]: thanks Samsung for pointing about this. If with consensus, we can add the Reference Time Information Reporting function/procedure into the stage 2 CR as a whole package. |
| ZTE | ok |
| Ericsson | St2 is not always added. Prefer the St2 is looked after by TS 38.370 Rapporteur as the Rel 17 updates work. |
| Nokia | Note that in the F1AP TP, the new PDC procedures were added as 8.12.x, i.e. as subsections under “8.12 Reference Time Information Reporting procedures” rather than e.g. a new “8.x PDC Measurement Reporting procedures”. The F1AP TP and 38.370 CR need to be aligned. |
| Moderator Summary: - 5 companies responded. - All companies are OK with the TS 38.370 CR. However, there is misalignment between the Stage 2 CR and the F1AP TP regarding whether the new procedures are part of existing Reference Time Information Reporting function or part of a new PDC Measurement Reporting function. - Since RAN3 has agreed that the DU reports measurements (and not e.g. Pre-compensated RTI), then perhaps the F1AP TP can be aligned to the 38.370 TP (i.e. the new F1AP procedures included under a new 8.x section). Proposed conclusion: - In the F1AP TP, create a new “8.x PDC Measurement Reporting procedures” section and a new “9.2.x PDC Measurement Reporting messages” section. |  |

## 4.3	When E-CID with “NR Timing Advance” measurement has already started

In [10], it is observed that the NR Timing Advance measurement in E-CID is “exactly the same” as we would need in PDC when TA method is used. Therefore, the following is proposed in [10]:

Proposal 1: If the E-CID measurement with “NR Timing Advance” code point has been started by LMF in NG-RAN node, gNB-CU should make use of the existing E-CID measurement when computing TA for PDC.

Proposal 2: If multiple E-CID measurements with “NR Timing Advance” code point exist, it is gNB-CU to pick up the right measurement to use, e.g. with desired “Periodicity”.

Proposal 3: Agree to specify if the E-CID with NR Timing Advance exist, gNB-CU shall reuse the measurement result.

Question 4: Please provide your views on Proposals 1-3 above.

| Company | Comments |
| --- | --- |
| Nokia | It is unclear whether the NR Timing Advance measurements obtained via E-CID procedures would be suitable for PDC, e.g. whether such measurements would have sufficient resolution/granularity. And in any case, reuse of such measurements does not seem precluded (i.e. does not seem to require any specification changes - up to gNB implementation). |
| Huawei | Basically share the Nokia view. And for this positioning NR timing advance: - The timing accuracy can not be ensured, (about 520ns granularity) - The DU has to triger the UE to perform RACH to acquire the Rx-Tx value (as described in the cover page based on RAN1 agreements in R3-221627) So we think that the ECID results is not fully accurate, but this can be left to CU implementation (i.e. no spec impact). |
| Samsung | Basically we share the similar view as Nokia and Huawei. And we agree that this can be left to CU implementation without spec impact. |
| ZTE | The measurement periodicity may be different, we prefer both of them can be performed simultaneously, and the details can be left to CU implementation. |
| CATT | We agree with above. We may left to CU implementation |
| Ericsson | Proposed one statement under “Interaction with the PDC Measurement Report procedure:” “If E-CID MEASUREMENT has started, the result might be used by gNB-CU for TADV when applicable.” We think it is beneficial to allowing it. |
| Moderator Summary: - 6 companies responded. - 5 companies indicated that the proposals have no specification impact (i.e. can be left to CU implementation) while 1 company think it is beneficial to add a sentence in the procedural section. Proposed conclusion: - No consensus to introduce any specification text related to interaction with E-CID measurement procedure (can be left to implementation). |  |

## 4.4	Possible impacts of SIB9 delivery of pre-compensated RTI

In [16], it is observed that it is possible for the DU to perform re-encoding of SIB9, so DU could pre-compensate the propagation delay for reference time info in SIB9 for TA-based PDC. Therefore, the following is proposed in [6]:

Proposal 4: For TA based PDC, DU is responsible for PD pre-compensation of reference time in SIB9.

Question 5: Please provide your views on proposal 4 above. If you agree with Proposal 4, please indicate possible specification impacts.

| Company | Comments |
| --- | --- |
| Nokia | RAN3 previously agreed that the gNB-CU decides whether/how to perform PDC, and this decision is presumably made by the CU on a per UE basis. Therefore, the scenario where DU performs PD pre-compensation is unclear – how does the DU know whether to pre-compensate the RTI in SIB9, and what TA value should be used? Is this a scenario where all UEs have the same TA value? |
| Huawei | Our understanding is that RAN2 has no agreements to use broadcast RTI as the PDC. So this is not an issue, unless RAN2 has any new progress. |
| Samsung | Our understanding is also that the pre-compensated RTI is performed per UE basis and delivered with unicast message. |
| ZTE | Agree the comments above. The gNB PDC only applies to unicast scenario. |
| CATT | Agree the comments above. |
| NTTDOCOMO | We understand the open issue [Can SIB9 be used to carry pre-compensated RTI?] is still under RAN2 discussion. |
| Ericsson | Open for further discussion. But this issue should not impact the closing of the WID at this meeting, as we have the basic functionality completed. |
| Nokia2 | In our understanding, RAN2 has just agreed that “SIB9 is not used to carry pre-compensated RTI”. |
| Moderator Summary: - 7 companies provided their views. - Based on latest RAN2 decision, it seems that SIB9 delivery of pre-compensated RTI is not supported. Proposed conclusion: - No RAN3 action needed, since RAN2 has decided that SIB9 cannot be used to carry pre-compensated RTI. |  |

# 5	Conclusions, Recommendations

Conclusions/recommendations from Round 1 discussion:

1.	For RTT-based PDC, PD pre-compensation is performed by the gNB-CU.

2.	Introduce a new UE-associated class 1 procedure (CU-initiated) and a new UE-associated class 2 procedure (DU-initiated) to support both RTT-based PDC and TA-based PDC.

3.	For RTT-based PDC, gNB-DU reports gNB Rx-Tx time difference to gNB-CU.

4.	No need to signal any Uu Synchronization Error Budget over F1 in Release 17.

5.	No need for any additional time synchronisation assistance information over Xn in Release 17.

6.	For TA-based PDC, further discuss whether DU reports NR Timing Advance (meaning PD pre-compensation is performed by the gNB-CU) or compensated RTI (meaning PD pre-compensation is performed by the gNB-DU).

Conclusions/recommendations from Round 2 discussion:

1.	Add RAN UE PDC Measurement ID IE in the PDC MEASUREMENT INITIATION REQUEST, PDC MEASUREMENT INITIATION RESPONSE, and PDC MEASUREMENT REPORT messages.

2.	Align the values of the PDC Measurement Periodicity IE with the values of the E-CID Measurement Periodicity IE.

3.	Encode the NR PDC Timing Advance IE as INTEGER type in units of [64*] ns. The range is 0ms to 2ms.

4.	No need for a Time Stamp IE in the PDC Measured Results Item IE.

5.	Add “Maximum is 1 in this release” to the description of maxnoMeasPDC.

6.	In the F1AP TP, create a new “8.x PDC Measurement Reporting procedures” section and a new “9.2.x PDC Measurement Reporting messages” section

Note: Further “refinement” of the periodicity values and the NR PDC Timing Advance granularity may be needed during the correction phase if needed.

# References

R3-221666, LS on PDC for Time Synchronization (RAN2)

R3-221874, (TP for NR_IIOT_URLLC_enh BL CR for TS 38.473) Time synchronization: resolution of F1AP open issues (Nokia, Nokia Shanghai Bell)

R3-221875, Time synchronization: resolution of mobility open issues (Nokia, Nokia Shanghai Bell)

R3-221877, Discussion on Time Synchronization enhancements (ZTE)

R3-221878, (TP for Introduction of Enhanced IIoT support over F1) Time Synchronization enhancements (ZTE)

R3-221951, Propagation Delay Compensation for TSN time synchronization (NTT DOCOMO INC.)

R3-221967, (TP for eIIOT BLCR for TS 38.473 and TS 38.423) Supporting PDC enhancements: TA-based PDC and mobility (Huawei)

R3-221968, (TP for eIIOT BLCR for TS 38.473) Supporting PDC enhancements: RTT based PDC (Huawei)

R3-222037, Discussion on Further enhanced NR-IIoT: Enhancements for support of time synchronization (Ericsson)

R3-222038, Discussion on PDC TA based and E-CID measurement (Ericsson)

R3-222212, Discussion on Propagation Delay Compensation Enhancements (CATT)

R3-222213, TP for BLCR for 38.473 on Propagation Delay Compensation Enhancements (CATT)

R3-221112, Summary of offline: Propagation delay compensation enhancements (Nokia)