3GPP TSG-RAN WG3 #115-e	R3-222589

Online, 21st February - 3rd March 2022

Agenda Item:	22.1

Source:	Ericsson (moderator)

Title:	Summary of Offline Discussion on a potential overall compromise for NR MBS Rel-17

Document for:	Approval

# Introduction

This document outlines an overall compromise for NR MBS Rel-17.

A graphical representation may look as follows:

# For the Chairman’s Notes

Agree the following package (apologize the shorthand wording)

- MC Session Parameters (QOS & Area Info) included in Multicast Distribution Response but not in Activation Request

- LS to SA2 to acknowledge RAN3 support of associated PDU Sessions with deactivated UP connection, join information carried in UE Ctxt signalling & DL NAS Transfer

- no MC Session parameters anywhere in UE associated signalling, apart from joining information and, if included, associated QoS flow info.

- Data Forwarding between supporting nodes supported

- On NG transparent HO CN containers carry mapping and forwarding info

- On Xn active MC Session parameters carried outside PDU Session Ctxt IEs in HO REQ, HO REQ ACK carries forwarding info

- LS to SA2 on NG impact for shared NG-U termination at NG-RAN (“common CU-UP”)

- No SN STATUS additions, “current COUNT” info in HO information sufficient, details to be seen (see CATT below)

- Support of separate F1-U bearers for ptp-retransmission and ptp-only MRB configurations

- F1/E1 MC MBS Session resource control in MBS-associated procedures only

- stop discussions on optimizing HO with non-supporting RAN nodes

- source gNB provides end marker towards non-supporting in implementation specific way

- duplication removal @ HO from non-supporting, if any, by UE only

- agree on the principles presented in the following TPs and the LS to SA2/RAN2 as uploaded in the “CB # MBS Compromise” folder and continue working on that basis::

- NGAP TP: R3-222584 (Revision of R3-222059)

- 38.401 TP: R3-222585 (Revision of R3-222060)

- E1AP TP: R3-222586 (Revision of R3-222061)

- F1AP TP: R3-222587 (Revision of R3-222494)

- XnAP TP: R3-222588R (Revision of R3-222063)

- draft LS out  to SA2/RAN2: R3-222590 “[Draft]  LS on further outstanding issues in TS 23.247”

# Discussion

## Feedback

Please provide your feedback

| Company | Comment |
| --- | --- |
| CATT | We prefer adding two bullets: - The “current COUNT” info, as an optional IE, comprises of both the PDCP Count (mandatory) and the MBS QFI SN (optional). - [Wording FFS] The PDCP COUNT of the MRB should be synchronized by adding every per-QoS-flow N3mb Sequence Number, if available, of each QoS flow which is mapped to this MRB. |
| Ericsson | it would be appreciated if RAN3 moves to the point outlined above, we have offered a huge amount in our compromise proposal. |

# Conclusion, Recommendations [if needed]

If needed

# References