3GPP TSG-RAN WG3 Meeting #115-e	R3-222648

- 21 Feb - 3 Mar 2022

- Online

Agenda item:	10.2.4

Source: 	ZTE, China Telecom, China Unicom

Title: 	(TP for SON BL CR 38.413) Inter-system Load Balancing

Document for:	Approval

Introduction

This contribution provides a TP for BL CR 38.413 on the inter-system load balancing, i.e. removing some “FFS” and introducing the PRB usage and RRC Connections in the Inter-system Resource Status Report IE.

Text Proposal for 38.413

First Change

9.3.3.yy1	Inter-system Resource Status Request

This IE contains information on the requested Inter-system Load Reporting reporting.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Reporting System |  |  |  |  |
| > E-UTRAN |  |  |  |  |
| >>E-UTRAN Cell To Report List |  | 1 |  |  |
| >>>E-UTRAN Cell To Report Item |  | 1 .. < maxnoofReportedCells> |  |  |
| >>> Cell ID | M |  | E-UTRAN CGI 9.3.1.9 | Contains the E-UTRAN CGI IE. |
| > NG-RAN |  |  |  |  |
| >>NG-RAN Cell To Report List |  | 1 |  |  |
| >>>NG-RAN Cell To Report Item |  | 1 .. <maxnoofReportedCells > |  |  |
| >>>> Cell ID | M |  | NG-RAN CGI 9.3.1.73 | Contains the NG-RAN CGI IE. |
| Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object the receiving node is requested to report. First Bit =Number of Active UEs, Second Bit =RRC connections, Third Bit =PRB. Other bits shall be ignored by the receiving node. |
| CHOICE report type | M |  |  |  |
| >Event based reporting |  |  |  |  |
| >>Inter-system Resource Threshold Low | M |  | INTEGER (0..100) | Inter-system Resource Status reporting is enabled when Composite Available Capacity is above this threshold or is disabled when Composite Available Capacity is below this threshold. The reporting node sends a report when the cell Composite Available Capacity becomes greater than or equal to the threshold. |
| >>Inter-system Resource Threshold High | M |  | INTEGER (0..100) | Inter-system Resource Status reporting is enabled when Composite Available Capacity is below this threshold or is disabled when Composite Available Capacity is above this threshold. The reporting node sends a report when the cell Composite Available Capacity becomes smaller than or equal to the threshold. |
| >>Number Of Measurement Reporting Levels | O |  | ENUMERATED (2, 3, 4, 5, 10, ...) | The reporting node divides the cell load scale into the indicated number of reporting levels, evenly distributed on a linear scale between the Inter-system Resource Threshold Low and the Inter-system Resource Threshold High. The reporting node sends a report each time the cell load changes from one reporting level to another. |
| >Periodic Reporting |  |  |  |  |
| >>Reporting Periodicity | M |  | ENUMERATED(single, 1000ms, 2000ms, 5000ms,10000ms, …) | Periodicity that can be used for reporting. If the value is “single” there is only one report. |

| Range bound | Explanation |
| --- | --- |
| maxnoofReportedCells | Maximum no. of cells that can be reported. Value is 256. |

9.3.3.yy2	Inter-system Resource Status Report

This IE contains the Inter-system load report.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Reporting System |  |  |  |  |
| > E-UTRAN |  |  |  |  |
| >>E-UTRAN Cell To Report List |  | 1 |  |  |
| >>>E-UTRAN Cell To Report Item |  | 1 .. < maxnoofReportedCells > |  |  |
| >>>>Cell ID | M |  | E-UTRAN CGI 9.3.1.9 | Contains the E-UTRAN CGI IE. |
| >>>>Composite Available Capacity Group | M |  |  | Contains the Composite Available Capacity Group IE as defined in TS 36.423[40]. |
| >>>>Number of Active UEs | O |  |  | Contains the Number of Active UEs IE as defined in TS 36.314. |
| >>>>RRC Connections | O |  |  | Contains the RRC Connections IE as defined in TS 38.423[24]. |
| >>>>Radio Resource Status | O |  |  | Contains the Radio Resource Status IE as defined in TS 36.423[40]. |
| >NG-RAN |  |  |  |  |
| >>NG-RAN Cell To Report List |  | 1 |  |  |
| >>> NG-RAN Cell To Report Item |  | 1 .. < maxnoofReportedCells > |  |  |
| >>>>Cell ID | M |  | NG-RAN CGI 9.3.1.73 | Contains the NG-RAN CGI IE. |
| >>>>Composite Available Capacity Group | M |  |  | Contains the Composite Available Capacity Group IE as defined in TS 36.423[40]. |
| >>>>Number of Active UEs | O |  |  | Contains the Number of Active UEs IE as defined in TS 38.423[24]. |
| >>>>RRC Connections | O |  |  | Contains the RRC Connections IE as defined in TS 38.423[24]. |
| >>>>Radio Resource Status | O |  |  | Contains the DL/UL GBR PRB usage for MIMO IE, DL/UL non-GBR PRB usage for MIMO IE and DL/UL Total PRB usage for MIMO IE, as defined in TS 38.423[24]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofReportedCells | Maximum no. of cells that can be reported. Value is 256. |

#### 9.3.3.yy3	Inter-system Load Reporting Reply

This IE indicates for which cell the load reporting was successfully initiated.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Reporting System |  |  |  |  |
| > E-UTRAN |  |  |  |  |
| >>E-UTRAN Cell To Report List |  | 1 |  |  |
| >>>E-UTRAN Cell To Report Item |  | 1 .. < maxnoofReportedCells> |  |  |
| >>> Cell ID | M |  | E-UTRAN CGI 9.3.1.9 | Contains the E-UTRAN CGI IE. |
| > NG-RAN |  |  |  |  |
| >>NG-RAN Cell To Report List |  | 1 |  |  |
| >>>NG-RAN Cell To Report Item |  | 1 .. <maxnoofReportedCells > |  |  |
| >>>> Cell ID | M |  | NG-RAN CGI 9.3.1.73 | Contains the NG-RAN CGI IE. |

| Range bound | Explanation |
| --- | --- |
| maxnoofReportedCells | Maximum no. of cells that can be reported. Value is 256. |

Next Change

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

id-AdditionalDLUPTNLInformationForHOList,

id-AdditionalNGU-UP-TNLInformation,

id-AdditionalRedundantDL-NGU-UP-TNLInformation,

id-AdditionalRedundantDLQosFlowPerTNLInformation,

id-AdditionalRedundantNGU-UP-TNLInformation,

id-AdditionalRedundantUL-NGU-UP-TNLInformation,

id-AdditionalUL-NGU-UP-TNLInformation,

id-AlternativeQoSParaSetList,

id-BurstArrivalTimeDownlink,

id-Cause,

id-CNPacketDelayBudgetDL,

id-CNPacketDelayBudgetUL,

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-CommonNetworkInstance,

id-ConfiguredTACIndication,

id-CurrentQoSParaSetIndex,

id-DAPSRequestInfo,

id-DAPSResponseInfoList,

id-DataForwardingNotPossible,

id-DataForwardingResponseERABList,

id-DirectForwardingPathAvailability,

id-DL-NGU-UP-TNLInformation,

id-EndpointIPAddressAndPort,

id-ExtendedPacketDelayBudget,

id-ExtendedRATRestrictionInformation,

id-ExtendedSliceSupportList,

id-ExtendedTAISliceSupportList,

id-ExtendedUEIdentityIndexValue,

id-GlobalCable-ID,

id-GlobalRANNodeID,

id-GlobalTNGF-ID,

id-GlobalTWIF-ID,

id-GlobalW-AGF-ID,

id-GUAMIType,

id-LastEUTRAN-PLMNIdentity,

id-LocationReportingAdditionalInfo,

id-MaximumIntegrityProtectedDataRate-DL,

id-MDTConfiguration,

id-MicoAllPLMN,

id-NetworkInstance,

id-NID,

id-NPN-MobilityInformation,

id-NPN-PagingAssistanceInformation,

id-NPN-Support,

id-OldAssociatedQosFlowList-ULendmarkerexpected,

id-PagingAssisDataforCEcapabUE,

id-PagingeDRXInformation,

id-PDUSessionAggregateMaximumBitRate,

id-PduSessionExpectedUEActivityBehaviour,

id-PDUSessionResourceFailedToSetupListCxtFail,

id-PDUSessionResourceReleaseResponseTransfer,

id-PDUSessionType,

id-PSCellInformation,

id-QosFlowAddOrModifyRequestList,

id-QosFlowFailedToSetupList,

id-QosFlowFeedbackList,

id-QosFlowParametersList,

id-QosFlowSetupRequestList,

id-QosFlowToReleaseList,

id-QosMonitoringRequest,

id-QosMonitoringReportingFrequency,

id-RAT-Information,

id-RedundantCommonNetworkInstance,

id-RedundantDL-NGU-TNLInformationReused,

id-RedundantDL-NGU-UP-TNLInformation,

id-RedundantDLQosFlowPerTNLInformation,

id-RedundantPDUSessionInformation,

id-RedundantQosFlowIndicator,

id-RedundantUL-NGU-UP-TNLInformation,

id-SCTP-TLAs,

id-SecondaryRATUsageInformation,

id-SecurityIndication,

id-SecurityResult,

id-SgNB-UE-X2AP-ID,

id-S-NSSAI,

id-SONInformationReport,

id-TNLAssociationTransportLayerAddressNGRAN,

id-TargetRNC-ID,

id-TraceCollectionEntityURI,

id-TSCTrafficCharacteristics,

id-UEHistoryInformationFromTheUE,

id-UERadioCapabilityForPaging,

id-UERadioCapabilityForPagingOfNB-IoT,

id-UL-NGU-UP-TNLInformation,

id-UL-NGU-UP-TNLModifyList,

id-ULForwarding,

id-ULForwardingUP-TNLInformation,

id-UsedRSNInformation,

id-UserLocationInformationTNGF,

id-UserLocationInformationTWIF,

id-UserLocationInformationW-AGF,

id-LastVisitedPSCell-List,

id-IntersystemSONInformationRequest,

id-IntersystemSONInformationResponse,

id-IntersystemSONInformationReply,

id-IntersystemResourceStatusUpdate,

id-EUTRAN-Cell-To-Report-List,

id-NG-RAN-Cell-To-Report-List,

id-IntersystemResourceThresholdLow,

id-IntersystemResourceThresholdHigh,

id-NumberOfMeasurementReportingLevels,

id-ReportingPeriodicity,

id-EUTRAN-CompositeAvailableCapacity-Group,

id-EUTRAN-NumberOfActiveUEs,

id-NG-RAN-NumberOfActiveUEs,

id-EUTRAN-NoofRRCConnections,

id-NG-RAN-NoofRRCConnections,

id-EUTRAN-RadioResourceStatus,

id-NG-RAN-RadioResourceStatus,

id-EnergySavingIndication,

id-SuccessfulHandoverReport-List,

id-NG-RAN-LastVisitedPSCell,

id-EUTRAN-LastVisitedPSCell,

id-nGRAN-PSCell-CGI,

id-eUTRA-PSCell-CGI,

id-timeUEStayedInCell,

maxnoofAllowedAreas,

maxnoofAllowedCAGsperPLMN,

maxnoofAllowedS-NSSAIs,

maxnoofBluetoothName,

maxnoofBPLMNs,

maxnoofCAGSperCell,

maxnoofCandidateCells,

maxnoofCellIDforMDT,

maxnoofCellIDforWarning,

maxnoofCellinAoI,

maxnoofCellinEAI,

maxnoofCellsingNB,

maxnoofCellsinngeNB,

maxnoofCellinTAI,

maxnoofCellsinUEHistoryInfo,

maxnoofCellsUEMovingTrajectory,

maxnoofDRBs,

maxnoofEmergencyAreaID,

maxnoofEAIforRestart,

maxnoofEPLMNs,

maxnoofEPLMNsPlusOne,

maxnoofE-RABs,

maxnoofErrors,

maxnoofExtSliceItems,

maxnoofForbTACs,

maxnoofFreqforMDT,

maxnoofMDTPLMNs,

maxnoofMultiConnectivity,

maxnoofMultiConnectivityMinusOne,

maxnoofNeighPCIforMDT,

maxnoofNGConnectionsToReset,

maxNRARFCN,

maxnoofNRCellBands,

maxnoofPC5QoSFlows,

maxnoofPDUSessions,

maxnoofPLMNs,

maxnoofQosFlows,

maxnoofQosParaSets,

maxnoofRANNodeinAoI,

maxnoofRecommendedCells,

maxnoofRecommendedRANNodes,

maxnoofAoI,

maxnoofSensorName,

maxnoofServedGUAMIs,

maxnoofSliceItems,

maxnoofTACs,

maxnoofTAforMDT,

maxnoofTAIforInactive,

maxnoofTAIforPaging,

maxnoofTAIforRestart,

maxnoofTAIforWarning,

maxnoofTAIinAoI,

maxnoofTimePeriods,

maxnoofTNLAssociations,

maxnoofWLANName,

maxnoofXnExtTLAs,

maxnoofXnGTP-TLAs,

maxnoofXnTLAs,

maxnoofPSCellsPerPrimaryCellinUEHistoryInfo,

maxnoofCellsinNGRANNode,

maxnoofReportedCells,

maxnoofSuccessfulHOReports

Next Change

-- --------------------------------------------------------------------

-- Inter System Resource Status Report

-- --------------------------------------------------------------------

IntersystemResourceStatusReport ::= SEQUENCE {

resourceStatusreportingsystem		ResourceStatusreportingsystem,

iE-Extensions					ProtocolExtensionContainer { { IntersystemResourceStatusReport-ExtIEs} }

}

IntersystemResourceStatusReport-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ResourceStatusreportingsystem ::= CHOICE {

eUTRAN-ReportingStatus		EUTRAN-ReportingStatusIEs,

nG-RAN-ReportingStatus		NG-RAN-ReportingStatusIEs,

choice-Extensions			ProtocolIE-SingleContainer { { ResourceStatusreportingsystem-ExtIEs}}

}

ResourceStatusreportingsystem-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

EUTRAN-ReportingStatusIEs::= SEQUENCE {

eUTRAN-Cell-To-Report-List					EUTRAN-Cell-To-Report-List,

eUTRAN-CompositeAvailableCapacity-Group			EUTRAN-CompositeAvailableCapacity-Group,

eUTRAN-NumberOfActiveUEs					EUTRAN-NumberOfActiveUEs        OPTIONAL,

eUTRAN-NoofRRCConnections                                  NG-RAN-NoofRRCConnections       OPTIONAL,

eUTRAN-RadioResourceStatus                                 EUTRAN-RadioResourceStatus      OPTIONAL,

...

}

EUTRAN-CompositeAvailableCapacity-Group	::= SEQUENCE {

dL-CompositeAvailableCapacity					CompositeAvailableCapacity,

uL-CompositeAvailableCapacity					CompositeAvailableCapacity,

iE-Extensions										ProtocolExtensionContainer { { EUTRAN-CompositeAvailableCapacity-Group-ExtIEs} },

...

}

EUTRAN-CompositeAvailableCapacity-Group-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CompositeAvailableCapacity ::= SEQUENCE {

cellCapacityClassValue							CellCapacityClassValue				OPTIONAL,

capacityValue									CapacityValue,

iE-Extensions									ProtocolExtensionContainer { {CompositeAvailableCapacity-ExtIEs} } OPTIONAL,

...

}

CompositeAvailableCapacity-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CellCapacityClassValue ::= INTEGER (1..100, ...)

CapacityValue ::= INTEGER (0..100)

EUTRAN-NumberOfActiveUEs ::= INTEGER (0..16777215, ...)

EUTRAN-RadioResourceStatus ::= SEQUENCE {

dL-GBR-PRB-usage							DL-GBR-PRB-usage,

uL-GBR-PRB-usage							UL-GBR-PRB-usage,

dL-non-GBR-PRB-usage						DL-non-GBR-PRB-usage,

uL-non-GBR-PRB-usage						UL-non-GBR-PRB-usage,

dL-Total-PRB-usage							DL-Total-PRB-usage,

uL-Total-PRB-usage							UL-Total-PRB-usage,

iE-Extensions								ProtocolExtensionContainer { {EUTRAN-RadioResourceStatus-ExtIEs} } OPTIONAL,

...

}

EUTRAN-RadioResourceStatus-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-DL-scheduling-PDCCH-CCE-usage		CRITICALITY ignore	EXTENSION DL-scheduling-PDCCH-CCE-usage		PRESENCE optional}|

{ID id-UL-scheduling-PDCCH-CCE-usage		CRITICALITY ignore	EXTENSION UL-scheduling-PDCCH-CCE-usage		PRESENCE optional},

...

}

DL-GBR-PRB-usage::= INTEGER (0..100)

DL-non-GBR-PRB-usage::= INTEGER (0..100)

DL-Total-PRB-usage::= INTEGER (0..100)

DL-scheduling-PDCCH-CCE-usage::= INTEGER (0..100)

UL-GBR-PRB-usage::= INTEGER (0..100)

UL-non-GBR-PRB-usage::= INTEGER (0..100)

UL-Total-PRB-usage::= INTEGER (0..100)

UL-scheduling-PDCCH-CCE-usage::= INTEGER (0..100)

NG-RAN-ReportingStatusIEs ::= SEQUENCE {

nG-RAN-Cell-To-Report-List			              NG-RAN-Cell-To-Report-List,

eUTRAN-CompositeAvailableCapacity-Group			EUTRAN-CompositeAvailableCapacity-Group,

nG-RAN-NumberOfActiveUEs					NG-RAN-NumberOfActiveUEs        OPTIONAL,

nG-RAN-NoofRRCConnections					NG-RAN-NoofRRCConnections       OPTIONAL,

nG-RAN-RadioResourceStatus                                 NG-RAN-RadioResourceStatus      OPTIONAL,

...

}

NG-RAN-NumberOfActiveUEs ::= INTEGER(0..16777215, ...)

NG-RAN-NoofRRCConnections ::= INTEGER (1..65536,...)

NG-RAN-RadioResourceStatus	::= SEQUENCE {

dL-GBR-PRB-usage-for-MIMO	              DL-GBR-PRB-usage-for-MIMO,

uL-GBR-PRB-usage-for-MIMO	  		    UL-GBR-PRB-usage-for-MIMO,

dL-non-GBR-PRB-usage-for-MIMO   		    DL-non-GBR-PRB-usage-for-MIMO,

uL-non-GBR-PRB-usage-for-MIMO            UL-non-GBR-PRB-usage-for-MIMO,

dL-Total-PRB-usage-for-MIMO	    		    DL-Total-PRB-usage-for-MIMO,

uL-Total-PRB-usage-for-MIMO	    		    UL-Total-PRB-usage-for-MIMO,

iE-Extensions							ProtocolExtensionContainer { { NG-RAN-RadioResourceStatus-ExtIEs} }	OPTIONAL,

...

}

NG-RAN-RadioResourceStatus-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

DL-GBR-PRB-usage-for-MIMO::= INTEGER (0..100)

DL-non-GBR-PRB-usage-for-MIMO::= INTEGER (0..100)

DL-Total-PRB-usage-for-MIMO::= INTEGER (0..100)

UL-GBR-PRB-usage-for-MIMO::= INTEGER (0..100)

UL-non-GBR-PRB-usage-for-MIMO::= INTEGER (0..100)

UL-Total-PRB-usage-for-MIMO::= INTEGER (0..100)

End of Changes

References

- R3-221545 BLCR to 38.413_Addition of SON features enhancement  Ericsson