3GPP TSG-RAN WG3 meeting #115-e	R3-222674

E-meeting, 21st Feb – 3rd Mar, 2022

Title:	(TP for NR_IAB_enh BL CR for TS 38.401) Inter-donor re-routing for IAB during topology update

Source: 	Huawei, Samsung

Agenda Item:	13.3.2

Document for:	Discussion

Introduction

This TP is produced according to the outcome of the following CB:

CB: # 1306_IAB_Multi-hop

- Is there a need for any additional specification details to close the topic?

- Should we capture which aspects of this topic are left to implementation or based on configuration?

- Is there a need to indicate to the donor-DU whether an IP address/prefix subject to tunnelling/exempt from IP address filtering is a source or a destination address/prefix? Is there a need to signal the mapping between IP address and tunnel identifier?

- Is there a need to introduce a new F1 procedure or can an existing one be reused?

(Nok - moderator)

Summary of offline disc R3-222465

- Annex: Text Proposal for TS 38.401

START OF CHANGE

8.x1.y	IAB Inter-donor-DU routing

When an IAB-donor-DU is configured with the information to support inter-donor-DU routing, the IAB-donor-DU may identify a re-routed UL IP packet based on the source IP address field of the UL packet, and forward the re-routed UL IP packet to the peer IAB-donor-DU via a tunnel. In the intra-donor-CU scenario for UL inter-donor-DU rerouting, the IAB-donor-DU and its peer IAB-donor-DU are controlled by the same IAB-donor-CU. In the inter-donor-CU case, the IAB-donor-DU and its peer IAB-donor-DU are controlled by different IAB-donor-CUs. The inter-donor-DU tunnel is a GTP-U tunnel. IAB-donor-CU configures the IAB-donor-DU with a list of TNL addresses and/or prefixes that are subject to inter-donor-DU tunneling and exempt from TNL address filtering.

NOTE:	Other tunnel types may be used for the inter-donor-DU tunnel, by implementation.