3GPP TSG-RAN WG3 Meeting #115-e	R3- 222729

E-meeting, 21 Feb – 3 Mar 2022

Title: 	(TP for SON BLCR for 38.300) Coverage and Capacity Optimization

Source: 	Huawei

Agenda item:	10.2.2

Document Type:	other

1. Introduction

This is a TP for 38.300.

Annex  – TP for 38.300

Start of the change

15.5.X	Support for CCO

15.5.X.1	General

The objective of NR Capacity and Coverage Optimization (CCO) function is to detect and mitigate coverage and cell edge interference issues.

15.5.X.2	OAM requirements

Each NG-RAN node may be configured with alternative coverage configurations by OAM. The alternative coverage configurations contain relevant radio parameters and may also include a range for how each parameter is allowed to be adjusted.

15.5.X.3	Dynamic coverage configuration changes

An NG-RAN node may autonomously adjust within and switch between coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes using the NG-RAN NODE CONFIGURATION UPDATE message with the list of cells and SSBs with modified coverage included. The list contains the CGI of each modified cell with its coverage state indicator and optionally the SSB index of each modified SSB with its coverage state indicator.

The indicator may be used at the receiving NG-RAN node to adjust the functions of the Mobility Robustness Optimisation, e.g. by using the indicator to retrieve a previously stored Mobility Robustness Optimisation state. The indicator may also be used at the receiving NG-RAN node to adopt CCO configurations matching with neighbouring cells configurations.

If the list includes indication about planned reconfiguration and possibly a list of replacing cells, the receiving NG-RAN node may use this to avoid connection or re-establishment failures during the reconfiguration. Also, if the sending NG-RAN node adds cells in inactive state, the receiving NG-RAN node may use this information to avoid connection or re-establishment failures. The receiving NG-RAN node may also use the notification to reduce the impact on mobility. The receiving NG-RAN node should avoid triggering handovers towards cell(s) that are indicated to be inactive.