3GPP TSG-RAN WG3 Meeting #115	R3-222761

E-meeting, 17 Feb – 03 Mar 2022

Title: 	(TP for SON BLCR for 38.300) MRO for SN Change Failure

Source: 	Huawei

Agenda item:	10.2.1.6

Document Type:	other

# 1. Introduction

This is a TP for 38.300

# Annex – TP on TS 38.300

Start of the first change

#### 15.5.2.x	PSCell change failure

##### 15.5.2.x.1	General

For analysis of PSCell change failures, the UE makes the SCG Failure Information available to the MN. If the MN can perform an initial analysis, it transfers the SCG Failure Information together with the analysis results to the relevant SN which is responsible for the PSCell change failures (see the section 13.x in TS 37.340 [21]). Otherwise, the MN transfers the SCG Failure Information to the last serving SN. If needed, the MN transfer the SCG Failure Information to the source SN (see the section 13.x in TS 37.340 [21]).

End of the last change