3GPP TSG-RAN WG3 #115-e																	      R3-222773

21th Feb – 3rd Mar 2022

Online

Title: 	(TP for RA-SDT BLCR to TS 38.300) Left issues for RA-SDT without anchor relocation case

Source: 	 ZTE, Ericsson, Nokia, Nokia Shanghai Bell, Intel Corporation, Samsung

Agenda item: 	24.2

Document for:	Discussion and Decision

- Introduction

Based on the progress, we provide the following TP.

- Text Proposal to TS38.300

Change Begins

# XX	Small Data Transmission

<Omit content from RAN2>

## XX.1	Support of RACH based SDT

For RACH based SDT, the UE in RRC_INACTIVE initiates RACH and requests RRC resume together with UL SDT data. If the UE accesses a gNB other than the last serving gNB, the UL SDT data is buffered at the receiving gNB, and then the receiving gNB triggers the XnAP Retrieve UE Context procedure. The receiving gNB indicates SDT to the last serving gNB and the last serving gNB decides whether to relocate the UE context or not. Other SDT assistance information (i.e., single packet, multiple packets) may also be provided by the receiving gNB to help the decision.

If the last serving gNB decides not to relocate the UE context, it transfers at least a partial UE context containing SDT RLC context information necessary for the receiving gNB to handle SDT. Then, UL/DL tunnels are established for DRBs configured for SDT between the receiving gNB and the last serving gNB and PDCP PDU of UL/DL data are transferred over the tunnels, until the last serving gNB decides to terminate the SDT session and move the UE back to RRC_INACTIVE by sending RRCRelease message. During the SDT session, the receiving gNB may also request to terminate the SDT session to the last serving gNB.

If the last serving gNB decides not to relocate the UE context, in case SDT is used for signalling, SRB PDCP PDUs  is transferred between the receiving gNB and the last serving gNB via the XnAP RRC Transfer procedure.

When the receiving gNB receives DL non-SDT data or DL non-SDT signalling from the core network and the UE context has been relocated to the receiving gNB, the receiving gNB may send the UE to RRC_CONNECTED state by directly sending the RRCResume message.

When the last serving gNB receives DL non-SDT data or DL non-SDT signalling from the core network and it has decided to not relocate the UE context, it may send the UE back to RRC_INACTIVE by sending the RRCRelease message.

### XX.2	RA-based SDT with UE context relocation

The overall procedure for RACH based small data transmission with UE context relocation is illustrated in the figure XX.2-1.

Figure XX.2-1. RA-based SDT with UE context relocation

1.  The UE sends an RRCResumeRequest as well as UL SDT data and/or UL SDT signalling to the receiving gNB.

2.  	The receiving gNB identifies the last serving gNB using the I-RNTI and retrieves the UE context by means of Xn-AP Retrieve UE Context procedure. The receiving gNB indicates that the UE request is for an SDT transaction and may also provide SDT assistance information (i.e., single packet, multiple packets).

3.  The last serving gNB decides to relocate UE context for SDT and responds with the RETRIEVE UE CONTEXT RESPONSE message. The UL SDT data, if any, is delivered to the UPF.

4-6. The receiving gNB decides to keep UE in RRC Inactive state for SDT.  If loss of DL user data buffered in the last serving gNB shall be prevented, the receiving gNB provides forwarding addresses. The receiving gNB also initiates NG-AP Path Switch procedure to establish a NG UE associated signalling connection to the serving AMF. After path switch, the UL NAS PDU, if any, is delivered to the AMF.

7. After the SDT transmission is completed, the receiving gNB generates and sends RRCRelease message including the Suspend indication to the UE to send the UE back to RRC_INACTIVE.

NOTE: In case DL non-SDT data or DL non-SDT signalling arrives, the receiving gNB may decide to directly send the UE to RRC_CONNECTED state by sending RRCResume message.

8. The receiving gNB indicates the last serving gNB to remove the UE context by sending the XnAP UE CONTEXT RELEASE message. The XnAP UE CONTEXT RELEASE message can be sent after step 6.

### XX.3	RA-based SDT without UE context relocation

The overall procedure for RACH based small data transmission without UE context relocation is illustrated in the figure XX.3-1.

Figure XX.3-1. RA-based SDT without UE context relocation

1/2.  	The steps 1/2 are as defined in steps 1/2 in Figure XX.2-1.

3.  	The last serving gNB decides not to relocate the UE context for SDT.

4.  	The last serving gNB transfers a partial UE context including the SDT related RLC context.

5. 	The receiving gNB acknowledges receiving the partial UE context and provides associated DL TNL address, if needed. After the Partial UE Context Retrieval procedure, the UE context is kept at the last serving gNB and the SDT related RLC context is established at the receiving gNB, then the UL SDT data is delivered to the UPF, the UL NAS PDU is delivered to AMF, if any.

NOTE 1: The DL signalling from the last serving gNB, if any, is forwarded to the receiving gNB via the RRC TRANSFER message, for which the receiving gNB delivers to the UE.

6. 	After SDT transmission is completed, the last serving gNB responds to the receiving gNB with the RETRIEVE UE CONTEXT FAILURE message including an encapsulated RRCRelease message. The RRCRelease message includes suspend configuration.

NOTE 2: The receiving gNB may send the RETRIEVE UE CONTEXT CONFIRM message to request the termination of SDT session before step 6.

NOTE 3: In case DL non-SDT data or DL non-SDT signalling arrives, the last serving gNB moves the UE back to RRC_INACTIVE by sending RRCRelease message.

7.	The receiving gNB sends the RRCRelease message to the UE.

8. 	The UE moves to RRC_INACTIVE mode.

Change Ends