3GPP TSG-RAN WG3 Meeting #115-e	R3-222786

- 21 Feb - 3 Mar 2022

- Online

Agenda item:	30.1

Source: 	ZTE, Nokia, Nokia Shanghai Bell

Title: 	(TP for IoT NTN BL CR 36.300) Stage 2 Clarification on IoT NTN

Document for:	Approval

Introduction

This contribution provides the TP for BL CR 36.300 on IoT NTN, i.e. clarification on mapped cell ID construction, description of the multiple TAC reporting and correction on MME (re-)selection by eNB.

Text Proposal for BL CR 36.300

23.x.6 Signalling

The Cell Identity, as defined in TS 36.413 [25] and TS 36.423 [42], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported.

-	The Cell Identity indicated by the eNB to the Core Network as part of the User Location Information, or as E-UTRAN CGI in the related S1AP messages;

-	The Cell Identity used for Paging Optimization in S1 interface;

-	The Cell Identity used for PWS.

For a BL UE or a UE in enhanced coverage, the Cell Identity included within the target identification of the handover messages allows identifying the correct target cell.

The mapping between Cell Identities and geographical areas is configured in the RAN and Core Network.

For a BL UE or a UE in enhanced coverage, the eNB is responsible for constructing the Mapped Cell ID based on the UE location info received from the UE. The mapping may be pre-configured (e.g., up to operator’s policy) or up to implementation.

NOTE: 	As described in TS 23.401 [17], the User Location Information may enable the MME to determine whether the UE is allowed to operate at its present location. Pre-configuration of special mapped cell identifiers may be used to indicate areas outside the serving PLMN’s country.

The eNB reports the broadcasted TAC(s) of the selected PLMN to the MME. In case the eNB knows the UE’s location information, the eNB may determine the TAI the UE is currently located in and provide that TAI to the MME.

23.x.7	MME(Re-)Selection by eNB

The eNB implements the NAS Node Selection Function specified in TS 36.410 [x].

For a RRC_CONNECTED UE, when the eNB is configured to ensure that the BL UE or the UE in enhanced coverage is using an MME that serves the country in which the UE is located. If the eNB detects that a BL UE or a UE in enhanced coverage is in a different country to that served by the serving MME, it should perform an S1 handover to change to an appropriate MME or initiate an UE Context Release Request procedure towards the serving MME (in which case the MME may decide to de-register the UE)..