3GPP TSG-RAN WG3 #115-e	R3-222837

21 Feb - 3 Mar 2022

Online

Title: 	(TP for SON BL CR for TS 38.413) Introduce UE History Information in MR-DC

Source: 	ZTE, Lenovo, Motorola Mobility, China Unicom

Agenda item:	10.2.1.4

Document for:	Approval

2	TP to TS 38.413

*** Start change ***

9.3.1.97	Last Visited NG-RAN Cell Information

This IE contains information about a cell. In case of NR cell, this IE contains information about a set of NR cells with the same NR ARFCN for reference point A, and the Global Cell ID IE identifies one of the NR cells in the set. The information is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Global Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  |  |  |
| Cell Type | M |  | 9.3.1.98 |  |  |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in seconds. If the duration is more than 4095s, this IE is set to 4095. |  |  |
| Time UE Stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. |  |  |
| HO Cause Value | O |  | Cause 9.3.1.2 | The cause for the handover. |  |  |
| Last Visited PSCell List |  | 0..<maxnoofPSCellsPerPrimaryCellinUEHistoryInfo> |  | List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list. | YES | ignore |
| >Last Visited PSCell Information | M |  | 9.2.3.X | The PSCell related information. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPSCellsPerPrimaryCellinUEHistoryInfo | Maximum number of last visited PSCell information records that can be reported in the IE. Value is 8. |

*** Next change ***

#### 9.2.3.X	Last Visited PSCell Information

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PSCell ID | O |  | NG-RAN CGI 9.3.1.73 | This IE is present when the SCG resources are configured for the UE. |
| Time Stay | M |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. Or the duration of the time when no SCG resources are configured for the UE. |

*** Start of ASN.1 change ***

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

-------- skip unchanged part ----------

id-UserLocationInformationTNGF,

id-UserLocationInformationTWIF,

id-UserLocationInformationW-AGF,

id-LastVisitedPSCellList,

id-IntersystemSONInformationRequest,

id-IntersystemSONInformationResponse,

id-IntersystemSONInformationReply,

id-IntersystemResourceStatusUpdate,

id-EUTRAN-Cell-To-Report-List,

id-NG-RAN-Cell-To-Report-List,

id-IntersystemResourceThresholdLow,

id-IntersystemResourceThresholdHigh,

id-NumberOfMeasurementReportingLevels,

id-ReportingPeriodicity,

id-EUTRAN-CompositeAvailableCapacity-Group,

id-EUTRAN-NumberOfActiveUEs,

id-NG-RAN-NumberOfActiveUEs,

id-NG-RAN-NoofRRCConnections,

id-EnergySavingIndication,

id-SuccessfulHandoverReport-List,

id-PSCellID,

id-TimeStay,

maxnoofAllowedAreas,

maxnoofAllowedCAGsperPLMN,

-------- skip unchanged part ----------

LastVisitedEUTRANCellInformation ::= OCTET STRING

LastVisitedGERANCellInformation ::= OCTET STRING

LastVisitedNGRANCellInformation::= SEQUENCE {

globalCellID								NGRAN-CGI,

cellType									CellType,

timeUEStayedInCell							TimeUEStayedInCell,

timeUEStayedInCellEnhancedGranularity		TimeUEStayedInCellEnhancedGranularity 			OPTIONAL,

hOCauseValue								Cause		 									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LastVisitedNGRANCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedNGRANCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-LastVisitedPSCell-List	CRITICALITY ignore	EXTENSION LastVisitedPSCell-List	PRESENCE optional },

...

}

LastVisitedPSCellList ::= SEQUENCE (SIZE(1.. maxnoofPSCellsPerPrimaryCellinUEHistoryInfo)) OF LastVisitedPSCell-Information

LastVisitedPSCellInformation ::= SEQUENCE {

pSCellID							NGRAN-CGI	 OPTIONAL		,

timeStay							INTEGER (0..40950),		,

iE-Extensions		ProtocolExtensionContainer { {LastVisitedPSCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedPSCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

LastVisitedUTRANCellInformation ::= OCTET STRING

LineType ::= ENUMERATED {

dsl,

pon,

...

}

-------- skip unchanged part ----------

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-------- skip unchanged part ----------

maxnoofXnGTP-TLAs					INTEGER ::= 16

maxnoofXnTLAs						INTEGER ::= 2

maxnoofCandidateCells				INTEGER ::= 32

maxNRARFCN							INTEGER ::= 327916

maxnoofPSCellsPerPrimaryCellinUEHistoryInfo	INTEGER ::= 8

maxnoofCellsinNGRANNode				INTEGER ::= 16384

maxnoofReportedCells				INTEGER ::= 256

maxnoofSuccessfulHOReports			INTEGER ::= 64

-- This value is FFS

-- **************************************************************

--

-- IEs

--

-- **************************************************************

-------- skip unchanged part ----------

id-ExtendedUEIdentityIndexValue							ProtocolIE-ID ::= 280

id-PduSessionExpectedUEActivityBehaviour					ProtocolIE-ID ::= 281

id-MicoAllPLMN											ProtocolIE-ID ::= 282

id-QosFlowFailedToSetupList								ProtocolIE-ID ::= 283

id-LastVisitedPSCellList								ProtocolIE-ID ::= xx1

id-IntersystemSONInformationRequest						ProtocolIE-ID ::= xx2

id-IntersystemSONInformationReply						ProtocolIE-ID ::= xx3

id-IntersystemSONInformationResponse					ProtocolIE-ID ::= xx4

id-EUTRAN-Cell-To-Report-List							ProtocolIE-ID ::= xx5

id-NG-RAN-Cell-To-Report-List							ProtocolIE-ID ::= xx6

id-IntersystemResourceThresholdLow						ProtocolIE-ID ::= xx7

id-IntersystemResourceThresholdHigh						ProtocolIE-ID ::= xx8

id-NumberOfMeasurementReportingLevels					ProtocolIE-ID ::= xx9

id-ReportingPeriodicity									ProtocolIE-ID ::= x10

id-EUTRAN-CompositeAvailableCapacity-Group				ProtocolIE-ID ::= x11

id-EUTRAN-NumberOfActiveUEs								ProtocolIE-ID ::= x12

id-NG-RAN-NumberOfActiveUEs								ProtocolIE-ID ::= x13

id-NG-RAN-NoofRRCConnections							ProtocolIE-ID ::= x14

id-EnergySavingIndication								ProtocolIE-ID ::= x15

id-IntersystemResourceStatusUpdate						ProtocolIE-ID ::= x16

id-SuccessfulHandoverReport-List						ProtocolIE-ID ::= x17

id-PSCellID 											ProtocolIE-ID ::= x18

id-TimeStay 											ProtocolIE-ID ::= x19

END

*** End of change ***