3GPP TSG-RAN WG3 #115-e	R3-222878

E-meeting, 21st February – 3rd March 2022

Source:	CATT

Title:	(TP for TS36.423 on SON) Support of PSCell MLB

Agenda Item:	10.2.1.5

Document for:	other

- Introduction

This is a correction on the errors introduces by R3-221359.

1) Remove the editor’s note in 8.3.6.1.

2) Correct the “NR Neighbour Cell” to “NR Cells Possibly Aggregated”.

3) Update ASN.1.

4) Restrict the upper bound of the length of NR cell Possibly Aggregated to 16.

- TP for TS 36.423

////////////////////////////////////////////////////////////////////////skip unrelated text////////////////////////////////////////////////////////////////////////

### 8.3.6	Resource Status Reporting Initiation

#### 8.3.6.1	General

This procedure is used by an eNB to request the reporting of load measurements to another eNB.

The procedure uses non UE-associated signalling.

#### 8.3.6.2	Successful Operation

Figure 8.3.6.2-1: Resource Status Reporting Initiation, successful operation

The procedure is initiated with a RESOURCE STATUS REQUEST message sent from eNB1 to eNB2. Upon receipt, eNB2:

-	shall initiate the requested measurement according to the parameters given in the request in case the Registration Request IE set to "start"; or

-	shall stop all cells measurements and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	if supported, stop cell measurements and terminate the reporting for cells indicated in the Cell To Report IE list, in case the Registration Request IE is set to "partial stop"; or

-	if supported, add cells indicated in the Cell To Report IE list to the measurements initiated before for the given measurement IDs, in case the Registration Request IE is set to "add".

If the eNB2 received a RESOURCE STATUS REQUEST message, which includes the Registration Request IE set to "stop", the Cell To Report IE list shall be ignored.

If the Registration Request IE is set to "start" then the Report Characteristics IE shall be included in RESOURCE STATUS REQUEST message. The eNB2 shall ignore the Report Characteristics IE, if the Registration Request IE is not set to "start".

The Report Characteristics IE indicates the type of objects eNB2 shall perform measurements on. For each cell, the eNB2 shall include in the RESOURCE STATUS UPDATE message:

-	the Radio Resource Status IE, if the first bit, "PRB Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the S1 TNL Load Indicator IE, if the second bit, "TNL Load Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Hardware Load Indicator IE, if the third bit, "HW Load Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Composite Available Capacity Group IE, if the fourth bit, "Composite Available Capacity Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If Cell Capacity Class Value IE is included within the Composite Available Capacity Group IE, this IE is used to assign weights to the available capacity indicated in the Capacity Value IE;

-	the ABS Status IE, if the fifth bit, "ABS Status Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1 and eNB1 had indicated the ABS pattern to eNB2;

-	the RSRP Measurement Report List IE, if the sixth bit, "RSRP Measurement Report Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the CSI Report IE, if the seventh bit, "CSI Report Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Radio Resource Status IE within the NR Neithbour Cell Measurement Result IE, if the eighth bit, "Neighbour Cell CAC Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1.

NOTE:	In order to avoid duplication, the eNB2 may include only one copy of Measurement Result for NR Cells Possibly Aggregated Item per RESOURCE STATUS UPDATE messages per NR cell, even if this NR cell possibly aggregated to multiple E-UTRA cells served by eNB2. The eNB2 should only include Measurement Result for NR Cells Possibly Aggregated Item for NR cells that may be aggregated with at least one cell served by eNB1.

If the Reporting Periodicity IE is included in the RESOURCE STATUS REQUEST message, eNB2 shall use its value as the time interval between two subsequent RESOURCE STATUS UPDATE messages that include the Radio Resource Status IE, S1 TNL Load Indicator IE, Hardware Load Indicator IE, Composite Available Capacity Group IE, or ABS Status IE.

If the Reporting Periodicity of RSRP Measurement Report IE is included in the RESOURCE STATUS REQUEST message, eNB2 shall use its value as the minimum time interval between two subsequent RESOURCE STATUS UPDATE messages that include the RSRP Measurement Report List IE.

If the Reporting Periodicity of CSI Report IE is included in the RESOURCE STATUS REQUEST message, eNB2 shall use its value as the minimum time interval between two subsequent RESOURCE STATUS UPDATE messages that include the CSI Report IE.

If eNB2 is capable to provide all requested resource status information, it shall initiate the measurement as requested by eNB1, and respond with the RESOURCE STATUS RESPONSE message.

If eNB2 is capable to provide some but not all of the requested resource status information and the Partial Success Indicator IE is present in the RESOURCE STATUS REQUEST message, it shall initiate the measurement for the admitted measurement objects and include the Measurement Initiation Result IE in the RESOURCE STATUS RESPONSE message.

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

#### 9.1.2.11	RESOURCE STATUS REQUEST

This message is sent by an eNB1 to neighbouring eNB2 to initiate the requested measurement according to the parameters given in the message.

Direction: eNB1  eNB2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| eNB1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by eNB1 | YES | reject |
| eNB2 Measurement ID | C-ifRegistrationRequestStoporPartialStoporAdd |  | INTEGER (1..4095,...) | Allocated by eNB2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …, partial stop, add) | Type of request for which the resource status is required. | YES | reject |
| Report Characteristics | O |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object the eNB2 is requested to report. First Bit = PRB Periodic, Second Bit = TNL load Ind Periodic, Third Bit = HW Load Ind Periodic, Fourth Bit = Composite Available Capacity Periodic, this bit should be set to 1 if at least one of the First, Second or Third bits is set to 1, Fifth Bit = ABS Status Periodic, Sixth Bit = RSRP Measurement Report Periodic, Seventh Bit = CSI Report Periodic, Eighth Bit =CAC for Possibly Aggregated Cells Periodic. Other bits shall be ignored by the eNB2. | YES | reject |
| Cell To Report |  | 1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxCellineNB> |  |  | EACH | ignore |
| >>Cell ID | M |  | ECGI 9.2.14 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(1000ms, 2000ms, 5000ms,10000ms, …) | Periodicity that can be used for reporting of PRB Periodic, TNL Load Ind Periodic, HW Load Ind Periodic, Composite Available Capacity Periodic or ABS Status Periodic. | YES | ignore |
| Partial Success Indicator | O |  | ENUMERATED(partial success allowed, ...) | Included if partial success is allowed | YES | ignore |
| Reporting Periodicity of RSRP Measurement Report | O |  | ENUMERATED(120ms, 240ms, 480ms, 640ms, ...) | Periodicity that can be used for the reporting of RSRP Measurement Report Periodic. | YES | ignore |
| Reporting Periodicity of CSI Report | O |  | ENUMERATED(5ms, 10ms, 20ms, 40ms, 80ms, ...) | Periodicity that can be used for the reporting of CSI Report Periodic. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporPartialStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop", "partial stop" or "add". |

#### 9.1.2.12	RESOURCE STATUS RESPONSE

This message is sent by the eNB2 to indicate that the requested measurement, for all or for a subset of the measurement objects included in the measurement is successfully initiated.

Direction: eNB2  eNB1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| eNB1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by eNB1 | YES | reject |
| eNB2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by eNB2 | YES | reject |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| Measurement Initiation Result |  | 0..1 |  | List of all cells in which measurement objects were requested, included when indicating partial success | YES | ignore |
| >Measurement Initiation Result Item |  | 1 .. <maxCellineNB> |  |  | EACH | ignore |
| >>Cell ID | M |  | ECGI 9.2.14 |  | – |  |
| >>Measurement Failure Cause List |  | 0..1 |  | Indicates that eNB2 could not initiate the measurement for at least one of the requested measurement objects in the cell | – |  |
| >>>Measurement Failure Cause Item |  | 1 .. <maxFailedMeasObjects> |  |  | EACH | ignore |
| >>>>Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object that failed to be initiated in the eNB2. First Bit = PRB Periodic, Second Bit = TNL load Ind Periodic, Third Bit = HW Load Ind Periodic, Fourth Bit = Composite Available Capacity Periodic, Fifth Bit = ABS Status Periodic, Sixth Bit = RSRP Measurement Report Periodic, Seventh Bit = CSI Report Periodic, Eighth Bit = CAC for Possibly Aggregated Cells Periodic. Other bits shall be ignored by the eNB1. | – |  |
| >>>>Cause | M |  | 9.2.6 | Failure cause for measurement objects for which the measurement cannot be initiated | – |  |

| Range bound | Explanation |
| --- | --- |
| maxFailedMeasObjects | Maximum number of measurement objects that can fail per measurement. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |

#### 9.1.2.13	RESOURCE STATUS FAILURE

This message is sent by the eNB2 to indicate that for none of the requested measurement objects the measurement can be initiated.

Direction: eNB2  eNB1.

| IE/Group Name | Presence |  | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  | 9.2.13 |  | YES | reject |
| eNB1 Measurement ID | M |  |  | INTEGER (1..4095,...) | Allocated by eNB1 | YES | reject |
| eNB2 Measurement ID | M |  |  | INTEGER (1..4095,...) | Allocated by eNB2 | YES | reject |
| Cause | M |  |  | 9.2.6 | Ignored by the receiver when the Complete Failure Cause Information IE is included | YES | ignore |
| Criticality Diagnostics |  | O |  | 9.2.7 |  | YES | ignore |
| Complete Failure Cause Information |  |  | 0..1 |  | Complete list of failure causes for all requested cells | YES | ignore |
| >Complete Failure Cause Information Item |  |  | 1 .. <maxCellineNB> |  |  | EACH | ignore |
| >>Cell ID | M |  |  | ECGI 9.2.14 |  | – |  |
| >>Measurement Failure Cause List |  |  | 1 |  |  | – |  |
| >>>Measurement Failure Cause Item |  |  | 1 .. <maxFailedMeasObjects> |  |  | EACH | ignore |
| >>>>Measurement Failed Report Characteristics | M |  |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object that failed to be initiated in the eNB2. First Bit = PRB Periodic, Second Bit = TNL load Ind Periodic, Third Bit = HW Load Ind Periodic, Fourth Bit = Composite Available Capacity Periodic, Fifth Bit = ABS Status Periodic, Sixth Bit = RSRP Measurement Report Periodic, Seventh Bit = CSI Report Periodic, Eighth Bit = CAC for Possibly Aggregated Cells Periodic. Other bits shall be ignored by the eNB1. | – |  |
| >>>>Cause | M |  |  | 9.2.6 | Failure cause for measurements that cannot be initiated | – |  |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxFailedMeasObjects | Max number of measurement objects that can fail per measurement. Value is 32. |

#### 9.1.2.14	RESOURCE STATUS UPDATE

This message is sent by eNB2 to neighbouring eNB1 to report the results of the requested measurements.

Direction: eNB2  eNB1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| eNB1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by eNB1 | YES | reject |
| eNB2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by eNB2 | YES | reject |
| Cell Measurement Result |  | 1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. <maxCellineNB> |  |  | EACH | ignore |
| >>Cell ID | M |  | ECGI 9.2.14 |  |  |  |
| >>Hardware Load Indicator | O |  | 9.2.34 |  |  |  |
| >>S1 TNL Load Indicator | O |  | 9.2.35 |  |  |  |
| >>Radio Resource Status | O |  | 9.2.37 |  |  |  |
| >>Composite Available Capacity Group | O |  | 9.2.44 |  | YES | ignore |
| >>ABS Status | O |  | 9.2.58 |  | YES | ignore |
| >>RSRP Measurement Report List | O |  | 9.2.76 |  | YES | ignore |
| >>CSI Report | O |  | 9.2.79 |  | YES | ignore |
| >>Cell Reporting Indicator | O |  | ENUMERATED(stop request, ...) |  | YES | ignore |
| >> Measurement Result for NR Cells Possibly Aggregated |  | 0..1 |  |  | YES | ignore |
| >>> Measurement Result for NR Cells Possibly Aggregated Item |  | 1 .. < maxnoofReportedNRCellsPossiblyAggregated > |  |  |  |  |
| >>>>Cell ID | M |  | NR CGI 9.2.111 |  |  |  |
| >>>>NR Composite Available Capacity Group | O |  | 9.2.163 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofReportedNRCellsPossiblyAggregated | Maximum no. of reported NR cells that may be aggregated with the serving cell. Value is 16. |

////////////////////////////////////////////////////////////////////////skip unrelated text////////////////////////////////////////////////////////////////////////

9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

X2AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-E-RAB-Item,

id-Number-of-Antennaports,

id-MBSFN-Subframe-Info,

id-PRACH-Configuration,

id-CSG-Id,

id-MDTConfiguration,

id-SignallingBasedMDTPLMNList,

id-MultibandInfoList,

id-FreqBandIndicatorPriority,

id-NeighbourTAC,

id-Time-UE-StayedInCell-EnhancedGranularity,

id-MBMS-Service-Area-List,

id-HO-cause,

id-eARFCNExtension,

id-DL-EARFCNExtension,

id-UL-EARFCNExtension,

id-M3Configuration,

id-M4Configuration,

id-M5Configuration,

id-MDT-Location-Info,

id-NRrestrictioninEPSasSecondaryRAT,

id-NRrestrictionin5GS,

id-AdditionalSpecialSubframe-Info,

id-UEID,

id-enhancedRNTP,

id-ProSeUEtoNetworkRelaying,

id-M6Configuration,

id-M7Configuration,

id-OffsetOfNbiotChannelNumberToDL-EARFCN,

id-OffsetOfNbiotChannelNumberToUL-EARFCN,

id-AdditionalSpecialSubframeExtension-Info,

id-BandwidthReducedSI,

id-extended-e-RAB-MaximumBitrateDL,

id-extended-e-RAB-MaximumBitrateUL,

id-extended-e-RAB-GuaranteedBitrateDL,

id-extended-e-RAB-GuaranteedBitrateUL,

id-extended-uEaggregateMaximumBitRateDownlink,

id-extended-uEaggregateMaximumBitRateUplink,

id-E-RABUsageReport-Item,

id-SecondaryRATUsageReport-Item,

id-UEAppLayerMeasConfig,

id-DL-scheduling-PDCCH-CCE-usage,

id-UL-scheduling-PDCCH-CCE-usage,

id-DownlinkPacketLossRate,

id-UplinkPacketLossRate,

id-serviceType,

id-ProtectedEUTRAResourceIndication,

id-NRS-NSSS-PowerOffset,

id-NSSS-NumOccasionDifferentPrecoder,

id-CNTypeRestrictions,

id-BluetoothMeasurementConfiguration,

id-WLANMeasurementConfiguration,

id-ECGI,

id-NRCGI,

id-MeNBCoordinationAssistanceInformation,

id-SgNBCoordinationAssistanceInformation,

id-NRNeighbourInfoToAdd,

id-LastNG-RANPLMNIdentity,

id-BPLMN-ID-Info-EUTRA,

id-NBIoT-UL-DL-AlignmentOffset,

id-UnlicensedSpectrumRestriction,

id-CarrierList,

id-FrequencyShift7p5khz,

id-NPRACHConfiguration,

id-MDTConfigurationNR,

id-CSI-RSTransmissionIndication,

id-QoS-Mapping-Information,

id-IntendedTDD-DL-ULConfiguration-NR,

id-TraceCollectionEntityURI,

id-SFN-Offset,

id-AdditionLocationInformation,

id-SSB-PositionsInBurst,

id-NRCellPRACHConfig,

id-ULCarrierList,

id-TDDULDLConfigurationCommonNR,

maxnoofBearers,

maxCellineNB,

maxEARFCN,

maxEARFCNPlusOne,

newmaxEARFCN,

maxInterfaces,

maxnoofBands,

maxnoofBPLMNs,

maxnoofAdditionalPLMNs,

maxnoofCells,

maxnoofEPLMNs,

maxnoofEPLMNsPlusOne,

maxnoofForbLACs,

maxnoofForbTACs,

maxnoofNeighbours,

maxnoofPRBs,

maxNrOfErrors,

maxPools,

maxnoofMBSFN,

maxnoofTAforMDT,

maxnoofCellIDforMDT,

maxnoofMBMSServiceAreaIdentities,

maxnoofMDTPLMNs,

maxnoofCoMPHypothesisSet,

maxnoofCoMPCells,

maxUEReport,

maxCellReport,

maxnoofPA,

maxCSIProcess,

maxCSIReport,

maxSubband,

maxnooftimeperiods,

maxnoofCellIDforQMC,

maxnoofTAforQMC,

maxnoofPLMNforQMC,

maxUEsinengNBDU,

maxnoofProtectedResourcePatterns,

maxnoNRcellsSpectrumSharingWithE-UTRA,

maxnoofNrCellBands,

maxnoofBluetoothName,

maxnoofWLANName,

maxofNRNeighbours,

maxnoofextBPLMNs,

maxnoofTLAs,

maxnoofGTPTLAs,

maxnoofTNLAssociations,

maxnoofCellsinCHO,	maxnoofPC5QoSFlows,

maxnoofSSBAreas,

maxnoofNRSCSs,

maxnoofNRPhysicalResourceBlocks,

maxnoofNonAnchorCarrierFreqConfig,

maxnoofRACHReports,

maxnoofPSCellsPerSN,

maxnoofReportedNRCellsPossiblyAggregated

FROM X2AP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM X2AP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-Single-Container{},

X2AP-PROTOCOL-EXTENSION,

X2AP-PROTOCOL-IES

FROM X2AP-Containers;

///////////////////////////////////////////////////////////////skip unrelated codes///////////////////////////////////////////////////////////////

#### -- N

///////////////////////////////////////////////////////////////skip unrelated codes///////////////////////////////////////////////////////////////

MeasurementResultforNRCellsPossiblyAggregated ::= SEQUENCE (SIZE(1.. maxnoofReportedNRCellsPossiblyAggregated)) OF MeasurementResultforNRCellsPossiblyAggregated-Item

MeasurementResultforNRCellsPossiblyAggregated-Item ::= SEQUENCE {

cellID										NRCGI,

nrCompositeAvailableCapacityGroup			NRCompositeAvailableCapacityGroup			OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {MeasurementResultforNRCellsPossiblyAggregated-Item-ExtIEs} }	OPTIONAL,

...

}

MeasurementResultforNRCellsPossiblyAggregated-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

///////////////////////////////////////////////////////////////skip unrelated codes///////////////////////////////////////////////////////////////

9.3.7	Constant definitions

///////////////////////////////////////////////////////////////skip unrelated codes///////////////////////////////////////////////////////////////

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxEARFCN									INTEGER ::= 65535

maxEARFCNPlusOne							INTEGER ::= 65536

newmaxEARFCN								INTEGER ::= 262143

maxInterfaces								INTEGER ::= 16

maxCellineNB								INTEGER ::= 256

maxnoofBands								INTEGER ::= 16

maxnoofBearers								INTEGER ::= 256

maxNrOfErrors								INTEGER ::= 256

maxnoofPDCP-SN								INTEGER ::= 16

maxnoofEPLMNs								INTEGER ::= 15

maxnoofEPLMNsPlusOne						INTEGER ::= 16

maxnoofForbLACs								INTEGER ::= 4096

maxnoofForbTACs								INTEGER ::= 4096

maxnoofBPLMNs								INTEGER ::= 6

maxnoofAdditionalPLMNs						INTEGER ::= 6

maxnoofNeighbours							INTEGER ::= 512

maxnoofPRBs									INTEGER ::= 110

maxPools									INTEGER ::= 16

maxnoofCells								INTEGER ::= 16

maxnoofMBSFN								INTEGER ::= 8

maxFailedMeasObjects						INTEGER ::= 32

maxnoofCellIDforMDT							INTEGER ::= 32

maxnoofTAforMDT								INTEGER ::= 8

maxnoofMBMSServiceAreaIdentities			INTEGER ::= 256

maxnoofMDTPLMNs								INTEGER ::= 16

maxnoofCoMPHypothesisSet					INTEGER ::= 256

maxnoofCoMPCells							INTEGER ::= 32

maxUEReport									INTEGER ::= 128

maxCellReport								INTEGER ::= 9

maxnoofPA									INTEGER ::= 3

maxCSIProcess								INTEGER ::= 4

maxCSIReport								INTEGER ::= 2

maxSubband									INTEGER ::= 14

maxofNRNeighbours							INTEGER ::= 1024

maxCellinengNB								INTEGER ::= 16384

--	maxnoofNRCarriers						INTEGER ::= 32

maxnooftimeperiods							INTEGER ::= 2

maxnoofCellIDforQMC							INTEGER ::= 32

maxnoofTAforQMC								INTEGER ::= 8

maxnoofPLMNforQMC							INTEGER ::= 16

maxUEsinengNBDU								INTEGER ::= 8192

maxnoofProtectedResourcePatterns			INTEGER ::= 16

maxnoNRcellsSpectrumSharingWithE-UTRA		INTEGER ::= 64

maxnoofNrCellBands							INTEGER ::= 32

maxnoofBluetoothName						INTEGER ::= 4

maxnoofWLANName								INTEGER ::= 4

maxnoofextBPLMNs							INTEGER ::= 12

maxnoofTLAs									INTEGER ::= 16

maxnoofGTPTLAs								INTEGER ::= 16

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofCellsinCHO							INTEGER ::= 8

maxnoofPC5QoSFlows 							INTEGER ::= 2048

maxnoofSSBAreas								INTEGER ::= 64

maxnoofNRSCSs								INTEGER ::= 5

maxnoofNRPhysicalResourceBlocks				INTEGER ::= 275

maxnoofNonAnchorCarrierFreqConfig			INTEGER ::= 15

maxnoofRACHReports							INTEGER ::= 64

maxnoofPSCellsPerSN                         INTEGER ::= 16

maxnoofReportedNRCellsPossiblyAggregated	INTEGER ::= 16

//////////////////////////////////////////////////////////////end/////////////////////////////////////////////////////////////