3GPP TSG-RAN3 Meeting #115-e	R3-222929

# Online, , 21th Feb 2022 – 3rd Mar 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.460 | CR | 0047 | rev | 6 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NR MBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-03-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | NR MBS Service is supported in release 17. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NR MBS is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR MBS is not supported in case of architecture for separation gNB-CU-CP and gNB-CU-UP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, 5.1.x (new), 6.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | rev1: resubmission in R3-210787 rev2: resubmission in R3-211484 rev3: resubmission in R3-213150 rev4: resubmission in R3-214613 rev5: resubmission in R3-220009 rev6: capture TP agreed in RAN3#115-e - R3-222896 [TP for BL CR 38.460] Multicast and Broadcast E1AP functions |  |  |  |  |  |  |  |  |

Start of the First Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.401: "NG-RAN; Architecture Description".

[3]	3GPP TS 38.461: "NG-RAN; E1 layer 1".

[4]	3GPP TS 38.462: "NG-RAN; E1 signalling transport".

[5]	3GPP TS 38.463: "NG-RAN; E1 Application Protocol (E1AP)".

[6]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[7]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[x]	3GPP TS 23.247: "5G multicast-broadcast services; Stage 2".

Start of the Next Change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

DL	Downlink

DRB	Data Radio Bearer

E1AP	E1 Application Protocol

IP	Internet Protocol

MBS	Multicast/Broadcast Service

PTP	Point to Point

PTM	Point to Multipoint

SCTP	Stream Control Transmission Protocol

TNL	Transport Network Layer

Start of the Next Change

5.1.x	E1 bearer context management function for NR MBS

The E1 bearer context management function for NR MBS consists of two sub-sets for functions, one for NR MBS broadcast, one for NR MBS multicast.

Both sets follow the principles of the E1 bearer context management functions, with the following differences

-	E1 NR MBS procedure concerns a single MBS Session Resource only.

-	E1 NR MBS procedures concern the control of MRB resources in gNB-CU-UP

Editor’s Note:	Specification of gNB-CU-UP initiated procedures for both, multicast and broadcast, are FFS and requires specification text if not defined.

-	E1 NR MBS procedures do not contol security information, as for NR MBS, PDCP does not apply security as specified in TS 38.300 [4].

-	QoS flow to MRB mapping is determined by the gNB-CU-CP or, in case of shared NR-U terminations, the gNB-CU-UP may be notified about the QoS flow to MRB mapping already determined in the bearer context for the shared NR-U termination. The gNB-CU-CP may inform the gNB-CU-UP whether it is contended with the already determined mapping decision.

NOTE:	Not all QoS flow parameters are applicable for NR MBS, as specified in TS 23.247 [x].

Editor’s Note:	Some specification text for data forwarding, where applicable, to be inserted here.

Editor’s Note:	Header Compression text to be inserted here as well. So far only schemes w/o feedback schemes assumed.

-	DL data arrival detection is not applicable for NR MBS.

-	Data volume reporting is not applicable for NR MBS.

-	Suspension and resumption of bearer contexts is not applicable for NR MBS.

-	CA based packet duplication is not applicable for NR MBS.

Start of the Next Change

## 6.x	NR MBS procedures

The E1 MBS procedures are listed below:

-	Broadcast E1AP MBS procedures

-	BC Bearer Context Setup

-	BC Bearer Context Modification (gNB-CU-CP inititated)

-	BC Bearer Context Modification (gNB-CU-UP inititated)

-	BC Bearer Context Release (gNB-CU-CP inititated)

-	BC Bearer Context Release (gNB-CU-UP inititated)

-	Multicast E1AP MBS procedures

-	MC Bearer Context Setup

-	MC Bearer Context Modification (gNB-CU-CP inititated)

-	MC Bearer Context Modification (gNB-CU-UP inititated)

-	MC Bearer Context Release (gNB-CU-CP inititated)

-	MC Bearer Context Release (gNB-CU-UP inititated)

End of the Changes