3GPP TSG-RAN WG3 Meeting #115-e	R3-222941

# E-meeting, 21 February – 3 March, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.410 | CR | 0034 | rev | 4 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BLCR to 38.410: Support of QoE Measurement Collection for NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, ZTE, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE-Core |  |  |  |  |  | Date: |  |  | 2022-03-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | BL CR for introduction of QoE measurement collection in NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of QMC function and QMC procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New feature not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.x1 (new), 6.x2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 1: Include TP agreed at RAN3#114-e: R3-216113 Added abbreviation: QoE Rev. 2: Submission to RAN3#114bis-e Rev. 3: Include TP agreed at RAN3#114bis-e: R3-221269. Submission to RAN3#115-e. Rev. 4: Include TP agreed at RAN3#115-e: R3-222889 |  |  |  |  |  |  |  |  |

## 3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

AMF	Access and Mobility Management Function

CIoT	Cellular IoT

NB-IoT	Narrow Band Internet of Things

NG-U	NG User plane interface

QMC	QoE Measurement Collection

QoE	Quality of Experience

RIM	Remote Interference Management

SMF	Session Management Function

UP	User Plane

UPF	User Plane Function

## 5.x1	QMC function

The QMC function provides means to manage the QMC sessions and support the mobility of QMC sessions over the NG interface.

## 6.x2	QMC procedures

The following procedures are used to control the QMC sessions in the UE and to transfer QMC session information to the target NG-RAN node during a UE’s intra-system intra-RAT mobility:

-	Initial Context Setup;

-	UE Context Modification;

-	Handover Preparation;

-	Handover Resource Allocation.