3GPP TSG-RAN3 Meeting #115-e	R3-221661

Electronic meeting, 21 February - 3 March 2022

3GPP TSG RAN WG2 Meeting #116bis-e	R2-2201712

Electronic meeting, 17th -25th  January, 2022

Title:	LS to RAN3 on CPAC

Release:	Rel-17

Work Item:	LTE_NR_DC_enh2-Core

Source:	RAN2

To:	RAN3

Cc:	-

Contact Person:

#### Name:	Erlin Zeng

E-mail Address:	erlin.zeng@catt.cn

Attachments:

1. Overall description:

RAN2 discussed some remaining open issues on SN-initiated CPC and MN-initiated CPAC and reached the following agreements:

SN-initiated CPC

RAN2 to confirm that in SN initiated inter-SN CPC, the S-SN is always informed about which candidates were accepted/rejected by T-SN.

RAN2 thinks that MN is optional to indicate S-SN the candidates accepted or rejected by T-SN after receiving the SN addition acknowledge message from T-SN. And if MN skips the indication to S-SN before sending the CPC configuration to the UE, it sends the indication of accepted cells by T-SN to S-SN in some later step in the procedure. Up to RAN3 how the signalling is done efficiently.

It is up to RAN3 to decide the message used for indicating the accepted PSCells from MN to S-SN, before sending the RRC Reconfiguration message including the CPC configurations to the UE.

It is up to RAN3 to decide the message used for indicating the accepted PSCells from MN to S-SN, after sending CPC configuration to the UE.

It is up to RAN3 to decide the message used for S-SN to provide the updated configuration.

It is up to RAN3 to decide the message used for providing the RRCReconfigurationComplete message from MN to S-SN.

MN-initiated CPAC

MN provides separate list of proposed PSCells to T-SN, and uses legacy candidate cell information (candidateCellInfoListMN) to provide the candidate cells recommended by MN to T-SN.

For MN initiated CPAC, only the cells within the list recommended by MN can be chosen by T-SN.

2. Actions:

To 3GPP RAN3

ACTION:

RAN2 respectfully asks RAN3 to take the above agreements into account, in their further work on SN-initiated CPC and MN-initiated CPAC When related decisions are taken in RAN3, RAN2 kindly requests RAN3 to share relevant information with regard to above.

3. Date of next TSG RAN WG2 meetings:

RAN2#117-e		21st February to 03 March 2022, E-meeting

RAN2#118-e		16th to 27th May 2022, E-meeting