3GPP TSG-RAN WG3 Meeting #116-e	R3-223058

Online, 9th – 19th May 2022

Agenda item:	9.1.10.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Resolution of open issues for time synchronisation enhancements

Document for:	Discussion and Decision

# 1	Introduction

In this paper, we provide our views on the remaining open issues for time synchronisation enhancements.

# 2	Discussion

## 2.1	PDC measurement periodicity

The values of the PDC Measurement Periodicity IE were chosen by RAN3 to be aligned with the values of the Measurement Periodicity IE in the NRPPa: E-CID MEASUREMENT INITIATION REQUEST message.

However, in case of RTT-based gNB side PDC, the gNB-CU needs both the gNB Rx-Tx time difference measurements (from the gNB-DU) and the UE Rx-Tx time difference measurements (from the UE) in order to perform the PDC. Therefore, it would be beneficial if the periodicity of the gNB Rx-Tx time difference measurements (F1AP) is aligned with that of the UE Rx-Tx time difference measurements (RRC).

RAN2 has agreed to the following periodicity values for UE Rx-Tx time difference measurements in RRC:

RxTxPeriodical-r17  ::=                     SEQUENCE {

rxTxReportInterval-r17                      RxTxReportInterval-r17,

reportAmount-r17                            ENUMERATED {r1, infinity, spare6, spare5, spare4, spare3, spare2, spare1},

...

}

RxTxReportInterval-r17 ::= ENUMERATED {ms80,ms120,ms160,ms240,ms320,ms480,ms640,ms1024,ms1280,ms2048,ms2560,ms5120,spare4,spare3,spare2,spare1}

It is therefore proposed to use these same values for the PDC measurement periodicity.

Proposal 1:	The values of the PDC Measurement Periodicity IE in F1AP should be aligned with the values of the RxTxReportInterval-r17 IE in RRC.

## 2.2	PDC timing advance granularity

The NR PDC Timing Advance IE is expressed in units of [64*] ns, which gives a granularity of 32ns that is aligned with the granularity of Rx-Tx measurements with k=5.  This accuracy seems sufficient for gNB side PDC.

Proposal 2:	No change is needed to the NR PDC Timing Advance IE, i.e. current value in units of [64*] ns and range up to 2ms are sufficient.

## 2.3	NG-based and Xn-based Handover

RAN3 has received an LS from SA2 in [1], asking RAN3 view on transfer of the UE-specific “5G access stratum time distribution indication and the Uu time synchronization error budget” during Xn/NGAP HO.

Related to SA2’s question, RAN3 reached the following agreements in RAN3#114 and RAN3#114bis which are already reflected in the Release 17 NGAP/XnAP specifications:

-	The Time Synchronisation Assistance Information IE includes two sub-IEs:

-	Time Distribution Indication IE encoded as ENUMERATED type with two codepoints (enabled, disabled);

-	Uu Time Synchronisation Error Budget IE encoded as INTEGER type having range 1ns to 1ms, with 1ns granularity.

-	The Time Synchronisation Assistance Information IE is introduced as an optional UE-level parameter in:

-	NGAP: INITIAL CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST, HANDOVER REQUEST, and PATH SWITCH REQUEST ACKNOWLEDGEMENT messages.

-	XnAP: HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE messages.

Therefore, RAN3 can respond to SA2 that the UE-specific “5G access stratum time distribution indication and the Uu time synchronization error budget” are transferred during handover as follows:

-	RAN3 has defined the Time Synchronisation Assistance Information IE to contain the time distribution indication and the Uu time synchronization error budget.

-	In case of Xn-based handover, the Time Synchronisation Assistance Information IE is passed from source gNB to target gNB using the XnAP: HANDOVER REQUEST message.

-	In case of NG-based handover, the Time Synchronisation Assistance Information IE is provided from the AMF to the target gNB using the NGAP: HANDOVER REQUEST message.

Proposal 3:	Respond to the SA2 LS that the 5G access stratum time distribution indication and the Uu time synchronization error budget are transferred during Xn/NGAP HO as above.

# 3	Conclusions

In this paper, we analysed the remaining open issues for time synchronization enhancements. The following is proposed:

Proposal 1:	The values of the PDC Measurement Periodicity IE in F1AP should be aligned with the values of the RxTxReportInterval-r17 IE in RRC.

Proposal 2:	No change is needed to the NR PDC Timing Advance IE, i.e. current value in units of [64*] ns and range up to 2ms are sufficient.

Proposal 3:	Respond to the SA2 LS that the 5G access stratum time distribution indication and the Uu time synchronization error budget are transferred during Xn/NGAP HO as above.

An F1AP CR for Proposal 1 is provided in [2], and a Reply LS for Proposal 3 is provided in Annex A.

# References

- R3-223034 LS On handover issue for 5G access stratum time distribution, SA2

- R3-223059 Correction of PDC Measurement Periodicity values, Nokia, Nokia Shanghai Bell

# Annex	A: Draft Reply LS to SA2

Title:	Reply LS on handover issue for 5G access stratum time distribution

Release:	Rel-17

Work Item:	IIoT

Source:	RAN3

To:	SA2

Cc:

Contact Person:

Name:	Sean Kelley

E-mail Address:	sean.kelley@nokia.com

Attachments:	n/a

1. Overall Description:

RAN3 would like to thank SA2 for the LS on handover issue for 5G access stratum time distribution. The RAN3 response to the SA2 question is as follows:

-	RAN3 has defined the Time Synchronisation Assistance Information IE to contain the time distribution indication and the Uu time synchronization error budget.

-	In case of Xn-based handover, the Time Synchronisation Assistance Information IE is passed from source gNB to target gNB using the XnAP: HANDOVER REQUEST message.

-	In case of NG-based handover, the Time Synchronisation Assistance Information IE is provided from the AMF to the target gNB using the NGAP: HANDOVER REQUEST message.

2. Actions:

To SA2 and RAN2 groups.

ACTION: 	RAN3 kindly asks SA2 to take the above into account.

3. Date of Next TSG-RAN WG3 Meetings:

RAN3#117	22 – 26 August 2022		Toulouse