3GPP TSG-RAN WG3 #116-e	R3-223082

# Online, 9th – 19th May 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1873 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on usage of RFSP index |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC., Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-3-17 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAT/Frequency Selection Priority(RFSP Index) which is encoded as INTEGER (1..256) that has been introduced in EPS and 5GC according to TS 23.401 and TS 23.501. This parameter is used by RAN locally in order to apply specific RRM strategies. E.g. - to derive UE specific cell reselection priorities to control idle mode camping. - to decide on redirecting active mode UEs to different frequency layers or RATs. While in RAN3 spec, RFSP index is specified as different IE name i.e. Subscriber Profile ID for RAT/Frequency Priority in LTE (TS 36.413) and Index to RAT/Frequency Selection Priority (TS 38.413) in NR respectively. With this different naming, for inter-RAT handover from LTE to NR, it is impossible to inherit RFSP index from eNB to gNB through Source NG-RAN node to Target NG-RAN node Transparent Container. Note the RFSP index can be included in Forward Relcation Request that is not differentiated among MME/SCSN/AMF according to TS29.274 7.3.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add change in subcaluse 8.4.1.2 as following: ------------------------------------------- For inter-system handover from eNB to NG-RAN node, if source eNB needs to send Subscriber Profile ID for RAT/Frequency priority IE to the target NG-RAN node, it shall rewrite the content of Subscriber Profile ID for RAT/Frequency priority IE to Index to RAT/Frequency Selection Priority IE and include it in the Source NG-RAN node to Target NG-RAN node Transparent Container IE, and the target NG-RAN node shall store it in the UE context. -------------------------------------------- Imapct analysis: This CR has isolated impact on Handover Preparation procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For inter-RAT handover from NR to LTE, it is impossible to inherit RFSP index from gNB to eNB through Source eNB to Target eNB Transparent Container. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

UNCHANGED TEXT OMITTED

8.4.1	Handover Preparation

8.4.1.1	General

The purpose of the Handover Preparation procedure is to request the preparation of resources at the target side via the EPC. There is only one Handover Preparation procedure ongoing at the same time for a certain UE.

8.4.1.2	Successful Operation

Figure 8.4.1.2-1: Handover preparation: successful operation

The source eNB initiates the handover preparation by sending the HANDOVER REQUIRED message to the serving MME. When the source eNB sends the HANDOVER REQUIRED message, it shall start the timer TS1RELOCprep. The source eNB shall indicate the appropriate cause value for the handover in the Cause IE.

The source eNB shall include the Source to Target Transparent Container IE in the HANDOVER REQUIRED message.

In case of intra-system handover, the information in the Source to Target Transparent Container IE shall be encoded according to the definition of the Source eNB to Target eNB Transparent Container IE. In case of handover to UTRAN, the information in the Source to Target Transparent Container IE shall be encoded according to the Source RNC to Target RNC Transparent Container IE definition as specified in TS 25.413 [19] and the source eNB shall include the UE History Information IE in the Source RNC to Target RNC Transparent Container IE. If the handover is to GERAN A/Gb mode then the information in the Source to Target Transparent Container IE shall be encoded according to the definition of the Source BSS to Target BSS Transparent Container IE as described in TS 48.018 [18]. If the handover is to NG-RAN, the information in the Source to Target Transparent Container IE shall be encoded according to the Source NG-RAN Node to Target NG-RAN Node Transparent ContainerIE definition as specified in TS 38.413 [44].

When the preparation, including the reservation of resources at the target side is ready, the MME responds with the HANDOVER COMMAND message to the source eNB.

If the Target to Source Transparent Container IE has been received by the MME from the handover target then the transparent container shall be included in the HANDOVER COMMAND message.

Upon reception of the HANDOVER COMMAND message the source eNB shall stop the timer TS1RELOCprep and start the timer TS1RELOCOverall.

In case of intra-system handover, the information in the Target to Source Transparent Container IE shall be encoded according to the definition of the Target eNB to Source eNB Transparent Container IE. In case of inter-system handover to UTRAN, the information in the Target to Source Transparent Container IE shall be encoded according to the Target RNC to Source RNC Transparent Container IE definition as specified in TS 25.413 [19]. In case of inter-system handover to GERAN A/Gb mode, the information in the Target to Source Transparent Container IE shall be encoded according to the Target BSS to Source BSS Transparent Container IE definition as described in TS 48.018 [18]. In case of inter-system handover to NG-RAN, the information in the Target to Source Transparent Container IE shall be encoded according to the Target NG-RAN Node to Source NG-RAN Node Transparent Container IE definition as specified in TS 38.413 [44].

For inter-system handover from eNB to NG-RAN node, if source eNB needs to send Subscriber Profile ID for RAT/Frequency priority IE to the target target NG-RAN node, it shall rewrite the content of Subscriber Profile ID for RAT/Frequency priority IE to Index to RAT/Frequency Selection Priority IE and include it in the Source NG-RAN node to Target NG-RAN node Transparent Container IE, and the target NG-RAN node shall store it in the UE context.

If the Direct Forwarding Path Availability IE is included in the Target NG-RAN Node to Source NG-RAN Node Transparent Container IE within the HANDOVER COMMAND message, the source eNB shall, if supported, use it for direct data forwarding between the source SN and the target NG-RAN node.

If there are any E-RABs that could not be admitted in the target, they shall be indicated in the E-RABs to Release List IE.

If the DL forwarding IE is included within the Source eNB to Target eNB Transparent Container IE of the HANDOVER REQUIRED message and it is set to “DL forwarding proposed”, it indicates that the source eNB proposes forwarding of downlink data.

If the MME receives the Direct Forwarding Path Availability IE in the HANDOVER REQUIRED message indicating that a direct data path is available, it shall handle it as specified in TS 23.401 [11].

If the CSG Id IE and no Cell Access Mode IE are received in the HANDOVER REQUIRED message, the MME shall perform the access control according to the CSG Subscription Data of that UE and, if the access control is successful or if at least one of the E-RABs has a particular ARP value (see TS 23.401 [11]), it shall continue the handover and propagate the CSG Id IE to the target side. If the access control is unsuccessful but at least one of the E-RABs has a particular ARP value (see TS 23.401 [11]) the MME shall also provide the CSG Membership Status IE set to “non member” to the target side.

If the CSG Id IE and the Cell Access Mode IE set to “hybrid” are received in the HANDOVER REQUIRED message, the MME shall provide the membership status of the UE and the CSG Id to the target side.

The source eNB shall include the SRVCC HO Indication IE in the HANDOVER REQUIRED message if the SRVCC operation is needed as defined in TS 23.216 [9]. The source eNB shall indicate to the MME in the SRVCC HO Indication IE if the handover shall be prepared for PS and CS domain or only for CS domain. The SRVCC HO Indication IE is set according to the target cell capability and UE capability. In case the target system is GERAN without DTM support or the UE is without DTM support, the source eNB shall indicate “CS only” in the SRVCC HO Indication IE and “PS service not available” in PS Service Not Available IE. In case the target system is either GERAN with DTM but without DTM HO support and the UE is supporting DTM or the target system is UTRAN without PS HO support, the source eNB shall indicate “CS only” in the SRVCC HO Indication IE. Otherwise, the source eNB shall indicate “PS and CS” in the SRVCC HO Indication IE.

In case of inter-system handover from E-UTRAN, the source eNB shall indicate in the Target ID IE, in case the target system is UTRAN, the Target RNC-ID of the RNC (including the Routing Area Code only in case the UTRAN PS domain is involved), in case the target system is GERAN the Cell Global Identity (including the Routing Area Code only in case the GERAN PS domain is involved) of the cell, and in case the target system is NG-RAN the Target NG-RAN Node ID of the NG-RAN node in the target system.

In case of inter-system handover from E-UTRAN to UTRAN, the source eNB shall, if supported, include the HO Cause Value IE in the UE History Information IE of the HANDOVER REQUIRED message.

In case the SRVCC operation is performed and the SRVCC HO Indication IE indicates that handover shall be prepared only for CS domain, and if

-	the target system is GERAN, then the source eNB

-	shall encode the information in the Source to Target Transparent Container IE within the HANDOVER REQUIRED message, according to the definition of the Old BSS to New BSS information IE as specified in TS 48.008 [23], and

-	shall not include the Source to Target Transparent Container Secondary IE in the HANDOVER REQUIRED message;

-	the target system is UTRAN, then the source eNB

-	shall encode the information in the Source to Target Transparent Container IE within the HANDOVER REQUIRED message according to the definition of the Source RNC to Target RNC Transparent Container IE as specified in TS 25.413 [19],

-	shall include the UE History Information IE in the Source RNC to Target RNC Transparent Container IE, and

-	shall not include the Source to Target Transparent Container Secondary IE in the HANDOVER REQUIRED message.

In case the SRVCC operation is performed, the SRVCC HO Indication IE in the HANDOVER REQUIRED message indicates that handover shall be prepared for PS and CS domain, and if

-	the target system is GERAN with DTM HO support, then the source eNB

-	shall encode the information in the Source to Target Transparent Container IE within the HANDOVER REQUIRED message according to the definition of the Source BSS to Target BSS Transparent Container IE as described in TS 48.018 [18],and

-	shall include the Source to Target Transparent Container Secondary IE in the HANDOVER REQUIRED message and encode information in it according to the definition of the Old BSS to New BSS information IE as specified in TS 48.008 [23];

-	the target system is UTRAN, then the source eNB

-	shall encode the information in the Source to Target Transparent Container IE within the HANDOVER REQUIRED message according to the definition of the Source RNC to Target RNC Transparent Container IE as specified in TS 25.413 [19],

-	shall include the UE History Information IE in the Source RNC to Target RNC Transparent Container IE, and

-	shall not include the Source to Target Transparent Container Secondary IE in the HANDOVER REQUIRED message.

In case the SRVCC operation is performed, the SRVCC HO Indication IE in the HANDOVER REQUIRED message indicates that handover shall be prepared only for CS domain, and if

-	the target system is GERAN, then the MME

-	shall encode the information in the Target to Source Transparent Container IE within the HANDOVER COMMAND message according to the definition of the Layer 3 Information IE as specified in TS 48.008 [23], and

-	shall not include the Target to Source Transparent Container Secondary IE in the HANDOVER COMMAND message;

-	the target system is UTRAN, then the MME

-	shall encode the information in the Target to Source Transparent Container IE within the HANDOVER COMMAND message according to the definition of the Target RNC to Source RNC Transparent Container IE as specified in TS 25.413 [19], and

-	shall not include the Target to Source Transparent Container Secondary IE in the HANDOVER COMMAND message.

In case the SRVCC operation is performed, the SRVCC HO Indication IE in the HANDOVER REQUIRED message indicates that handover shall be prepared for PS and CS domain,

-	the target system is GERAN with DTM HO support, and if

-	the Handover Preparation procedure has succeeded in the CS and PS domain, then the MME

-	shall encode the information in the Target to Source Transparent Container IE within the HANDOVER COMMAND message according to the definition of the Layer 3 Information IE as specified in TS 48.008 [23], and

-	shall include the Target to Source Transparent Container Secondary IE in the HANDOVER COMMAND message and encode information in it according to the definition of the Target BSS to Source BSS Transparent Container IE as specified in TS 48.018 [18];

-	the Handover Preparation procedure has succeeded in the CS domain only, then the MME

-	shall encode the information in the Target to Source Transparent Container IE within the HANDOVER COMMAND message according to the definition of the Layer 3 Information IE as specified in TS 48.008 [23], and

-	shall not include the Target to Source Transparent Container Secondary IE in the HANDOVER COMMAND message;

-	the target system is UTRAN, then the Handover Preparation procedure shall be considered successful if the Handover Preparation procedure has succeeded in the CS domain, and the MME

-	shall encode the information in the Target to Source Transparent Container IE within the HANDOVER COMMAND message according to the definition of the Target RNC to Source RNC Transparent Container IE as specified in TS 25.413 [19], and

-	shall not include the Target to Source Transparent Container Secondary IE in the HANDOVER COMMAND message.

If the HANDOVER COMMAND message contains the DL GTP-TEID IE and the DL Transport Layer Address IE for a given bearer in the E-RABs Subject to Forwarding List IE, then the source eNB shall consider that the forwarding of downlink data for this given bearer is possible.

If the HANDOVER COMMAND message contains the UL GTP-TEID IE and the UL Transport Layer Address IE for a given bearer in the E-RABs Subject to Forwarding List IE, then it means the target eNB has requested the forwarding of uplink data for this given bearer.

If the DAPS Request Information IE is included for an E-RAB in the Source eNB to Target eNB Transparent Container IE within the HANDOVER REQUIRED message, it indicates that the request concerns a DAPS Handover for that E-RAB, as described in TS 36.300 [14].

Interactions with E-RAB Management procedures:

If, after a HANDOVER REQUIRED message is sent and before the Handover Preparation procedure is terminated, the source eNB receives an MME initiated E-RAB Management procedure on the same UE associated signalling connection, the source eNB shall either:

1.	cancel the Handover Preparation procedure by executing the Handover Cancel procedure with an appropriate cause value. After successful completion of the Handover Cancel procedure, the source eNB shall continue the MME initiated E-RAB Management procedure

or

2.	terminate the MME initiated E-RAB Management procedure by sending the appropriate response message with an appropriate cause value, e.g., “S1 intra system Handover Triggered”, “S1 inter system Handover Triggered” to the MME and then the source eNB shall continue with the handover procedure.

8.4.1.3	Unsuccessful Operation

Figure 8.4.1.3-1: Handover preparation: unsuccessful operation

If the EPC or the target system is not able to accept any of the bearers or a failure occurs during the Handover Preparation, the MME sends the HANDOVER PREPARATION FAILURE message with an appropriate cause value to the source eNB.

If the CSG Id IE and no Cell Access Mode IE are received in the HANDOVER REQUIRED message and the access control is unsuccessful and none of the E-RABs has a particular ARP value (see TS 23.401 [11]) the MME shall send the HANDOVER PREPARATION FAILURE message with an appropriate cause value to the source eNB, except when one of the E-RABs has a particular ARP value (see TS 23.401 [11]). Upon reception, the source eNB may decide to prevent handover for that UE towards CSG (Closed Access Mode) cells with corresponding CSG Id.

Interaction with Handover Cancel procedure:

If there is no response from the EPC to the HANDOVER REQUIRED message before timer TS1RELOCprep expires in the source eNB, the source eNB should cancel the Handover Preparation procedure by initiating the Handover Cancel procedure with the appropriate value for the Cause IE. The source eNB shall ignore any HANDOVER COMMAND message or HANDOVER PREPARATION FAILURE message received after the initiation of the Handover Cancel procedure.

8.4.1.4	Abnormal Conditions

If the eNB receives at least one E-RAB ID included in the E-RABs Subject to Forwarding List IE without at least one valid associated tunnel address pair (in either UL or DL), then the eNB shall consider it as a logical error and act as described in subclause 10.4. A GTP tunnel address pair is considered valid if both the GTP-TEID IE and the Transport Layer Address IE are present.

UNCHANGED TEXT OMITTED

END OF CHANGES