3GPP TSG-RAN WG3 #116-e							        R3-223105

Online, 9th – 19th May 2022

Source:               ZTE

Title:                    Consideration on SA2 LS on handover issue for 5G access stratum time distribution

Agenda item:      9.1.10.1

Document for:    Discussion and Decision

Introduction

In the Rel-17, RAN3 has already finished the related specification for the 5G access stratum time distribution indication and the Uu time synchronization error budget. In RAN3#116-e meeting, we received a LS[1] on  handover issue for 5G access stratum time distribution as follows:

1. Overall Description: In the Rel-17, the AMF may send the 5G access stratum time distribution indication and the Uu time synchronization error budget to NG-RAN for a specific UE. Question: What is RAN3 view on transfer of such context during Xn/NGAP HO? According to RAN3 answer, SA2 will update the stage 2 specification. To RAN WG3 group. ACTION: RAN3 is kindly requested to answer the question above.

In this contribution, we will mainly discuss the reply LS to [1], and give our proposals.

Discussion

RAN3 has agreed to introduce a Time Synchronisation Assistance Information IE that includes a Time Distribution Indication IE and Uu Time Synchronisation Error Budget IE. Wherein the Time Synchronisation Assistance Information IE is a UE-level parameter reflecting the most stringent time synchronisation budget for a given UE. The following structure for the Time Synchronisation Assistance Information IE:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Time Distribution Indication | M |  | ENUMERATED (enabled, disabled, …) |  |
| Uu Time Synchronisation Error Budget | C-ifEnabled |  | INTEGER (1..1000000, …) | Expressed in units of 1ns. |

Furthermore, RAN3 has agreed that the Time Synchronisation Assistance Information IE is a UE-level parameter that can be optionally included over NGAP in the following messages [2]:

- INITIAL CONTEXT SETUP REQUEST,

- UE CONTEXT MODIFICATION REQUEST,

- HANDOVER REQUEST,

- PATH SWITCH REQUEST ACKNOWLEDGEMENT.

And RAN3 has agreed to introduce the Time Synchronisation Assistance Information IE (same as NGAP) as an optional UE-level parameter in the following XnAP messages [3]:

- HANDOVER REQUEST,

- RETRIEVE UE CONTEXT RESPONSE.

For the Xn-based handover case, in order that the target gNB can quickly synchronize the clock with the UE, it is beneficial for the source gNB to send the above Time Synchronisation Assistance Information to the target gNB in the HANDOVER REQUEST. Furthermore, for some special scenarios (e.g. the source gNB is legacy gNB and cannot deliver the Time Synchronisation Assistance Information to the target gNB, or the Time Synchronisation Assistance Information changes at the time of handover), the core network can send new or updated Time Synchronisation Assistance Information to the target gNB through the PACH SWITCH REQUEST ACKNOWLEDGE message. Among them, the PACH SWITCH REQUEST ACKNOWLEDGE message appears in the final stage of Xn-based handover.

For the Ng-based handover case, in order that the target gNB can quickly synchronize the clock with the UE, it is beneficial for the AMF to send the above Time Synchronisation Assistance Information to the target gNB in the HANDOVER REQUEST. The core network also has the option to update the information at any time via the UE CONTEXT MODIFICATION REQUEST message. However, the Path Switch Request procedure would provide the core network a trigger to update the Time Synchronization Assistance Information (e.g., new Uu Time Synchronisation Error Budget value, or enable/disable time distribution for the UE) at the target gNB if the AMF has new information available provided by the TSCTSF that was not updated at the source gNB.

In addition, in the previous discussion of SA2, some companies had doubts about the need to include Time Synchronisation Assistance Information in PACH SWITCH REQUEST ACKNOWLEDGE message. Therefore, we add a NOTE to our LS reply to explain the above question, and provide the Reply LS on handover issue for 5G access stratum time distribution in [4].

Proposal 1: Reply LS to SA2 and mention the reason why RAN3 included Time Synchronisation Assistance Information in PACH SWITCH REQUEST ACKNOWLEDGE message.

Conclusions

In this contribution, we make the following observations and proposals:

Proposal 1: Reply LS to SA2 and mention the reason why RAN3 included Time Synchronisation Assistance Information in PACH SWITCH REQUEST ACKNOWLEDGE message.

- References

- 3GPP, R3-223034, LS On handover issue for 5G access stratum time distribution, RAN3#116, May, 2022.

- 3GPP, R3-221552, Introduction of enhanced Industrial IoT over NG, RAN3#115, Mar, 2022

- 3GPP, R3-221553, Introduction of enhanced Industrial IoT over XN, RAN3#115, Mar, 2022

- 3GPP, R3-223106, Reply LS on handover issue for 5G access stratum time distribution, RAN3#116, May, 2022.