3GPP TSG-RAN WG3 #116-e																		  R3-223109

9th – 19th May 2022

Online

Title: 	Discussion on how to buffer the CG-SDT data

Source: 	 ZTE, China Telecom

Agenda item: 	9.1.9.1

Document for:	Discussion and Decision

- Introduction

In the last RAN3 #115 emeeting, the issue on “for CG-SDT, whether gNB-DU or gNB-CU-UP shall buffer the SDT UL data/NAS PDU” is left into this RAN3 meeting. Two options are listed in R3-222652

- Option 1: The UL small data/UL NAS PDU shall be buffered at gNB-DU until it receives SN modification requet message including a new indicator (e.g., verification pass information)

- Option 2: The gNB-CU-UP could buffer the data from gNB-DU before the resume indication received from CU-CP (No stage3 impact)

In the last meeting, nearly half-half companies support option1-option2 and some companies have no strong view.

- Discussion

  - Overall procedure of option 1 and option 2

Option 1: gNB-DU buffers

- gNB-DU buffers the UL SDT data and/or UL NAS PDU

- gNB-CU-CP verifies the UE successfully

- Step 11/12, gNB-CU-CP intiates E1AP procedure to resume the SDT DRB/SDB

- Step 13/14, gNB-CU-CP initiats F1AP procedure to indicate verification pass, including the reused IE or new IE

- gNB-DU transmits the bufferer UL SDT data and/or UL NAS PDU

Option 2: gNB-CU-UP buffers

- gNB-DU does not buffer the UL SDT data and/or UL NAS PDU, instead sends them to gNB-CU directly

- gNB-CU-CU/UP buffers the UL SDT data and/or UL NAS PDU

- gNB-CU-CP verifies the UE successfully

- Step 11/12, the gNB-CU-CP intiates E1AP procedure to resume the SDT DRB/SDB

- gNB-CU-CP/UP transmits the bufferer UL SDT data and/or UL NAS PDU

In summary, for option 1, after verification pass, the gNB-CU shall send the UE CONTEXT MODIFICATION REQUEST message to the gNB-DU to indicate the successful verification of UE, then gNB-DU sends the buffered UL SDT data to gNB-CU-UP, and for option 2, it is simpler thatn option 1, i.e., there is NO stage 3 sepcification impact, only some clarification is needed in stage 2 specification.

Observation 1: Behind the above two options, the essential issue is that before the gNB-CU-CP verifies UE successful, whether the UL SDT data is allowed to send to gNB-CU-UP or has to be buffered at gNB-DU.

Proposal 1: If gNB-DU has to buffer UL SDT packet before UE verification pass, option 1 shall be selected.

Proposal 2: If gNB-DU is allowed to transmit UL SDT packet to gNB-CU before UE verification pass, option 2 shall be selected.

  - A new compromised option

If the above issue cannot be achievied consensus, we suggest to provide a compromised option 3 based on option1 and option2.

Option 3: The gNB-CU-CP indicates the gNB-DU whether the UL SDT packet shall be buffered or not.

1)	A new IE (e.g., “CG-SDT buffered indicattion”) as an optional IE, is added in the F1AP: UE CONTEXT RELEASE COMMAND message from gNB-CU to gNB-DU, at step 6.

2)	Another new IE (e.g., “CG-SDT transferred indicator”) as an optional IE, is added in the F1AP: UE CONTEXT MODIFICATION REQUEST message from gNB-CU to gNB-DU.

When the IE (e.g., “CG-SDT buffered indicattion”) is absent, it is same as option 2.

When the IE (e.g., “CG-SDT buffered indicattion”) is present, it is same as option 1.

Propsosal 3: If no consensus on whether gNB-DU shall buffer the SDT packet before UE verification pass, the gNB-CU-CP indicates the gNB-DU whether the UL SDT packet shall be buffered or not.

Proposal 3.1: In detail, a new option IE (e.g., “CG-SDT buffered indicattion”) is added in the F1AP: UE CONTEXT RELEASE COMMAND message. If gNB-CU-CP indicates gNB-DU to buffer UL SDT packet, this IE is present, otherwise, this IE is absent.

The corresponding CR is R3-223110 “CR for how to buffer the CG-SDT data to 38.473”.

- Conclusion

After the above analysis, we provide the following observations and proposals

Observation 1: Behind the above two options, the essential issue is that before the gNB-CU-CP verifies UE successful, whether the UL SDT data is allowed to send to gNB-CU-UP or has to be buffered at gNB-DU.

Proposal 1: If gNB-DU has to buffer UL SDT packet before UE verification pass, option 1 shall be selected.

Proposal 2: If gNB-DU can transmit UL SDT packet to gNB-CU before UE verification pass, option 2 shall be selected.

Propsosal 3: If no consensus on whether gNB-DU shall buffer the SDT packet before UE verification pass, the gNB-CU-CP indicates the gNB-DU whether the UL SDT packet shall be buffered or not.

Proposal 3.1: In detail, a new option IE (e.g., “CG-SDT buffered indicattion”) is added in the F1AP: UE CONTEXT RELEASE COMMAND message. If gNB-CU-CP indicates gNB-DU to buffer UL SDT packet, this IE is present, otherwise, this IE is absent.