3GPP TSG-RAN WG3 Meeting #116-e	R3-223131

# E-meeting, 9 – 19 May, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0872 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on NR QMC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE-Core |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Align RAN visible QoE information over F1 with information received from the UE. Enable the DU to group successively received RAN visible QoE reports per application session or QoE measurement configuration (QoE Reference). Missing encoding rule for PLMN information in QoE reference. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The RRC octet string conveyed over F1 contains the RAN-VisibleMeasurements-r17 IE. Renamed IE. QoE Reference added to the QOE INFORMATION TRANSFER message. Add encoding rule for PLMN information in QoE reference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN visible QoE information over F1 is not aligned with information received from the UE. The DU can't group RAN visible QoE measurements received successively from the same application session. Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.16.1, 9.3.1.260, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 9.2.16.1	QOE INFORMATION TRANSFER

This message is sent by a gNB-CU to a gNB-DU, to indicate information related to RAN visible QoE.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| QoE Information List |  |  | 0..1 |  | YES | ignore |
| >QoE Information Item |  |  | 1..<maxnoofQoEInformation> |  | Each | ignore |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | As defined in clause 5.2 of TS 28.405 [45]. It consists of MCC+MNC+QMC ID, where MCC+MNC are encoded as in 9.3.1.14 and are coming with the trace activation request from the management system to identify one PLMN containing the management system, and QMC ID is a 3-bytes octet string. |  |  |
| >>QoE Measurements | O |  | 9.3.1.260 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoEInformation | Maximum no. of QoE information for one UE, the maximum value is 16. |

#### 9.3.1.260	QoE Measurements

This IE provides the RAN visible QoE measurements to gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RAN Visible QoE Measurements | O |  | OCTET STRING | Contains the RAN-VisibleMeasurements-r17 IE defined in TS 38.331 [8]. |

### 9.4.5	Information Element Definitions

<<< skip unchanged ASN.1 >>>

BroadcastPNI-NPN-ID-List-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BufferSizeThresh ::= INTEGER(0..16777215)

<<< skip unchanged ASN.1 >>>

PLMN-Identity ::= OCTET STRING (SIZE(3))

PortNumber ::= BIT STRING (SIZE (16))

<<< skip unchanged ASN.1 >>>

QoEInformationList ::= SEQUENCE (SIZE(1.. maxnoofQoEInformation)) OF QoEInformationList-Item

QoEInformationList-Item ::= SEQUENCE {

qoEReference		QoEReference,

qoEMeasurements			QoEMeasurements  OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { QoEInformationList-ItemExtIEs} } OPTIONAL

}

QoEInformationList-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

QoEMeasurements ::= SEQUENCE {

RANVisisbleQoEMeasurements				OCTET STRING  OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { QoEMeasurementsExtIEs} } OPTIONAL,

...

}

QoEMeasurementsExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

QoEReference ::= OCTET STRING (SIZE(6))

QoS-Characteristics ::= CHOICE {

non-Dynamic-5QI				NonDynamic5QIDescriptor,

dynamic-5QI					Dynamic5QIDescriptor,

choice-extension			ProtocolIE-SingleContainer { { QoS-Characteristics-ExtIEs } }

}

QoS-Characteristics-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}