3GPP TSG-RAN WG3 Meeting #116-e	R3-223137

# E-meeting, 9th May-19th May- 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0205 | rev | 0 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (CR for 38.401) Correction on SON features enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-Core |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In current specification of TS 38.401, there is no stage 2 description on how the SHR Report is signalled from gNB-CU to gNB-DU, which shoud be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the signalling of SHR report information from gNB-CU to gNB-DU in section 8.14.1.2. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). The impact can be considered isolated because the change only conerns the SON functions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 2 description of SHR report in MRO procedure in CU-DU split architecture is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

//////////////////////////////////////////////////////////////////Start of the change//////////////////////////////////////////////////////////////////////

8.14	Self-optimisation

8.14.1	Overall procedures for MRO

The following clauses describe the overall procedures for MRO involving F1.

8.14.1.1	Signalling of RLF information from gNB-CU to gNB-DU

The signalling flow for signalling of RLF information from gNB-CU to gNB-DU is shown in Figure 8.14.1.1-1, where the example where NG-RAN nodes exchange the RLF Report via the Xn: FAILURE INDICATION message has been considered.

Figure 8.14.1.1-1	Example of signalling of RLF information from gNB-CU to gNB-DU in NG RAN

1.	A UE with a logged RLF Report connects to a cell in gNB2 and it signals the RLF Report to gNB2 by means of the RRC UE Information Request/Response procedures.

2. 	The gNB2 sends an Xn: Failure Indication to gNB1-CU where the UE may have previously been connected prior to the connection failure. This includes also the RLF Report.

3.	The gNB1-CU sends the F1: Access and Mobility Indication message to the gNB1-DU, including the RLF Report.

It is also possible for the gNB-CU receiving the RLF Report from the UE to signal it directly to the gNB-DU by means of the F1: Access and Mobility Indication procedure.

8.14.1.2	Signalling of SHR information from gNB-CU to gNB-DU

The signalling flow for signalling of SHR information from gNB-CU to gNB-DU involving F1 is shown in Figure 8.14.1.2-1.

Figure 8.14.1.2-1	Example of signalling of SHR information from gNB-CU to gNB-DU in NG RAN

1.	A UE with a SHR configuration and connects to a cell in gNB2 after a successful handover, it signals the SHR Report to gNB2 by means of the RRC UE Information Request/Response procedures.

2. 	The gNB2 sends an Xn: Access and Mobility Indication message to gNB1-CU where the UE may have previously been connected prior to the handover. This includes also the SHR Report.

3.	The gNB1-CU sends the F1: Access and Mobility Indication message to the gNB1-DU, including the SHR Report. The gNB-DU may take it into account for optimisation of mobility parameters.

///////////////////////////////////////////////////////////////////End of the change /////////////////////////////////////////////////////////////////////