3GPP TSG-RAN WG3 Meeting #116-e	R3-223144

# E-meeting, 09 May – 19 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | DraftCR | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for Support of SDT procedure over RACH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, China Telecom, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Core |  |  |  |  |  | Date: |  |  | 2022-4-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Issue 1 : The last sentence in the second paragraph in 18.1 is needs further clarification to include the name of the procedure refered to in the sentance. - Issue 1 description: The sentence currently mentions “During the SDT session, the receiving gNB may also request to terminate the SDT session to the last serving gNB.” without the procedure name - proposal: Add the procedure name at the end of the sentence as follows. During the SDT session, the receiving gNB may also request to terminate the SDT session to the last serving gNB via the UE Context Retrieve Confirmation procedure. Issue 2 : The sentence in the third paragraph in 18.1 is needs further clarification to exclude RRCRelease message as which is also a SRB PDCP PDU but is not transferred using XnAP RRC Transfer procedure. - Issue 2 description: The sentence currently does not exclude RRCRelease message as which is also a SRB PDCP PDU but is not transferred using XnAP RRC Transfer procedure - proposal: Add the suggested update as follows to refer to NAS messages which are transferred using XnAP RRC Transfer procedure . If the last serving gNB decides not to relocate the UE context, in case SDT is used for signalling, the corresponding SRB PDCP PDUs isare transferred between the receiving gNB and the last serving gNB via the XnAP RRC Transfer procedure. Issue 3 : The sentence in the forth and fifth paragraph in 18.1 describes DL-non SDT data arrival handling only. - Issue 3 description: The sentence in the forth and fifth paragraph in 18.1 currently describes DL-non SDT data arrival handling only. However similar handling is applicable to UL-non SDT data arrival indication from the UE which is currently missing and needs to be added - proposal: Handling for UL-non SDT data arrival indication from the UE which added in forth and fifth paragraph in 18.1 . Issue 4 : Step 3 in 18.2 need calarification that it is the receiving gNB that delivers UL SDT data to UPF. - Issue 4 description: Step 3 in 18.2 is currently unclear as which node delivers the UL SDT data to UPF and needs clarification. - proposal: Step 3 is updated as follows The last serving gNB decides to relocate UE context for SDT and responds with the RETRIEVE UE CONTEXT RESPONSE message. The UL SDT data, if any, is delivered from the receiving gNB to the UPF Issue 5 : Step 4-6 in 18.2 need calarification that it is the receiving gNB that delivers UL NAS PDU to AMF. - Issue 5 description: Step 4-6 in 18.2 is currently unclear as which node delivers the UL NAS PDU to AMF and needs clarification. - proposal: Step 4-6 is updated as follows The receiving gNB decides to keep UE in RRC Inactive state for SDT. If loss of DL user data buffered in the last serving gNB shall be prevented, the receiving gNB provides forwarding addresses. The receiving gNB alsoinitiates NG-AP Path Switch procedure to establish a NG UE associated signalling connection to the serving AMF. After path switch, the UL NAS PDU, if any, is delivered from the receiving gNB to AMF Issue 6 : Note after Step 7 in 18.2 needs to be removed. - Issue 6 description: Note after Step 7 in 18.2 is already covered in fourth paragraph of 18.1 and need not be repeated - proposal: Note after Step 7 is removed Issue 7 : Step 3 need clarification as it contradicts with step 4 description. - Issue 7 description: The wording “…not to relocate the UE Context for SDT” is incorrect as it contradicts with the wording that the partial UE context is relocated in step 4 - proposal: Change the wording in step 3 as follows The last serving gNB decides to keep the UE context not to relocate the UE context for SDT. Issue 8 : Step 5 in 18.3 need calarification that it is the last serving gNB that delivers UL SDT to UPF and UL NAS PDU to AMF. - Issue 8 description: Step 5 in 18.3 is currently unclear as which node delivers the UL SDT to UPF and UL NAS PDU to AMF and needs clarification. - proposal: Step 5 is updated as follows The receiving gNB acknowledges receiving the partial UE context and provides associated DL TNL address, if needed. After the Partial UE Context Retrieval procedure, the UE context is kept at the last serving gNB and the SDT related RLC context is established at the receiving gNB, then the UL SDT data is delivered from the last serving gNB to the UPF, the UL NAS PDU is delivered from the last serving gNB to AMF, if any. Issue 9 : Note1 after Step 5 in 18.3 is for DL Signalling. Similar note is needed for UL NAS Signalling. - Issue 9 description: Note 2 after Step 5 in 18.3 is added for UL NAS Signalling - proposal: Note 2 after Step 5 in 18.3 is added for UL NAS Signalling as shown below NOTE 2: The UL NAS PDU from the UE, if any, is forwarded to the last serving gNB via the RRC TRANSFER message, for which the last serving gNB delivers to the AMF Issue 10 : Note 3 after Step 6 in 18.3 needs to be removed. - Issue 10 description: Note after Step 6 in 18.3 is already covered in last paragraph of 18.1 and need not be repeated - proposal: Note 3 after Step 6 is removed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections as indicated in the proposals above and shown in the Section 18 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 Specification remains unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18.1, 18.2, 18.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------Begin Change-------------------------------------------

# 18	Small Data Transmission

## 18.0	General

Small Data Transmission (SDT) is a procedure allowing data and/or signalling transmission while remaining in RRC_INACTIVE state (i.e. without transitioning to RRC_CONNECTED state). SDT is enabled on a radio bearer basis and is initiated by the UE only if less than a configured amount of UL data awaits transmission across all radio bearers for which SDT is enabled, the DL RSRP is above a configured threshold, and a valid SDT resource is available as specified in clause YYY of TS 38.321 [6].

Editor’s Note: MAC TS clause references to be added once the section numbers are available.

SDT procedure is initiated with either a transmission over RACH (configured via system information) or over Type 1 CG resources (configured via dedicated signaling in RRCRelease). The SDT resources can be configured on initial BWP for both RACH and CG. RACH and CG resources for SDT can be configured on either or both of NUL and SUL carriers. The CG resources for SDT are valid only within the cell the UE received RRCRelease and transitioned to RRC_INACTIVE state. For RACH, the network can configure 2-step and/or 4-step RA resources for SDT. When both 2-step and 4-step RA resources for SDT are configured, the UE selects the RA type according to subclause 9.2.6. CFRA is not supported for SDT over RACH.

Once initiated, the SDT procedure is either:

-	successfully completed after the UE is directed to RRC_IDLE (via RRCRelease) or RRC_INACTIVE (via RRCRelease or RRCReject) or to RRC_CONNECTED (via RRCResume or RRCSetup); or

-	unsuccessfully completed upon cell re-selection, expiry of the SDT failure detection timer, a MAC entity reaching a configured maximum PRACH preamble transmission threshold, an RLC entity reaching a configured maximum retransmission threshold, or expiry of SDT-specific timing alignment timer while SDT procedure is ongoing over CG and the UE has not received a response from the network after the initial PUSCH transmission.

Upon unsuccessful completion of the SDT procedure, the UE transitions to RRC_IDLE.

The initial PUSCH transmission during the SDT procedure includes at least the CCCH message. When using CG resources for initial SDT transmission, the UE can perform autonomous retransmission of the initial transmission if the UE does not receive confirmation from the network (dynamic UL grant or DL assignment) before a configured timer expires as specified in clause ZZZ of TS 38.321 [6]. After the initial PUSCH transmission, subsequent transmissions are handled differently depending on the type of resource used to initiate the SDT procedure:

-	When using CG resources, the network can schedule subsequent UL transmissions using dynamic grants or they can take place on the following CG resource occasions. The DL transmissions are scheduled using dynamic assignments. The UE can initiate subsequent UL transmission only after reception of confirmation (dynamic UL grant or DL assignment) for the initial PUSCH transmission from the network. For subsequent UL transmission, the UE cannot initiate re-transmission over a CG resource.

-	When using RACH resources, the network can schedule subsequent UL and DL transmissions using dynamic UL grants and DL assignments, respectively, after the completion of the RA procedure.

While the SDT procedure is ongoing, if data appears in a buffer of any radio bearer not enabled for SDT, the UE initiates a transmission of a non-SDT data arrival indication using UEAssistanceInformation message to the network and, if available, includes the resume cause.

SDT procedure over CG resources can only be initiated with valid UL timing alignment. The UL timing alignment is maintained by the UE based on a SDT-specific timing alignment timer configured by the network via dedicated signalling and, for initial CG-SDT transmission, also by DL RSRP of configured number of highest ranked SSBs which are above a configured RSRP threshold. Upon expiry of the SDT-specific timing alignment timer, the CG resources are released while maintaining the CG resource configuration.

Logical channel restrictions configured by the network while in RRC_CONNECTED state and/or in RRCRelease message for radio bearers enabled for SDT, if any, are applied by the UE during SDT procedure.

The network may configure UE to apply ROHC continuity for SDT either when the UE initiates SDT in the cell where the UE received RRCRelease and transitioned to RRC_INACTIVE state or when the UE initiates SDT in a cell of its RNA.

## 18.1	Support of SDT procedure over RACH

For SDT procedure over RACH, if the UE accesses a gNB other than the last serving gNB, the UL SDT data/signalling is buffered at the receiving gNB, and then the receiving gNB triggers the XnAP Retrieve UE Context procedure. The receiving gNB indicates SDT to the last serving gNB and the last serving gNB decides whether to relocate the UE context or not. Other SDT assistance information (i.e., single packet, multiple packets) may also be provided by the receiving gNB to help the decision.

If the last serving gNB decides not to relocate the UE context, it transfers at least a partial UE context containing SDT RLC context information necessary for the receiving gNB to handle SDT. Then, UL/DL tunnels are established for DRBs configured for SDT between the receiving gNB and the last serving gNB and PDCP PDU of UL/DL data are transferred over the tunnels, until the last serving gNB decides to terminate the SDT session and move the UE back to RRC_INACTIVE by sending RRCRelease message. During the SDT session, the receiving gNB may also request to terminate the SDT session to the last serving gNB via the UE Context Retrieve Confirmation procedure.

If the last serving gNB decides not to relocate the UE context, in case SDT is used for signalling, the corresponding SRB PDCP PDUs are transferred between the receiving gNB and the last serving gNB via the XnAP RRC Transfer procedure.

When the receiving gNB receives DL data or DL signalling over radio bearer(s) not enabled for SDT from the core network or receives UE assistance information (i.e. UL non-SDT data arrival indication) from the UE and the UE context has been relocated to the receiving gNB, the receiving gNB may send the UE to RRC_CONNECTED state by directly sending the RRC Resume message.

When the last serving gNB receives DL data or DL signalling over radio bearer(s) not enabled for SDT from the core network and it has decided to not relocate the UE context, it may send the UE back to RRC_INACTIVE by sending the RRCRelease message.

## 18.2	SDT with UE context relocation

The overall procedure for SDT procedure over RACH with UE context relocation is illustrated in the figure 18.2-1.

Figure 18.2-1. RA-based SDT with UE context relocation

1.	The UE sends an RRCResumeRequest as well as UL SDT data and/or UL SDT signalling to the receiving gNB.

2.	The receiving gNB identifies the last serving gNB using the I-RNTI and retrieves the UE context by means of Xn-AP Retrieve UE Context procedure. The receiving gNB indicates that the UE request is for an SDT transaction and may also provide SDT assistance information (i.e., single packet, multiple packets).

3.	The last serving gNB decides to relocate UE context and responds with the RETRIEVE UE CONTEXT RESPONSE message. The UL SDT data, if any, is delivered from the receiving gNB to the UPF.

4-6.	The receiving gNB decides to keep UE in RRC Inactive state for SDT. If loss of DL user data buffered in the last serving gNB shall be prevented, the receiving gNB provides forwarding addresses. The receiving gNB alsoinitiates NG-AP Path Switch procedure to establish a NG UE associated signalling connection to the serving AMF. After path switch, the UL NAS PDU, if any, is delivered from the receiving gNB to AMF.

7.	After the SDT transmission is completed, the receiving gNB generates and sends RRCRelease message including the Suspend indication to the UE to send the UE back to RRC_INACTIVE.

8.	The receiving gNB indicates the last serving gNB to remove the UE context by sending the XnAP UE CONTEXT RELEASE message. The XnAP UE CONTEXT RELEASE message can be sent after step 6.

## 18.3	SDT without UE context relocation

The overall procedure for SDT procedure over RACH without UE context relocation is illustrated in the figure 18.3-1.

Figure 18.3-1. RA-based SDT without UE context relocation

1/2.	The steps 1/2 are as defined in steps 1/2 in Figure 18.2-1.

3.	The last serving gNB decides to keep the UE context.

4.	The last serving gNB transfers a partial UE context including the SDT related RLC context.

5.	The receiving gNB acknowledges receiving the partial UE context and provides associated DL TNL address, if needed. After the Partial UE Context Retrieval procedure, the UE context is kept at the last serving gNB and the SDT related RLC context is established at the receiving gNB, then the UL SDT data is delivered from the last serving gNB to the UPF, the UL NAS PDU is delivered from the last serving gNB to AMF, if any.

NOTE 1:	The DL signalling from the last serving gNB, if any, is forwarded to the receiving gNB via the RRC TRANSFER message, for which the receiving gNB delivers to the UE.

NOTE 2: The UL NAS PDU from the UE, if any, is forwarded to the last serving gNB via the RRC TRANSFER message, for which the last serving gNB delivers to the AMF

6.	After SDT transmission is completed, the last serving gNB responds to the receiving gNB with the RETRIEVE UE CONTEXT FAILURE message including an encapsulated RRCRelease message. The RRCRelease message includes suspend configuration.

NOTE 3:	The receiving gNB may send the RETRIEVE UE CONTEXT CONFIRM message to request the termination of SDT session before step 6.

.

7.	The receiving gNB sends the RRCRelease message to the UE.

8.	The UE moves to RRC_INACTIVE mode.

-------------------------------------------End Change-------------------------------------------