3GPP TSG-RAN WG3#116                                             	                     R3-223157

E-Meeting, 9 – 19 May 2022

Agenda item:	9.3.5

Source:	Nokia, Nokia Shanghai Bell, Orange, Verizon Wireless

Title: 	Correction of NAS PDU Delivery

Document for:	Approval

# Introduction

At the last RAN3#114 meeting the case of NAS PDU delivery for the PDU Session Modify was discussed without final conclusion:

RAN3#114e:

Choose one of the following text proposals for PDU Session modification corrections in the next meeting:

Opt1: If the NAS-PDU IE is received for the PDU session, the NG-RAN node shall pass it to the UE.

Opt2: The NG-RAN node shall pass the NAS-PDU IE received for the PDU session to the UE when modifying the PDU session configuration. The NG-RAN node does not send the NAS PDUs associated to the failed PDU sessions to the UE.

# When the NG-RAN receives a PDU Session Resource Modify Request message which only includes the PDU Session AMBR IE, NG-RAN shall send NAS-PDU to UE when the PDU Session AMBR IE is modified successfully?

In the meantime, SA2 has been consulted by RAN3 and the reply was received at RAN3#115 in [2] but too late and postponed to this meeting:

| - R3-222505 | - Response LS on NAS PDU delivery during PDU Session modification procedure (SA2) | - LS in Postpone to next meeting E///, Nok, HW, ZTE: Prefer to discuss this topic in the next meeting based on the LS received. CATT: Fine to discuss this in next meeting. |
| --- | --- | --- |

# This paper continues the discussion and concludes.

# Discussion

It was clearly shown at RAN3#114 that the NAS PDU contained in the PDU Session Modify request transfer IE can contain three types of modifications (see the Annex B of this document and section 8.3.9 of TS 24.501):

- NAS Modification containing parameters transparent to the gNB i.e. modification of RQI timer (type 1).

- NAS Modification containing parameters linked to a modification requested to gNB in the PDU Session Modify request transfer IE e.g. addition/release of QoS Flow (type 2).

- NAS Modification containing both parameters transparent to the gNB and parameters linked to a modification requested to gNB in the PDU Session Modify request transfer IE (type 3).

The current procedural text is not satisfactory because in case of a request to change the PDU Session AMBR which does not include radio bearer reconfiguration, the change can be successful, and the NAS PDU not delivered.

Two main solutions have been discussed so far:

- Solution 1: send the NAS PDU always regardless of the PDU session modification success.

- Solution 2: send the NAS PDU when the PDU Session modification is successful or partial success (gNB returns PDU Session Modify Response), not send the NAS PDU if the PDU session modification is not successful.

In fact, what matters is that gNB and SMF remain in sync. Therefore, both solutions work provided that the rule is clear. The drawback of each solution is analysed in detail as follows, depending on the type of modification:

Solution 1:

For type 1, there no request to gNB (gNB always successful). So gNB and SMF will always be in sync.

For type 2, every time the modification is partial success or no success in the gNB, because the NAS PDU still has been sent, the SMF will need to send another NAS PDU session modification command to correct the UE.

For type 3, every time the modification is partial success or no success in the gNB, because the NAS PDU still has been sent, the SMF will need to send another NAS PDU session modification command to correct the UE on the part concerning the gNB modification.

Solution 2:

For type 1, there no request to gNB (gNB always successful). So gNB and SMF will always be in sync.

For type 2, because the sending of the NAS PDU by gNB is correlated with the success in gNB, UE and SMF remain in sync when the modification is no success: no need for SMF to resend the NAS message. When partial success, SMF needs to resend another NAS PDU session modification command to correct the UE.

For type 3, for partial success, the SMF anyway needs to resend the NAS message. For no success, the SMF still needs to send to UE the parameter transparent to gNB. However it is not in desync with the UE.

The comparison can be summarized in the following table:

|  | Solution 1 | Solution 2 |
| --- | --- | --- |
| Type 1 | SMF doesn’t retransmit | SMF doesn’t retransmit |
| Type 2 partial success | SMF retransmit | SMF retransmit |
| Type 2 no success | SMF retransmit | SMF doesn’t retransmit |
| Type 3 partial success | SMF retransmit | SMF retransmit |
| Type 3 no success | SMF retransmit | SMF retransmit (the transparent part) |

As can be seen from the summary table, both solutions work. Solution 2 allows to optimize the case “Type 2 no success” which is a case with total failure. It is probably not the most frequent case but is still an advantage.

UE triggered PDU Session Modification: Result from consulting SA2 (see [2])

Solution 2 presents another advantage in case of UE triggered PDU session modification. When the modification requested by the UE fails in gNB, if the NAS PDU is not sent to the UE, SA2 clarified in [2] that this enables the SMF to reply with NAS PDU Modification Reject to the UE.

- Regarding Option 2, understanding 2 is correct:

  - If RAN replies with PDU SESSION RESOURCE MODIFY RESPONSE containing “PDU Session Resource Modify Response Item” for the PDU Session, then SMF assumes that NAS PDU was sent to UE, even if some QoS Flow modifications failed.

  - If RAN replies with PDU SESSION RESOURCE MODIFY RESPONSE containing “PDU Session Resource Failed to Modify Response Item” for the PDU Session, then SMF assumes that NAS PDU was NOT sent to UE. In this case the SMF could send a NAS Reject to the UE.

In contrast, with solution 1, the SMF would have to first reply with “NAS Accept” to the UE (despite the modification was unsuccessful in gNB) and send again another NAS PDU Modification to revert the UE. Solution 1 is more cumbersome.

Proposal 1: agree solution 2.

# Conclusion and Proposal

The handling of the NAS PDU in the PDU Session Modify Request message has been investigated through all possible cases with the conclusion that both solutions work and a small advantage for solution 2.

Proposal 1: agree solution 2, CR in [3] and LS back in the annex A below.

# References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO, Inc.

- R3-222505, Response LS on NAS PDU Delivery during PDU Session Modification procedure.

- R3-223158, Correction of NAS PDU Delivery during PDU Session Modification

# Annex A

3GPP TSG-RAN WG3#116			R3-22xxxx

E-Meeting, 9 – 19 May 2022

Title:	Reply LS on NAS PDU delivery during PDU Session modification procedure

Response to:	Response LS on NAS PDU delivery during PDU Session modification procedure (R3-222505, S2-2201676)

Release:	Rel-16

Work Item:	TEI16

Source:	RAN3

To:	SA2

Cc:

Contact Person:

#### Name:	Philippe Godin

Tel. Number:	+33 1 6040 0304

E-mail Address:	philippe.godin@nokia.com

Attachments:

1. Overall Description:

RAN3 thanks SA2 for their guidance on NAS PDU delivery during PDU Session modification procedure and would like to provide the following feedback.

RAN3 has agreed the solution where the gNB only passes the NAS PDU to the UE when the modification of the PDU session is at least partially successful. The agreed CR is attached.

2. Actions:

To SA2 group:

ACTION: 	SA2 to take the above feedback into account.

3. Date of Next TSG-RAN WG3 Meetings:

TSG-RAN WG3#117                       15 – 19 August 2022			Toulouse, France

# Annex B: content of the NAS PDU Session Modification Command

### 8.3.9 PDU session modification command #### 8.3.9.1 Message definition The PDU SESSION MODIFICATION COMMAND message is sent by the SMF to the UE to indicate a modification of a PDU session. See table 8.3.9.1.1 Message type: PDU SESSION MODIFICATION COMMAND Significance: dual Direction: network to UE Table 8.3.9.1.1: PDU SESSION MODIFICATION COMMAND message content IEI; Information Element; Type/Reference; Presence; Format; Length. Extended protocol discriminator; Extended protocol discriminator 9.2; M; V; 1. PDU session ID; PDU session identity 9.4; M; V; 1. PTI; Procedure transaction identity 9.6; M; V; 1. PDU SESSION MODIFICATION COMMAND message identity; Message type 9.7; M; V; 1. 59; 5GSM cause; 5GSM cause 9.11.4.2; O; TV; 2. 2A; Session AMBR; Session-AMBR 9.11.4.14; O; TLV; 8. 56; RQ timer value; GPRS timer 9.11.2.3; O; TV; 2. 8-; Always-on PDU session indication; Always-on PDU session indication 9.11.4.3; O; TV; 1. 7A; Authorized QoS rules; QoS rules 9.11.4.13; O; TLV-E; 7-65538. 75; Mapped EPS bearer contexts; Mapped EPS bearer contexts 9.11.4.8; O; TLV-E; 7-65538. 79; Authorized QoS flow descriptions; QoS flow descriptions 9.11.4.12; O; TLV-E; 6-65538. 7B; Extended protocol configuration options; Extended protocol configuration options 9.11.4.6; O; TLV-E; 4-65538. 77; ATSSS container; ATSSS container 9.11.4.22; O; TLV-E; 3-65538. 66; IP header compression configuration; IP header compression configuration 9.11.4.24; O; TLV; 5-257. 74; Port management information container; Port management information container 9.11.4.27; O; TLV-E; 4-65538. 1E; Serving PLMN rate control; Serving PLMN rate control 9.11.4.20; O; TLV; 4. 1F; Ethernet header compression configuration; Ethernet header compression configuration 9.11.4.28; O; TLV; 3.