3GPP TSG-RAN WG3 Meeting #116-e	R3-223168

# Online,  9th – 19th May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR |  | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SDT corrections in stage-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clean-up of stage-2 for SDT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add description in section 18 to differentiate full/partial UE context retrieval. - Replace “i.e.,” for SDT assistance info to “e.g.,” for future proof. - Change the place of UL small data transferring in the signaling flow. - Rename SDT transaction. - Other clean-up. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity remains in stage-2 for SDT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18.1, 18.2, 18.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 18.1	Support of SDT procedure over RACH

For SDT procedure over RACH, if the UE accesses a gNB other than the last serving gNB, the UL SDT data/signalling is buffered at the receiving gNB, and then the receiving gNB triggers the XnAP Retrieve UE Context procedure. The receiving gNB indicates SDT to the last serving gNB and the last serving gNB decides whether to relocate the UE context or not. Other SDT assistance information (e.g.., single packet, multiple packets) may also be provided by the receiving gNB to help the decision of UE context relocation.

If the last serving gNB decides not to relocate the full UE context, it transfers at least partial UE context containing SDT RLC context information necessary for the receiving gNB to handle SDT via the Partial UE Context Transfer procedure. Then, UL/DL tunnels are established for DRBs configured for SDT between the receiving gNB and the last serving gNB. The PDCP PDU of UL/DL data are transferred over the tunnels, until the last serving gNB decides to terminate the SDT session and move the UE back to RRC_INACTIVE by sending the RRCRelease message. During the SDT session, the receiving gNB may also request via the Retrieve UE Context Confirm procedure to terminate the SDT session to the last serving gNB.

If the last serving gNB decides not to relocate the full UE context, in case SDT is used for signalling, SRB PDCP PDUs are transferred between the receiving gNB and the last serving gNB via the XnAP RRC Transfer procedure.

When the receiving gNB receives DL data or DL signalling over radio bearer(s) not enabled for SDT from the core network and the UE context has been relocated to the receiving gNB, the receiving gNB may send the UE to RRC_CONNECTED state by directly sending the RRCResume message.

When the last serving gNB receives DL data or DL signalling over radio bearer(s) not enabled for SDT from the core network and it has decided to not relocate the UE context, it may send the UE back to RRC_INACTIVE by sending the RRCRelease message.

## 18.2	SDT with UE context relocation

The overall procedure for SDT procedure over RACH with UE context relocation is illustrated in the figure 18.2-1.

Figure 18.2-1. RA-based SDT with UE context relocation

1.	The UE sends an RRCResumeRequest as well as UL SDT data and/or UL SDT signalling to the receiving gNB.

2.	The receiving gNB identifies the last serving gNB using the I-RNTI and retrieves the UE context by means of Xn-AP Retrieve UE Context procedure. The receiving gNB indicates that the UE request is for an SDT session and may also provide SDT assistance information (e.g., single packet, multiple packets).

3.	The last serving gNB decides to relocate UE context for SDT and responds with the RETRIEVE UE CONTEXT RESPONSE message. The UL SDT data, if any, is delivered to the UPF.

4-6.	The receiving gNB decides to keep UE in RRC INACTIVE state for SDT. If loss of DL user data buffered in the last serving gNB shall be prevented, the receiving gNB provides forwarding addresses via the Xn-U ADDRESS INDICATION message. The receiving gNB also initiates NG-AP Path Switch procedure to establish a NG-AP UE associated signalling connection to the serving AMF. After the Path Switch procedure, the UL NAS PDU is delivered to AMF.

7.	After the SDT transmission is completed, the receiving gNB generates and sends the RRCRelease message including the Suspend Indication to the UE to send the UE back to RRC_INACTIVE state.

NOTE:	In case DL non-SDT data or DL non-SDT signalling arrives, the receiving gNB may decide to directly send the UE to RRC_CONNECTED state by sending the RRCResume message.

8.	The receiving gNB indicates to the last serving gNB to remove the UE context by sending the XnAP UE CONTEXT RELEASE message. The XnAP UE CONTEXT RELEASE message can be sent after step 6.

## 18.3	SDT without UE context relocation

The overall procedure for SDT procedure over RACH without UE context relocation is illustrated in the figure 18.3-1.

Figure 18.3-1. RA-based SDT without UE context relocation

1/2.	The steps 1/2 are as defined in steps 1/2 in Figure 18.2-1.

3.	The last serving gNB decides not to relocate the full UE context for SDT.

4.	The last serving gNB transfers a partial UE context including the SDT related RLC context.

5.	The receiving gNB acknowledges receiving the partial UE context and provides associated DL TNL address, if needed. The UE context is kept at the last serving gNB and the SDT related RLC context is established at the receiving gNB, then the UL SDT data is delivered to the UPF, the UL NAS PDU is delivered to AMF, if any.

NOTE 1:	The DL signalling from the last serving gNB, if any, is forwarded to the receiving gNB via the RRC TRANSFER message, for which the receiving gNB delivers it to the UE.

6.	After SDT transmission is completed, the last serving gNB responds to the receiving gNB with the RETRIEVE UE CONTEXT FAILURE message including an encapsulated RRCRelease message. The RRCRelease message includes suspend configuration.

NOTE 2:	The receiving gNB may send the RETRIEVE UE CONTEXT CONFIRM message to request the termination of SDT session before step 6.

NOTE 3:	In case DL non-SDT data or DL non-SDT signalling arrives, the last serving gNB moves the UE back to RRC_INACTIVE state by sending the RRCRelease message.

7.	The receiving gNB sends the RRCRelease message to the UE.

8.	The UE moves to RRC_INACTIVE state.