3GPP TSG-RAN WG3 #116-e	R3-223178

# Online, 9  – 19 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0786 | rev | - | Current version: | 15.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for handling racing condition of signalling – Handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC, NTT DOCOMO Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | During e.g. inter-system handover there may be a racing condition that the target gNB receives successfully RRC:ULInformationTransfer message before receives successfully RRC:RRCReconfigurationComplete message. The Target gNB should handle this race condition by suspend sending the NGAP:UPLINK NAS TRANSPORT message to AMF until it has sent NGAP: HANDOVER NOTIFY message to the AMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add interaction in Handover Notification procedure text for the target NG-RAN node to handle the racing condition i.e. the target NG-RAN node shall send UPLINK NAS TRANSPORT message only after it has sent HANDOVER NOTIFY message to AMF. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release), the isolated impact is on the interaction between the Handover Notification procedure and the Uplink NAS Transport procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No clear spcification then AMF may fail the handover. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Change >>>>>>>>>>>>>>>>>>>>

8.4.3	Handover Notification

8.4.3.1	General

The purpose of the Handover Notification procedure is to indicate to the AMF that the UE has arrived to the target cell and the NG-based handover has been successfully completed.

8.4.3.2	Successful Operation

Figure 8.4.3.2-1: Handover notification

The target NG-RAN node shall send the HANDOVER NOTIFY message to the AMF when the UE has been identified in the target cell and the NG-based handover has been successfully completed.

Interactions with Uplink NAS Transport procedure:

If the target NG-RAN node receives uplink NAS message from the UE, the target NG-RAN node shall send the UPLINK NAS TRANSPORT message only after it has sent HANDOVER NOTIFY message to the AMF.

8.4.3.3	Abnormal Conditions

Void.