3GPP TSG-RAN WG3 #116-e                                                                           R3-223188

Online, 9 May – 19 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0787 | rev | 0 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS Session Status indication in Xn interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduce a new MBS Session status indication in the MBS Session Information List of Handover Request message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add MBS Session Status indication in MBS Session Information List IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Target gNB is not aware whether the MBS session is inactive or active. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.36, 9.2.3.aaa |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev-0: |  |  |  |  |  |  |  |  |

-------------------------------------------------Start of the change--------------------------------------------------

#### 9.2.1.36	MBS Session Information List

This IE contains MBS session resource related information used at UE context transfer between NG-RAN nodes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session Information To Be Setup List |  | 1..<maxnoofMBSSessions> |  |  |
| >MBS Session ID | M |  | 9.2.3.146 |  |
| >MBS Area Session ID | O |  | 9.2.3.148 | MBS Area Session ID of the UE at the NG-RAN node from which the UE context is transferred |
| >MBS Service Area | O |  | 9.2.3.150 |  |
| >MBS Session Status | M |  | 9.2.3.aaa |  |
| >MBS QoS Flows to Add List |  | 1..<maxnoofMBSQoSFlows> |  |  |
| >>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.2.3.10 |  |
| >>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.2.3.5 |  |
| >MBS Mapping and Data Forwarding Request Info from source NG-RAN node | O |  | 9.2.1.39 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSSessions | Maximum no. of MBS Sessions. Value is 8192. |
| maxnoofMBSQoSFlows | Maximum no. of QoS flows allowed within one MBS session. Value is 64. |

--------------------------------------------------Next change-------------------------------------------------------

#### 9.2.3.aaa	MBS Session Status

This IE indicates whether multicast session is activation or deactivation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session Status | M |  | ENUMERATED (activation, deactivation, …) |  |

--------------------------------------------------Next change-------------------------------------------------------

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

………

SKIP UNCHANGED PART

MBS-SessionInformation-List ::= SEQUENCE (SIZE(1..maxnoofMBSSessions)) OF MBS-SessionInformation-Item

MBS-SessionInformation-Item ::= SEQUENCE {

mBS-Session-ID			MBS-Session-ID,

mBS-Area-Session-ID		MBS-Area-Session-ID								OPTIONAL,

mBS-ServiceArea			MBS-ServiceArea									OPTIONAL,

mBS-SessionStatus       MBS-SessionStatus,

mBS-QoSFlowsToAdd-List	MBS-QoSFlowsToAdd-List,

mBS-MappingandDataForwardingInfofromSource		MBS-MappingandDataForwardingInfofromSource	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { MBS-SessionInformation-Item-ExtIEs} }	OPTIONAL,

...

}

………

SKIP UNCHANGED PART

MBS-SessionInformationResponse-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MBSSessionStatus ::= ENUMERATED {

activated,

deactivated,

...

}

--------------------------------------------------End of the change-------------------------------------------------------