3GPP TSG-RAN WG3 Meeting #116-e	R3-223196

E-meeting, 09 May – 19 May 2022

Title: 	Introducing Report Amount for M4, M5, M6, M7 measurements for E-UTRAN

Source: 	Huawei, Deutsche Telekom, Orange

Agenda item:	9.2

Document Type:	Discussion

1. Introduction

This contribution proposes to introduce report amount for M4, M5, M6, M7 measurements for E-UTRAN.

2. Discussion

At RAN3 114 e-meeting, RAN3 agreed to introduce the Report Amount for MDT M4 ~ M7 Measurements in NG-RAN in Rel-17. Due to lack of time, there was a FFS left for E-UTRAN.

It is FFS whether the introduction of the Report Amount is beneficial also for E-UTRAN

This issue was moved to TEI17 in 114bis and 115 e-meetings, and was not treated either due to time limitation.

31.2.6. Others Including other left issues, e.g., CSI-RS configuration exchange, left issues on MDT Report Amount, new XnAP Cause, 2-step RRC resume, Secondary RAT Configuration/Co-ordination between MN and SN...

Since the intention is to introduction of the same report amount control to EUTRAN connecting to EPC as the one in NG-RAN, it is reasonable to discuss and agree it as a LTE correction.

In the previous LS from SA5 in [1], it stated the benefits to introduce the report amount:

SA5 confirms that reportAmount is beneficial to configure for MDT measurements M4, M5, M6 and M7.

In the OAM there is the possibility to delete an existing Trace Job (traceJob) in order to stop the measurement reporting. However, this will stop all measurements configured in this Trace Job (e.g. not only M4, but also M2, M3, M6, M7 as well as trace). Additionally, initializing the deletion of a Trace Job means additional effort and signalling. These issues can be avoided by configuring a number of measurement reports aka reportAmount per MDT measurement.

When the RAN chooses any allowed value for the reportAmount for MDT measurements M4, M5, M6 and M7, the OAM does not know how many reports to expect.

Those above mentioned benefits are applicable for both NG-RAN and E-UTRAN. Therefore, it is proposed:

Proposal: To introduce the Report Amount for M4, M5, M6, M7 measurements for E-UTRAN as a LTE correction in Rel-17.

3. Conclusion

In this contribution, we propose:

Proposal: To introduce the Report Amount for M4, M5, M6, M7 measurements for E-UTRAN as a LTE correction in Rel-17.

The corresponding CRs for S1AP and X2AP are provided in [2] and [3].

5. Reference

- S5-214523, Reply LS on Report Amount for M4, M5, M6, M7 measurements.

- R3-223197	Introducing Report Amount for M4, M5, M6, M7 measurements for E-UTRAN.

- R3-223198	Introducing Report Amount for M4, M5, M6, M7 measurements for E-UTRAN.