3GPP TSG-RAN WG3 Meeting #116-e	R3-223209

E-meeting, 09 May – 19 May 2022

Title: 	Measurement Amount on NRPPa

Source: 	Huawei

Agenda item:	9.3.9

Document Type:	Discussion

# 1. Introduction

The discussion on Measurement Amount on NRPPa started in RAN3#114, with a conclusion to continuing in TEI17. However due to time limitation, there was no opportunity to submit new RAN3 TEI-17 topics since RAN3#114, which makes sense due to priority on WI.

This contribution reminds the technical aspects and past discussion.

# 2. Discussion

In the RAN3#113 meeting, there is one paper also mentioned that in the LPP the periodical measurements have the following content:

PeriodicalReportingCriteria ::=		SEQUENCE {

reportingAmount						ENUMERATED {

ra1, ra2, ra4, ra8, ra16, ra32,

ra64, ra-Infinity

} DEFAULT ra-Infinity,

reportingInterval					ENUMERATED {

noPeriodicalReporting, ri0-25,

ri0-5, ri1, ri2, ri4, ri8, ri16, ri32, ri64

}

}

It can be seen that the LMF use the reportingInterval IE to request the UE to either conduct/report the measurements in one shot or periodically. Along with the periodical measurements, the LMF also include a reportingAmount IE, which has the following field description:

reportingAmount indicates the number of periodic location information reports requested. Enumerated values correspond to 1, 2, 4, 8, 16, 32, 64, or infinite/indefinite number of reports. If the reportingAmount is 'infinite/indefinite', the target device shou-ld continue periodic reporting until an LPP Abort message is received. The value 'ra1' shall not be used by a sender.

However, in the NRPPa Measurement Request message, only periodicity information is included and the amount information is missing. The LMF would always need to send a Measurement Abort message to stop the gNB measurements.

Observation: The amount information is missing when periodic reporting is required in NRPPa Measurement Request message. The LMF would always need to send a Measurement Abort message to stop the gNB measurements.

In the NRPPa Measurement Request message, the Measurement Periodicity IE is included if the ReportCharacteristics is valued “Periodic”. It is proposed to include a Measurement Amount IE along with the Measurement Periodicity IE is included if the ReportCharacteristics is valued “Periodic”. The value of the Measurement Amount IE can be aligned with LPP to be between 1 to 64 and infinity.

Proposal: To introduce a Measurement Amount IE in the NRPPa Measurement Request message when the ReportCharacteristics is set to "Periodic".

# 3. Conclusion

By this contribution we retrieve a remaining TEI-17 issue about the Measurement Amount on NRPPa, which was started in RAN3#114 and not completed.

We have the following observation and proposal:

Observation: The amount information is missing when periodic reporting is required in NRPPa Measurement Request message. The LMF would always need to send a Measurement Abort message to stop the gNB measurements.

Proposal: To introduce a Measurement Amount IE in the NRPPa Measurement Request message when the ReportCharacteristics is set to "Periodic".

The CR to NRPPa and F1AP are in [1] and [2].

# 4. Reference

- R3-223210	CR to 38.455 on Measurement Amount

- R3-223211	CR to 38.473 on Measurement Amount