3GPP TSG-RAN WG3 Meeting #116-e                                                                  R3-223226

E-meeting, May 9 – 19, 2022

Title:	Reply LS on PEI and UE Subgrouping

Response to:	R2-2204240

Release:	Release 17

Work Item:     	NR_UE_pow_sav_enh-Core

Source:         	RAN3

To:             	RAN2

CC:             	SA2

Contact person:

Name:	Liu Ziqiao

E-mail Address:	liuziqiao@catt.cn

- Overall Description

RAN3 would like to thank RAN2 for the LS on PEI and UE Subgrouping, and the following anwers were made:

Question 1: Whether the mismatched understanding about the ‘last used cell’ between UE and NW still exists in NR, if so, whether the LTE method (i.e. to introduce ‘no last cell update’ indication in RRCRelease Message) can be reused?

Answer 1: RAN3 has identified at least one case (i.e. MME overload for CP CIoT) that the mismatched understanding about the ‘last used cell’ between UE and NW will occur in LTE. However, there is no such case in NR. Hence the mismatched understanding about the ‘last used cell’ between UE and NW does not exist in NR.

Question 2: Whether this problematic scenario can be avoided or needs to be resolved through signaling?

Answer 2: RAN3 thinks this scenario is a corner case. The CN-assigned subgrouping is per RA which means that all gNBs in RA support CN-assigned subgrouping. This problematic scenario can be avoided through implementation.

- Actions

RAN3 respectfully asks RAN2 and SA2 to take the above information into account.

- Dates of next RAN3 meetings

3GPP TSG RAN3#117e		August 22 –August 26 2022