3GPP TSG-RAN WG3 Meeting # 116e	R3-223248

# E-meeting,  9th - 19th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to receiving gNB behaviour upon SDT completion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Core |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The content of partial UE context had been agreed; however, the “at least” a partial UE context may suggest that the last serving gNB can still transfer more than just partial UE context, while in the same sentence it has stated that the last serving gNB decides not to relocate the (full) UE context. - It is unclear how the receiving gNB handles the established SDT RLC entity and the UE context for the SDT without UE context relocation when the SDT transmission is completed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove “at least” and change RRC message style in section 18.1. - For SDT without UE context relocation, the receiving gNB releases the established SDT RLC entity and the partial UE context when the SDT transmission is completed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The receiving gNB behavior about the UE context remains ambigious upon SDT completion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18.1, 18.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------First Change -------------------------------------------

## 18.1	Support of SDT procedure over RACH

For SDT procedure over RACH, if the UE accesses a gNB other than the last serving gNB, the UL SDT data/signalling is buffered at the receiving gNB, and then the receiving gNB triggers the XnAP Retrieve UE Context procedure. The receiving gNB indicates SDT to the last serving gNB and the last serving gNB decides whether to relocate the UE context or not. Other SDT assistance information (i.e., single packet, multiple packets) may also be provided by the receiving gNB to help the decision.

If the last serving gNB decides not to relocate the UE context, it transfers a partial UE context containing SDT RLC context information necessary for the receiving gNB to handle SDT. Then, UL/DL tunnels are established for DRBs configured for SDT between the receiving gNB and the last serving gNB and PDCP PDU of UL/DL data are transferred over the tunnels, until the last serving gNB decides to terminate the SDT session and move the UE back to RRC_INACTIVE by sending RRCRelease message. During the SDT session, the receiving gNB may also request to terminate the SDT session to the last serving gNB.

If the last serving gNB decides not to relocate the UE context, in case SDT is used for signalling, SRB PDCP PDUs is transferred between the receiving gNB and the last serving gNB via the XnAP RRC Transfer procedure.

When the receiving gNB receives DL data or DL signalling over radio bearer(s) not enabled for SDT from the core network and the UE context has been relocated to the receiving gNB, the receiving gNB may send the UE to RRC_CONNECTED state by directly sending the RRCResume message.

When the last serving gNB receives DL data or DL signalling over radio bearer(s) not enabled for SDT from the core network and it has decided to not relocate the UE context, it may send the UE back to RRC_INACTIVE by sending the RRCRelease message.

-------------------------------------------Next Change -------------------------------------------

## 18.3	SDT without UE context relocation

The overall procedure for SDT procedure over RACH without UE context relocation is illustrated in the figure 18.3-1.

Figure 18.3-1. RA-based SDT without UE context relocation

1/2.	The steps 1/2 are as defined in steps 1/2 in Figure 18.2-1.

3.	The last serving gNB decides not to relocate the UE context for SDT.

4.	The last serving gNB transfers a partial UE context including the SDT related RLC context.

5.	The receiving gNB acknowledges receiving the partial UE context and provides associated DL TNL address, if needed. After the Partial UE Context Retrieval procedure, the UE context is kept at the last serving gNB and the SDT related RLC context is established at the receiving gNB, then the UL SDT data is delivered to the UPF, the UL NAS PDU is delivered to AMF, if any.

NOTE 1:	The DL signalling from the last serving gNB, if any, is forwarded to the receiving gNB via the RRC TRANSFER message, for which the receiving gNB delivers to the UE.

6.	After SDT transmission is completed, the last serving gNB responds to the receiving gNB with the RETRIEVE UE CONTEXT FAILURE message including an encapsulated RRCRelease message. The RRCRelease message includes suspend configuration. The receiving gNB shall release the established SDT RLC entity and the partial UE context.

NOTE 2:	The receiving gNB may send the RETRIEVE UE CONTEXT CONFIRM message to request the termination of SDT session before step 6.

NOTE 3:	In case DL non-SDT data or DL non-SDT signalling arrives, the last serving gNB moves the UE back to RRC_INACTIVE by sending RRCRelease message.

7.	The receiving gNB sends the RRCRelease message to the UE.

8.	The UE moves to RRC_INACTIVE mode.