3GPP TSG-RAN WG3 Meeting #116-e	R3-223280

# E-meeting, 9-19 May, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on RA-SDT overall procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE |  |  |  |  |  | Date: |  |  | 2022-04-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For SDT with UE context relocation case: - The UL NAS PDU is not always exist, thus “if any” should be added. For SDT without UE context relocation case: - the description of step 5 is not clear enough on how the data or signalling is handled between the receiving gNB and the last serving gNB, which may cause some consufion. - In RAN2 LS R3-223019, RAN2 agreed to use RAN paging procedure to trigger another RRC resume procedure for DL non-SDT transmission after UE is sent to Inactive via RRCRelease. Corresponding texts should be added in the stage 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Make refinement to the Figure 18.2-1 and the texts accordingly. - Make refinement to the texts in step 5 of the Figure 18.3-1. - Add texts in section 18.1 and 18.3 to align with the LS reply from RAN2, R3-223019. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The overall procedures for SDT with/without anchor relocation are not clear enough, which may cause some confusion or misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18.1, 18.2, 18.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Begin of change >>>>>>>>>>>>>>>>>>>>

## 18.1	Support of SDT procedure over RACH

For SDT procedure over RACH, if the UE accesses a gNB other than the last serving gNB, the UL SDT data/signalling is buffered at the receiving gNB, and then the receiving gNB triggers the XnAP Retrieve UE Context procedure. The receiving gNB indicates SDT to the last serving gNB and the last serving gNB decides whether to relocate the UE context or not. Other SDT assistance information (i.e., single packet, multiple packets) may also be provided by the receiving gNB to help the decision.

If the last serving gNB decides not to relocate the UE context, it transfers at least a partial UE context containing SDT RLC context information necessary for the receiving gNB to handle SDT. Then, UL/DL tunnels are established for DRBs configured for SDT between the receiving gNB and the last serving gNB and PDCP PDU of UL/DL data are transferred over the tunnels, until the last serving gNB decides to terminate the SDT session and move the UE back to RRC_INACTIVE by sending RRCRelease message. During the SDT session, the receiving gNB may also request to terminate the SDT session to the last serving gNB.

If the last serving gNB decides not to relocate the UE context, in case SDT is used for signalling, SRB PDCP PDUs is transferred between the receiving gNB and the last serving gNB via the XnAP RRC Transfer procedure.

When the receiving gNB receives DL data or DL signalling over radio bearer(s) not enabled for SDT from the core network and the UE context has been relocated to the receiving gNB, the receiving gNB may send the UE to RRC_CONNECTED state by directly sending the RRC Resume message.

When the last serving gNB receives DL data or DL signalling over radio bearer(s) not enabled for SDT from the core network and it has decided to not relocate the UE context, it may send the UE back to RRC_INACTIVE by sending the RRCRelease message, and then the last serving gNB uses RAN paging to trigger the following-up RRC resume procedure for the non-SDT data/signalling transmission.

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

18.2	SDT with UE context relocation

The overall procedure for SDT procedure over RACH with UE context relocation is illustrated in the figure 18.2-1.

Figure 18.2-1. RA-based SDT with UE context relocation

1.	The UE sends an RRCResumeRequest as well as UL SDT data and/or UL SDT signalling to the receiving gNB.

2.	The receiving gNB identifies the last serving gNB using the I-RNTI and retrieves the UE context by means of Xn-AP Retrieve UE Context procedure. The receiving gNB indicates that the UE request is for an SDT transaction and may also provide SDT assistance information (i.e., single packet, multiple packets).

3.	The last serving gNB decides to relocate UE context for SDT and responds with the RETRIEVE UE CONTEXT RESPONSE message. The UL SDT data, if any, is delivered to the UPF.

4-6.	The receiving gNB decides to keep UE in RRC Inactive state for SDT. If loss of DL user data buffered in the last serving gNB shall be prevented, the receiving gNB provides forwarding addresses. The receiving gNB also initiates NG-AP Path Switch procedure to establish a NG UE associated signalling connection to the serving AMF. After path switch, the buffered UL NAS PDU in the receiving gNB, if any, is delivered to AMF. And then, the subsequent UL/DL SDT data and/or signalling are transferred between UE and core network via the new gNB.

7.	After the SDT transmission is completed, the receiving gNB generates and sends RRCRelease message including the Suspend indication to the UE to send the UE back to RRC_INACTIVE.

NOTE:	In case DL non-SDT data or DL non-SDT signalling arrives, the receiving gNB may decide to directly send the UE to RRC_CONNECTED state by sending RRCResume message.

8.	The receiving gNB indicates the last serving gNB to remove the UE context by sending the XnAP UE CONTEXT RELEASE message. The XnAP UE CONTEXT RELEASE message can be sent after step 6.

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

18.3	SDT without UE context relocation

The overall procedure for SDT procedure over RACH without UE context relocation is illustrated in the figure 18.3-1.

Figure 18.3-1. RA-based SDT without UE context relocation

1/2.	The steps 1/2 are as defined in steps 1/2 in Figure 18.2-1.

3.	The last serving gNB decides not to relocate the UE context for SDT.

4.	The last serving gNB transfers a partial UE context including the SDT related RLC context.

5.	The receiving gNB acknowledges receiving the partial UE context and provides associated DL TNL address, if needed. After the Partial UE Context Retrieval procedure, the UE context is kept at the last serving gNB and the SDT related RLC context is established at the receiving gNB-. The UL SDT data and/or signalling, if any, are forwarded to the last serving gNB, and then delivered to the core network.

NOTE 1:	The DL signalling from the last serving gNB, if any, is forwarded to the receiving gNB via the RRC TRANSFER message, for which the receiving gNB delivers to the UE.

6.	After SDT transmission is completed, the last serving gNB responds to the receiving gNB with the RETRIEVE UE CONTEXT FAILURE message including an encapsulated RRCRelease message. The RRCRelease message includes suspend configuration.

NOTE 2:	The receiving gNB may send the RETRIEVE UE CONTEXT CONFIRM message to request the termination of SDT session before step 6.

NOTE 3:	In case DL non-SDT data or DL non-SDT signalling arrives, the last serving gNB moves the UE back to RRC_INACTIVE by sending RRCRelease message, and then the last serving gNB uses RAN paging to trigger the following-up RRC resume procedure for the non-SDT data/signalling transmission.

7.	The receiving gNB sends the RRCRelease message to the UE.

8.	The UE moves to RRC_INACTIVE mode.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>