3GPP TSG-RAN WG3 Meeting #116-e   	              R3-223291

9th May – 19th May 2022

Online

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0796 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support flexible I-RNTI Decode [RRCInactive] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Radisys, Reliance JIO, Nokia, Nokia Shanghai Bell, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There could be a mismatch in the I-RNTI profile configuration between neighbouring gNBs belonging to different vendors or networks. In such I-RNTI profile mismatch scenario between source and target, the target gNB receiving the Retrieve UE Context Request message may not be able to match its configured Local NG-RAN Node Identifier and the one decoded from the I-RNTI. In this case, the target gNB should try decoding the received I-RNTI using all the I-RNTI profiles defined in Annex C in TS 38.300. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If the target gNB is unable to match its configured Local NG-RAN Node Identifier with the decoded one from received I-RNTI, then the target gNB may try to decode the received I-RNTI using all the I-RNTI profiles available in Annex C in TS 38.300 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The target gNB will not be able to find the UE context from the received I-RNTI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.46 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the first change

#### 9.2.3.46	I-RNTI

The I-RNTI is defined for allocation in an NR or E-UTRA serving cell as a reference to a UE Context within an NG-RAN node. The I-RNTI is partitioned into two parts, the first part identifies the NG-RAN node that allocated the I-RNTI and the second part identifies the UE context stored in this NG-RAN node, refer to Annex C in TS 38.300 [9], or the I-RNTI is partitioned into three parts, the first part indicates the length of NG-RAN Node ID part of the NG-RAN Node that allocated the I-RNTI, the second part identifies the NG-RAN node that allocated the I-RNTI and the third part identifies the UE context stored in this NG-RAN node, refer to Annex F in TS 38.300 [9].

NOTE: 	If the target NG-RAN node is unable to match its configured Local NG-RAN Node Identifier with the Local NG-RAN Node Identifier decoded from I-RNTI, then the target NG-RAN node should try decoding the I-RNTI using all I-RNTI profiles defined in Annex C in TS 38.300 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE I-RNTI |  |  |  |  |
| >I-RNTI full |  |  |  |  |
| >>I-RNTI full | M |  | BIT STRING (SIZE (40)) | This IE is used to identify the suspended UE context of a UE in RRC_INACTIVE using 40 bits (refer to I-RNTI-Value IE in TS 38.331 [10] and I-RNTI IE in TS 36.331 [14]). |
| >I-RNTI short |  |  |  |  |
| >>I-RNTI short | M |  | BIT STRING (SIZE (24)) | This IE is used to identify the suspended UE context of a UE in RRC_INACTIVE using 24 bits (refer to ShortI-RNTI-Value IE in TS 38.331 [10] and ShortI-RNTI IE in TS 36.331 [14]). |

End of the change