3GPP TSG-RAN WG3 #116-e	R3-223381

# Online, 9th - 19th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0807 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS XnAP Corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1/ resolve Editor's Note on MBS Session status indication in §9.2.1.36 2/ Value of maxnoofMBSSessionsofUE in the MBS Session Information List IE too high 3/ Values of maxnoofCellsforMBS and maxnoofTAIforMBS in MBS Service Area Information IE too high 4/ No S-NSSAI within the MBS Session Information List IE |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1/ remove Editor's Note and provide explicit statements in the Handover Preparation procedure 2/ Value of maxnoofMBSSessionsofUE in the MBS Session Information List IE modified from 8192 to 256 3/ Values of maxnoofCellsforMBS and maxnoofTAIforMBS in MBS Service Area Information IE modified from 8192/1024 to 256/256 4/ S-NSSAI included in the MBS Session Information List IE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MBS feature remains incomplete and erroneous |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2, 9.2.1.36, 9.2.1.38, 9.2.3.149, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the MBS Session Information List IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, establish MBS session resources as specified in TS 23.247 [46] and TS 38.300 [9], if applicable.

If the HANDOVER REQUEST message includes the MBS Area Session ID IE, the target NG-RAN shall use this information as an indication from which MBS Area Session ID the UE is handed over. If the HANDOVER REQUEST message contains the MBS Service Area IE in the MBS Session Information List IE, the target NG-RAN shall use this information to setup respective MBS Session Resources.

If the HANDOVER REQUEST ACKNOWLEDGE message contains in the MBS Session Information Response List IE the MBS Data Forwarding Response Info IE that the source NG-RAN node shall use the information for forwarding MBS traffic to the target NG-RAN node.

If the MBS Session Associated Information List IE is included in the PDU Session Resources To Be Setup List IE in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use the information contained in the Associated QoS Flows Information List IE as specified in TS 23.247 [46].

For each MBS Session for which the MBS Mapping and Data Forwarding Request Info from source NG-RAN node IE is included, the target NG-RAN node shall assume that the MBS Session is active. For each MRB indicated in the MBS Mapping and Data Forwarding Request Info from source NG-RAN node IE, the target NG-RAN node shall use the MRB ID IE and the MRB Progress Information IE which includes the highest PDCP SN of the packet which has already been delivered to the UE for the MRB to decide whether to apply data forwarding for that MRB and to establish respective resources.

The source NG-RAN shall, for each MRB in the MBS Data Forwarding Response Info from target NG-RAN node IE in the HANDOVER REQUEST ACKNOWLEDGE message, start data forwarding to the indicated DL Forwarding UP TNL Information. If the MRB Progress Information IE is included the source NG-RAN node may use the information to determine when to stop data forwarding.

If the Time Synchronisation Assistance Information IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it as defined in TS 23.501 [7].

If the QMC Configuration Information IE is contained in the HANDOVER REQUEST message, the NG-RAN node shall, if supported, take it into account for QoE measurements handling, as described in TS 38.300 [9].

If the UE Slice-Maximum Bit Rate List IE is contained in HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received UE Slice Maximum Bit Rate List in the UE context, and use the received UE Slice Maximum Bit Rate value for each S-NSSAI for the concerned UE as specified in TS 23.501 [7].

Interaction with SN Status Transfer procedure:

If the UE Context Kept Indicator IE set to "True" and the DRBs transferred to MN IE are included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, include the uplink/downlink PDCP SN and HFN status received from the S-NG-RAN node in the SN Status Transfer procedure towards the target NG-RAN node, as specified in TS 37.340 [8].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.36	MBS Session Information List

This IE contains MBS session resource related information used at UE context transfer between NG-RAN nodes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session Information To Be Setup List |  | 1..<maxnoofMBSSessions> |  |  |
| >MBS Session ID | M |  | 9.2.3.146 |  |
| >>S-NSSAI | M |  | 9.2.3.21 |  |
| >MBS Area Session ID | O |  | 9.2.3.148 | MBS Area Session ID of the UE at the NG-RAN node from which the UE context is transferred |
| >MBS Service Area | O |  | 9.2.3.150 |  |
| >MBS QoS Flows to Add List |  | 1..<maxnoofMBSQoSFlows> |  |  |
| >>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.2.3.10 |  |
| >>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.2.3.5 |  |
| >MBS Mapping and Data Forwarding Request Info from source NG-RAN node | O |  | 9.2.1.39 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSSessions | Maximum no. of MBS Sessions. Value is 256. |
| maxnoofMBSQoSFlows | Maximum no. of QoS flows allowed within one MBS session. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.38	MBS Session Information Response List

This IE contains MBS session resource related information to be provided in response to information provided in the MBS Session Information List IE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session Information Response List |  | 1..<maxnoofMBSSessions> |  |  |
| >MBS Session ID | M |  | 9.2.3.146 |  |
| >MBS Data Forwarding Response Info from target NG-RAN node | O |  | 9.2.1.40 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSSessions | Maximum no. of MBS Sessions. Value is 256. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.149	MBS Service Area information

This IE contains the MBS service area information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Service Area Cell List |  | 0..<maxnoofCellsforMBS> |  |  |
| >NR CGI | M |  | 9.2.2.7 |  |
| MBS Service Area TAI List |  | 0..<maxnoofTAIforMBS> |  |  |
| >PLMN Identity | M |  | 9.2.2.4 |  |
| >TAC | M |  | 9.2.2.5 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsforMBS | Maximum no. of cells allowed within one MBS Service Area. Value is 256. |
| maxnoofTAIforMBS | Maximum no. of TAs allowed within one MBS Service Area. Value is 256. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

MBS-SessionInformation-List ::= SEQUENCE (SIZE(1..maxnoofMBSSessions)) OF MBS-SessionInformation-Item

MBS-SessionInformation-Item ::= SEQUENCE {

mBS-Session-ID			MBS-Session-ID,

s-NSSAI					S-NSSAI,

mBS-Area-Session-ID		MBS-Area-Session-ID													OPTIONAL,

mBS-ServiceArea			MBS-ServiceArea														OPTIONAL,

mBS-QoSFlowsToAdd-List	MBS-QoSFlowsToAdd-List,

mBS-MappingandDataForwardingInfofromSource		MBS-MappingandDataForwardingInfofromSource	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { MBS-SessionInformation-Item-ExtIEs} }	OPTIONAL,

...

}

MBS-SessionInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Last Change >>>>>>>>>>>>>>>>>>>>

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-sNStatusTransfer												ProcedureCode ::= 1

id-handoverCancel												ProcedureCode ::= 2

id-retrieveUEContext											ProcedureCode ::= 3

id-rANPaging													ProcedureCode ::= 4

id-xnUAddressIndication											ProcedureCode ::= 5

id-uEContextRelease												ProcedureCode ::= 6

id-sNGRANnodeAdditionPreparation								ProcedureCode ::= 7

id-sNGRANnodeReconfigurationCompletion							ProcedureCode ::= 8

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 9

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 10

id-mNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 11

id-sNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 12

id-sNGRANnodeCounterCheck										ProcedureCode ::= 13

id-sNGRANnodeChange												ProcedureCode ::= 14

id-rRCTransfer													ProcedureCode ::= 15

id-xnRemoval													ProcedureCode ::= 16

id-xnSetup														ProcedureCode ::= 17

id-nGRANnodeConfigurationUpdate									ProcedureCode ::= 18

id-cellActivation												ProcedureCode ::= 19

id-reset														ProcedureCode ::= 20

id-errorIndication												ProcedureCode ::= 21

id-privateMessage												ProcedureCode ::= 22

id-notificationControl											ProcedureCode ::= 23

id-activityNotification											ProcedureCode ::= 24

id-e-UTRA-NR-CellResourceCoordination							ProcedureCode ::= 25

id-secondaryRATDataUsageReport									ProcedureCode ::= 26

id-deactivateTrace												ProcedureCode ::= 27

id-traceStart													ProcedureCode ::= 28

id-handoverSuccess												ProcedureCode ::= 29

id-conditionalHandoverCancel									ProcedureCode ::= 30

id-earlyStatusTransfer											ProcedureCode ::= 31

id-failureIndication												ProcedureCode ::= 32

id-handoverReport												ProcedureCode ::= 33

id-resourceStatusReportingInitiation							ProcedureCode ::= 34

id-resourceStatusReporting										ProcedureCode ::= 35

id-mobilitySettingsChange										ProcedureCode ::= 36

id-accessAndMobilityIndication									ProcedureCode ::= 37

id-cellTrafficTrace 											ProcedureCode ::= 38

id-RANMulticastGroupPaging										ProcedureCode ::= 39

id-scgFailureInformationReport									ProcedureCode ::= 40

id-scgFailureInformationTransfer								ProcedureCode ::= 41

id-scgFailureTransfer											ProcedureCode ::= 42

id-f1CTrafficTransfer											ProcedureCode ::= 43

id-iABTransportMigrationManagement								ProcedureCode ::= 44

id-iABTransportMigrationModification							ProcedureCode ::= 45

id-iABResourceCoordination										ProcedureCode ::= 46

id-retrieveUEContextConfirm										ProcedureCode ::= 47

id-cPCCancel													ProcedureCode ::= 48

id-partialUEContextTransfer										ProcedureCode ::= 49

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxEARFCN									INTEGER ::= 262143

maxnoofAllowedAreas							INTEGER ::= 16

maxnoofAMFRegions							INTEGER ::= 16

maxnoofAoIs									INTEGER ::= 64

maxnoofBluetoothName						INTEGER ::= 4

maxnoofBPLMNs								INTEGER ::= 12

maxnoofCAGs									INTEGER ::= 12

maxnoofCAGsperPLMN							INTEGER ::= 256

maxnoofCellIDforMDT							INTEGER ::= 32

maxnoofCellsinAoI							INTEGER ::= 256

maxnoofCellsinUEHistoryInfo					INTEGER ::= 16

maxnoofCellsinNG-RANnode					INTEGER ::= 16384

maxnoofCellsinRNA							INTEGER ::= 32

maxnoofCellsUEMovingTrajectory				INTEGER ::= 16

maxnoofDRBs									INTEGER ::= 32

maxnoofEUTRABands							INTEGER ::= 16

maxnoofEUTRABPLMNs							INTEGER ::= 6

maxnoofEPLMNs								INTEGER ::= 15

maxnoofExtSliceItems						INTEGER ::= 65535

maxnoofEPLMNsplus1							INTEGER ::= 16

maxnoofForbiddenTACs							INTEGER ::= 4096

maxnoofFreqforMDT							INTEGER ::= 8

maxnoofMBSFNEUTRA							INTEGER ::= 8

maxnoofMDTPLMNs								INTEGER ::= 16

maxnoofMultiConnectivityMinusOne			INTEGER ::= 3

maxnoofNeighbours							INTEGER ::= 1024

maxnoofNeighPCIforMDT						INTEGER ::= 32

maxnoofNIDs									INTEGER ::= 12

maxnoofNRCellBands							INTEGER ::= 32

maxnoofPLMNs								INTEGER ::= 16

maxnoofPDUSessions							INTEGER ::= 256

maxnoofProtectedResourcePatterns			INTEGER ::= 16

maxnoofQoSFlows								INTEGER ::= 64

maxnoofQoSParaSets							INTEGER ::= 8

maxnoofRANAreaCodes							INTEGER ::= 32

maxnoofRANAreasinRNA						INTEGER ::= 16

maxnoofRANNodesinAoI						INTEGER ::= 64

maxnoofSCellGroups							INTEGER ::= 3

maxnoofSCellGroupsplus1						INTEGER ::= 4

maxnoofSensorName							INTEGER ::= 3

maxnoofSliceItems							INTEGER ::= 1024

maxnoofSNPNIDs								INTEGER ::= 12

maxnoofsupportedPLMNs						INTEGER ::= 12

maxnoofsupportedTACs						INTEGER ::= 256

maxnoofTAforMDT								INTEGER ::= 8

maxnoofTAI									INTEGER ::= 16

maxnoofTAIsinAoI 							INTEGER ::= 16

maxnooftimeperiods							INTEGER ::= 2

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofUEContexts							INTEGER ::= 8192

maxNRARFCN									INTEGER ::= 3279165

maxNrOfErrors								INTEGER ::= 256

maxnoofslots								INTEGER ::= 5120

maxnoofExtTLAs								INTEGER ::= 16

maxnoofGTPTLAs								INTEGER ::= 16

maxnoofCHOcells								INTEGER ::= 8

maxnoofPC5QoSFlows 							INTEGER ::= 2064

maxnoofSSBAreas								INTEGER ::= 64

maxnoofRACHReports							INTEGER ::= 64

maxnoofNRSCSs								INTEGER ::= 5

maxnoofPhysicalResourceBlocks				INTEGER ::= 275

maxnoofAdditionalPDCPDuplicationTNL			INTEGER ::= 2

maxnoofRLCDuplicationstate					INTEGER ::= 3

maxnoofWLANName								INTEGER ::= 4

maxnoofNonAnchorCarrierFreqConfig			INTEGER ::= 15

maxnoofDataForwardingTunneltoE-UTRAN		INTEGER ::= 256

maxnoofMBSFSAs								INTEGER ::= 256

maxnoofUEIDIndicesforMBSPaging				INTEGER ::= 4096

maxnoofMBSQoSFlows							INTEGER ::= 64

maxnoofMRBs									INTEGER ::= 32

maxnoofCellsforMBS							INTEGER ::= 256

maxnoofMBSServiceAreaInformation				INTEGER ::= 256

maxnoofTAIforMBS 							INTEGER ::= 256

maxnoofAssociatedMBSSessions				INTEGER ::= 32

maxnoofMBSSessions							INTEGER ::= 256

maxnoofSuccessfulHOReports					INTEGER ::= 64

maxnoofPSCellsPerSN 						INTEGER ::= 8

maxnoofNR-UChannelIDs						INTEGER ::= 4

maxnoofCellsinCHO							INTEGER ::= 8

maxnoofCHOexecutioncond						INTEGER ::= 2

maxnoofServedCellsIAB						INTEGER ::= 512

maxnoofServingCells							INTEGER ::= 32

maxnoofBHInfo								INTEGER ::= 1024

maxnoofTrafficIndexEntries					INTEGER	::=	1024

maxnoofTLAsIAB								INTEGER	::=	1024

maxnoofBAPControlPDURLCCHs					INTEGER ::= 2

maxnoofIABSTCInfo							INTEGER ::= 45

maxnoofSymbols								INTEGER ::= 14

maxnoofDUFSlots								INTEGER ::= 320

maxnoofHSNASlots							INTEGER ::= 5120

maxnoofRBsetsPerCell						INTEGER ::= 8

maxnoofRBsetsPerCell1						INTEGER ::= 7

maxnoofChildIABNodes						INTEGER ::= 1024

maxnoofPhysicalResourceBlocks1				INTEGER ::= 274

maxnoofPSCellCandidates						INTEGER ::= 8

maxnoofTargetSNs 							INTEGER ::= 8

maxnoofUEAppLayerMeas						INTEGER ::= 16

maxnoofSNSSAIforQMC							INTEGER ::= 16

maxnoofCellIDforQMC							INTEGER ::= 32

maxnoofPLMNforQMC							INTEGER ::= 16

maxnoofTAforQMC								INTEGER ::= 8

maxnoofMTCItems								INTEGER ::= 16

maxnoofCSIRSconfigurations					INTEGER ::= 96

maxnoofCSIRSneighbourCells					INTEGER ::= 16

maxnoofCSIRSneighbourCellsInMTC				INTEGER ::= 16

maxnoofNeighbour-NG-RAN-Nodes				INTEGER ::= 256

maxnoofSRBs									INTEGER ::= 5

maxnoofSMBR									INTEGER ::= 8

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>