3GPP TSG-RAN WG3 Meeting #116-e	R3-223400

# 9-19 May 2022

# Online

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1885 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of LTE-M Satellite Indication over S1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN |  |  |  |  |  | Date: |  |  | 2022-04-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the reply LS from SA2 in R3-223030, there is no need to have a separate new indication (LTE-M Satellite Indication) in UE CAPABILITY INFO INDICATION message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the IE LTE-M Satellite Indication in UE CAPABILITY INFO INDICATION message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current LTE-M Satellite Indication in UE CAPABILITY INFO INDICATION message is redundant. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9.2, 9.1.10, 9.3.3, 9.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>> START OF CHANGE <<<

8.9.2	Successful Operation

Figure 8.9.2-1: UE Capability Info Indication procedure. Successful operation.

The eNB controlling a UE-associated logical S1-connection initiates the procedure by sending a UE CAPABILITY INFO INDICATION message to the MME including the UE capability information. The UE CAPABILITY INFO INDICATION message may also include paging specific UE capability information within the UE Radio Capability for Paging IE. The UE capability information received by the MME shall replace previously stored corresponding UE capability information in the MME for the UE, as described in TS 23.401 [11].

If UE CAPABILITY INFO INDICATION message contains the LTE-M indication IE, the MME shall, if supported, store this information in the UE context and use it according to TS 23.401 [11].

If the UE indicates the support for UE Application Layer Measurement, the eNB shall if supported include the UE Application Layer Measurement Capability IE in the UE CAPABILITY INFO INDICATION message. The MME shall, if supported, store and use this information when initiating UE Application Layer Measurement.

If UE CAPABILITY INFO INDICATION message contains the UE Radio Capability – NR Format IE, the MME shall, if supported, use it according to TS 23.401 [11].

If the UE RADIO CAPABILITY INFO INDICATION message includes the UE Radio Capability for Paging IE and the UE Radio Capability for Paging – NR Format IE, the MME shall, if supported, use it according to TS 23.401 [11].

>>> Next CHANGE <<<

9.1.10	UE CAPABILITY INFO INDICATION

This message is sent by the eNB to provide UE Radio Capability information to the MME.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| UE Radio Capability | M |  | 9.2.1.27 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.2.1.98 |  | YES | ignore |
| UE Application Layer Measurement Capability | O |  | BIT STRING (SIZE(8)) | Each bit in the bitmap indicates an UE Application layer measurement capability, refer to TS 25.331[10]. Bit 0 = QoE Measurement for streaming service Bit 1 = QoE Measurement for MTSI service Value ‘1’ indicates “Capable” and value ‘0’ indicates “not Capable”. Unused bits are reserved for future use. | YES | ignore |
| LTE-M Indication | O |  | 9.2.1.135 |  | YES | ignore |
| UE Radio Capability – NR Format | O |  | 9.2.1.154 |  | YES | ignore |
| UE Radio Capability for Paging – NR Format | O |  | 9.2.1.160 |  | YES | ignore |

>>> Next CHANGE <<<

9.3.3	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for S1AP.

--

-- **************************************************************

S1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

UEAggregateMaximumBitrate,

BearerType,

Cause,

CellAccessMode,

Cdma2000HORequiredIndication,

Cdma2000HOStatus,

Cdma2000OneXSRVCCInfo,

Cdma2000OneXRAND,

Cdma2000PDU,

Cdma2000RATType,

Cdma2000SectorID,

EUTRANRoundTripDelayEstimationInfo,

CNDomain,

ConcurrentWarningMessageIndicator,

CriticalityDiagnostics,

CSFallbackIndicator,

CSG-Id,

CSG-IdList,

CSGMembershipStatus,

Data-Forwarding-Not-Possible,

Direct-Forwarding-Path-Availability,

Global-ENB-ID,

EUTRAN-CGI,

ENBname,

ENB-StatusTransfer-TransparentContainer,

ENB-UE-S1AP-ID,

ExtendedRepetitionPeriod,

GTP-TEID,

GUMMEI,

GUMMEIType,

HandoverRestrictionList,

HandoverType,

Masked-IMEISV,

LAI,

LPPa-PDU,

ManagementBasedMDTAllowed,

MDTPLMNList,

MMEname,

MMERelaySupportIndicator,

MME-UE-S1AP-ID,

MSClassmark2,

MSClassmark3,

NAS-PDU,

NASSecurityParametersfromE-UTRAN,

NASSecurityParameterstoE-UTRAN,

OverloadResponse,

PagingDRX,

PagingPriority,

PLMNidentity,

ProSeAuthorized,

RIMTransfer,

RelativeMMECapacity,

RequestType,

E-RAB-ID,

E-RABLevelQoSParameters,

E-RABList,

RelayNode-Indicator,

Routing-ID,

SecurityKey,

SecurityContext,

ServedGUMMEIs,

SONConfigurationTransfer,

Source-ToTarget-TransparentContainer,

SourceBSS-ToTargetBSS-TransparentContainer,

SourceeNB-ToTargeteNB-TransparentContainer,

SourceRNC-ToTargetRNC-TransparentContainer,

SubscriberProfileIDforRFP,

SRVCCOperationNotPossible,

SRVCCOperationPossible,

SRVCCHOIndication,

SupportedTAs,

TAI,

Target-ToSource-TransparentContainer,

TargetBSS-ToSourceBSS-TransparentContainer,

TargeteNB-ToSourceeNB-TransparentContainer,

TargetID,

TargetRNC-ToSourceRNC-TransparentContainer,

TimeToWait,

TraceActivation,

TrafficLoadReductionIndication,

E-UTRAN-Trace-ID,

TransportLayerAddress,

UEIdentityIndexValue,

UEPagingID,

UERadioCapability,

UERadioCapabilityForPaging,

UE-RetentionInformation,

UE-S1AP-IDs,

UE-associatedLogicalS1-ConnectionItem,

UESecurityCapabilities,

S-TMSI,

MessageIdentifier,

SerialNumber,

WarningAreaList,

RepetitionPeriod,

NumberofBroadcastRequest,

WarningType,

WarningSecurityInfo,

DataCodingScheme,

WarningMessageContents,

BroadcastCompletedAreaList,

RRC-Establishment-Cause,

BroadcastCancelledAreaList,

PS-ServiceNotAvailable,

GUMMEIList,

Correlation-ID,

GWContextReleaseIndication,

PrivacyIndicator,

VoiceSupportMatchIndicator,

TunnelInformation,

KillAllWarningMessages,

TransportInformation,

LHN-ID,

UserLocationInformation,

AdditionalCSFallbackIndicator,

ECGIListForRestart,

TAIListForRestart,

EmergencyAreaIDListForRestart,

ExpectedUEBehaviour,

Paging-eDRXInformation,

Extended-UEIdentityIndexValue,

MME-Group-ID,

Additional-GUTI,

PWSfailedECGIList,

CellIdentifierAndCELevelForCECapableUEs,

AssistanceDataForPaging,

InformationOnRecommendedCellsAndENBsForPaging,

UE-Usage-Type,

UEUserPlaneCIoTSupportIndicator,

NB-IoT-DefaultPagingDRX,

NB-IoT-Paging-eDRXInformation,

CE-mode-B-SupportIndicator,

NB-IoT-UEIdentityIndexValue,

V2XServicesAuthorized,

DCN-ID,

ServedDCNs,

UESidelinkAggregateMaximumBitrate,

DLNASPDUDeliveryAckRequest,

Coverage-Level,

EnhancedCoverageRestricted,

DL-CP-SecurityInformation,

UL-CP-SecurityInformation,

SecondaryRATDataUsageRequest,

SecondaryRATDataUsageReportList,

HandoverFlag,

NRUESecurityCapabilities,

UE-Application-Layer-Measurement-Capability,

CE-ModeBRestricted,

Packet-LossRate,

UECapabilityInfoRequest,

SourceNgRanNode-ToTargetNgRanNode-TransparentContainer,

TargetNgRanNode-ToSourceNgRanNode-TransparentContainer,

EndIndication,

EDT-Session,

AerialUEsubscriptionInformation,

PendingDataIndication,

WarningAreaCoordinates,

Subscription-Based-UE-DifferentiationInfo,

PSCellInformation,

NR-CGI,

ConnectedengNBList,

EN-DCSONConfigurationTransfer,

TimeSinceSecondaryNodeRelease,

AdditionalRRMPriorityIndex,

IAB-Authorized,

IAB-Node-Indication,

IAB-Supported,

DataSize,

Ethernet-Type,

NRV2XServicesAuthorized,

NRUESidelinkAggregateMaximumBitrate,

PC5QoSParameters,

IntersystemSONConfigurationTransfer,

UERadioCapabilityID,

NotifySourceeNB,

ENB-EarlyStatusTransfer-TransparentContainer,

WUS-Assistance-Information,

NB-IoT-PagingDRX,

PagingCause,

SecurityIndication,

SecurityResult,

LTE-NTN-TAI-Information

>>> Next CHANGE <<<

-- **************************************************************

--

#### -- UE CAPABILITY INFO INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- UE Capability Info Indication

--

-- **************************************************************

UECapabilityInfoIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UECapabilityInfoIndicationIEs} },

...

}

UECapabilityInfoIndicationIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID				CRITICALITY reject	TYPE MME-UE-S1AP-ID					PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID				CRITICALITY reject	TYPE ENB-UE-S1AP-ID					PRESENCE mandatory}|

{ ID id-UERadioCapability			CRITICALITY ignore	TYPE UERadioCapability				PRESENCE mandatory}|

{ ID id-UERadioCapabilityForPaging	CRITICALITY ignore	TYPE UERadioCapabilityForPaging		PRESENCE optional}|

{ ID id-UE-Application-Layer-Measurement-Capability		CRITICALITY ignore	TYPE UE-Application-Layer-Measurement-Capability							PRESENCE optional}|

{ ID id-LTE-M-Indication				CRITICALITY ignore	TYPE LTE-M-Indication				PRESENCE optional}|

{ ID id-UERadioCapability-NR-Format	CRITICALITY ignore	TYPE UERadioCapability				PRESENCE optional}|

{ ID id-UERadioCapabilityForPaging-NR-Format	CRITICALITY ignore	TYPE UERadioCapabilityForPaging				PRESENCE optional}

,

...

}

>>> Next CHANGE <<<

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-MME-UE-S1AP-ID									ProtocolIE-ID ::= 0

id-HandoverType										ProtocolIE-ID ::= 1

id-Cause											ProtocolIE-ID ::= 2

id-SourceID											ProtocolIE-ID ::= 3

id-TargetID											ProtocolIE-ID ::= 4

id-eNB-UE-S1AP-ID									ProtocolIE-ID ::= 8

id-E-RABSubjecttoDataForwardingList					ProtocolIE-ID ::= 12

id-E-RABtoReleaseListHOCmd							ProtocolIE-ID ::= 13

id-E-RABDataForwardingItem							ProtocolIE-ID ::= 14

id-E-RABReleaseItemBearerRelComp					ProtocolIE-ID ::= 15

id-E-RABToBeSetupListBearerSUReq					ProtocolIE-ID ::= 16

id-E-RABToBeSetupItemBearerSUReq					ProtocolIE-ID ::= 17

id-E-RABAdmittedList								ProtocolIE-ID ::= 18

id-E-RABFailedToSetupListHOReqAck					ProtocolIE-ID ::= 19

id-E-RABAdmittedItem								ProtocolIE-ID ::= 20

id-E-RABFailedtoSetupItemHOReqAck					ProtocolIE-ID ::= 21

id-E-RABToBeSwitchedDLList							ProtocolIE-ID ::= 22

id-E-RABToBeSwitchedDLItem							ProtocolIE-ID ::= 23

id-E-RABToBeSetupListCtxtSUReq						ProtocolIE-ID ::= 24

id-TraceActivation									ProtocolIE-ID ::= 25

id-NAS-PDU											ProtocolIE-ID ::= 26

id-E-RABToBeSetupItemHOReq							ProtocolIE-ID ::= 27

id-E-RABSetupListBearerSURes						ProtocolIE-ID ::= 28

id-E-RABFailedToSetupListBearerSURes				ProtocolIE-ID ::= 29

id-E-RABToBeModifiedListBearerModReq				ProtocolIE-ID ::= 30

id-E-RABModifyListBearerModRes						ProtocolIE-ID ::= 31

id-E-RABFailedToModifyList							ProtocolIE-ID ::= 32

id-E-RABToBeReleasedList							ProtocolIE-ID ::= 33

id-E-RABFailedToReleaseList							ProtocolIE-ID ::= 34

id-E-RABItem										ProtocolIE-ID ::= 35

id-E-RABToBeModifiedItemBearerModReq				ProtocolIE-ID ::= 36

id-E-RABModifyItemBearerModRes						ProtocolIE-ID ::= 37

id-E-RABReleaseItem									ProtocolIE-ID ::= 38

id-E-RABSetupItemBearerSURes						ProtocolIE-ID ::= 39

id-SecurityContext									ProtocolIE-ID ::= 40

id-HandoverRestrictionList							ProtocolIE-ID ::= 41

id-UEPagingID										ProtocolIE-ID ::= 43

id-pagingDRX										ProtocolIE-ID ::= 44

id-TAIList											ProtocolIE-ID ::= 46

id-TAIItem											ProtocolIE-ID ::= 47

id-E-RABFailedToSetupListCtxtSURes					ProtocolIE-ID ::= 48

id-E-RABReleaseItemHOCmd							ProtocolIE-ID ::= 49

id-E-RABSetupItemCtxtSURes							ProtocolIE-ID ::= 50

id-E-RABSetupListCtxtSURes							ProtocolIE-ID ::= 51

id-E-RABToBeSetupItemCtxtSUReq						ProtocolIE-ID ::= 52

id-E-RABToBeSetupListHOReq							ProtocolIE-ID ::= 53

id-GERANtoLTEHOInformationRes						ProtocolIE-ID ::= 55

id-UTRANtoLTEHOInformationRes						ProtocolIE-ID ::= 57

id-CriticalityDiagnostics 							ProtocolIE-ID ::= 58

id-Global-ENB-ID									ProtocolIE-ID ::= 59

id-eNBname											ProtocolIE-ID ::= 60

id-MMEname											ProtocolIE-ID ::= 61

id-ServedPLMNs										ProtocolIE-ID ::= 63

id-SupportedTAs										ProtocolIE-ID ::= 64

id-TimeToWait										ProtocolIE-ID ::= 65

id-uEaggregateMaximumBitrate						ProtocolIE-ID ::= 66

id-TAI												ProtocolIE-ID ::= 67

id-E-RABReleaseListBearerRelComp					ProtocolIE-ID ::= 69

id-cdma2000PDU										ProtocolIE-ID ::= 70

id-cdma2000RATType									ProtocolIE-ID ::= 71

id-cdma2000SectorID									ProtocolIE-ID ::= 72

id-SecurityKey										ProtocolIE-ID ::= 73

id-UERadioCapability								ProtocolIE-ID ::= 74

id-GUMMEI-ID										ProtocolIE-ID ::= 75

id-E-RABInformationListItem							ProtocolIE-ID ::= 78

id-Direct-Forwarding-Path-Availability				ProtocolIE-ID ::= 79

id-UEIdentityIndexValue								ProtocolIE-ID ::= 80

id-cdma2000HOStatus									ProtocolIE-ID ::= 83

id-cdma2000HORequiredIndication						ProtocolIE-ID ::= 84

id-E-UTRAN-Trace-ID									ProtocolIE-ID ::= 86

id-RelativeMMECapacity								ProtocolIE-ID ::= 87

id-SourceMME-UE-S1AP-ID								ProtocolIE-ID ::= 88

id-Bearers-SubjectToStatusTransfer-Item				ProtocolIE-ID ::= 89

id-eNB-StatusTransfer-TransparentContainer			ProtocolIE-ID ::= 90

id-UE-associatedLogicalS1-ConnectionItem			ProtocolIE-ID ::= 91

id-ResetType										ProtocolIE-ID ::= 92

id-UE-associatedLogicalS1-ConnectionListResAck		ProtocolIE-ID ::= 93

id-E-RABToBeSwitchedULItem							ProtocolIE-ID ::= 94

id-E-RABToBeSwitchedULList							ProtocolIE-ID ::= 95

id-S-TMSI											ProtocolIE-ID ::= 96

id-cdma2000OneXRAND									ProtocolIE-ID ::= 97

id-RequestType										ProtocolIE-ID ::= 98

id-UE-S1AP-IDs										ProtocolIE-ID ::= 99

id-EUTRAN-CGI										ProtocolIE-ID ::= 100

id-OverloadResponse									ProtocolIE-ID ::= 101

id-cdma2000OneXSRVCCInfo							ProtocolIE-ID ::= 102

id-E-RABFailedToBeReleasedList						ProtocolIE-ID ::= 103

id-Source-ToTarget-TransparentContainer				ProtocolIE-ID ::= 104

id-ServedGUMMEIs									ProtocolIE-ID ::= 105

id-SubscriberProfileIDforRFP						ProtocolIE-ID ::= 106

id-UESecurityCapabilities							ProtocolIE-ID ::= 107

id-CSFallbackIndicator								ProtocolIE-ID ::= 108

id-CNDomain											ProtocolIE-ID ::= 109

id-E-RABReleasedList								ProtocolIE-ID ::= 110

id-MessageIdentifier								ProtocolIE-ID ::= 111

id-SerialNumber										ProtocolIE-ID ::= 112

id-WarningAreaList									ProtocolIE-ID ::= 113

id-RepetitionPeriod									ProtocolIE-ID ::= 114

id-NumberofBroadcastRequest							ProtocolIE-ID ::= 115

id-WarningType										ProtocolIE-ID ::= 116

id-WarningSecurityInfo								ProtocolIE-ID ::= 117

id-DataCodingScheme									ProtocolIE-ID ::= 118

id-WarningMessageContents							ProtocolIE-ID ::= 119

id-BroadcastCompletedAreaList						ProtocolIE-ID ::= 120

id-Inter-SystemInformationTransferTypeEDT			ProtocolIE-ID ::= 121

id-Inter-SystemInformationTransferTypeMDT			ProtocolIE-ID ::= 122

id-Target-ToSource-TransparentContainer				ProtocolIE-ID ::= 123

id-SRVCCOperationPossible							ProtocolIE-ID ::= 124

id-SRVCCHOIndication								ProtocolIE-ID ::= 125

id-NAS-DownlinkCount								ProtocolIE-ID ::= 126

id-CSG-Id											ProtocolIE-ID ::= 127

id-CSG-IdList										ProtocolIE-ID ::= 128

id-SONConfigurationTransferECT						ProtocolIE-ID ::= 129

id-SONConfigurationTransferMCT						ProtocolIE-ID ::= 130

id-TraceCollectionEntityIPAddress					ProtocolIE-ID ::= 131

id-MSClassmark2										ProtocolIE-ID ::= 132

id-MSClassmark3										ProtocolIE-ID ::= 133

id-RRC-Establishment-Cause							ProtocolIE-ID ::= 134

id-NASSecurityParametersfromE-UTRAN					ProtocolIE-ID ::= 135

id-NASSecurityParameterstoE-UTRAN					ProtocolIE-ID ::= 136

id-DefaultPagingDRX									ProtocolIE-ID ::= 137

id-Source-ToTarget-TransparentContainer-Secondary	ProtocolIE-ID ::= 138

id-Target-ToSource-TransparentContainer-Secondary	ProtocolIE-ID ::= 139

id-EUTRANRoundTripDelayEstimationInfo				ProtocolIE-ID ::= 140

id-BroadcastCancelledAreaList						ProtocolIE-ID ::= 141

id-ConcurrentWarningMessageIndicator				ProtocolIE-ID ::= 142

id-Data-Forwarding-Not-Possible						ProtocolIE-ID ::= 143

id-ExtendedRepetitionPeriod							ProtocolIE-ID ::= 144

id-CellAccessMode									ProtocolIE-ID ::= 145

id-CSGMembershipStatus 								ProtocolIE-ID ::= 146

id-LPPa-PDU											ProtocolIE-ID ::= 147

id-Routing-ID										ProtocolIE-ID ::= 148

id-Time-Synchronisation-Info							ProtocolIE-ID ::= 149

id-PS-ServiceNotAvailable							ProtocolIE-ID ::= 150

id-PagingPriority									ProtocolIE-ID ::= 151

id-x2TNLConfigurationInfo							ProtocolIE-ID ::= 152

id-eNBX2ExtendedTransportLayerAddresses				ProtocolIE-ID ::= 153

id-GUMMEIList										ProtocolIE-ID ::= 154

id-GW-TransportLayerAddress							ProtocolIE-ID ::= 155

id-Correlation-ID									ProtocolIE-ID ::= 156

id-SourceMME-GUMMEI									ProtocolIE-ID ::= 157

id-MME-UE-S1AP-ID-2									ProtocolIE-ID ::= 158

id-RegisteredLAI									ProtocolIE-ID ::= 159

id-RelayNode-Indicator								ProtocolIE-ID ::= 160

id-TrafficLoadReductionIndication					ProtocolIE-ID ::= 161

id-MDTConfiguration									ProtocolIE-ID ::= 162

id-MMERelaySupportIndicator							ProtocolIE-ID ::= 163

id-GWContextReleaseIndication						ProtocolIE-ID ::= 164

id-ManagementBasedMDTAllowed						ProtocolIE-ID ::= 165

id-PrivacyIndicator									ProtocolIE-ID ::= 166

id-Time-UE-StayedInCell-EnhancedGranularity			ProtocolIE-ID ::= 167

id-HO-Cause											ProtocolIE-ID ::= 168

id-VoiceSupportMatchIndicator						ProtocolIE-ID ::= 169

id-GUMMEIType										ProtocolIE-ID ::= 170

id-M3Configuration									ProtocolIE-ID ::= 171

id-M4Configuration									ProtocolIE-ID ::= 172

id-M5Configuration									ProtocolIE-ID ::= 173

id-MDT-Location-Info								ProtocolIE-ID ::= 174

id-MobilityInformation								ProtocolIE-ID ::= 175

id-Tunnel-Information-for-BBF						ProtocolIE-ID ::= 176

id-ManagementBasedMDTPLMNList						ProtocolIE-ID ::= 177

id-SignallingBasedMDTPLMNList						ProtocolIE-ID ::= 178

id-ULCOUNTValueExtended								ProtocolIE-ID ::= 179

id-DLCOUNTValueExtended								ProtocolIE-ID ::= 180

id-ReceiveStatusOfULPDCPSDUsExtended				ProtocolIE-ID ::= 181

id-ECGIListForRestart								ProtocolIE-ID ::= 182

id-SIPTO-Correlation-ID								ProtocolIE-ID ::= 183

id-SIPTO-L-GW-TransportLayerAddress					ProtocolIE-ID ::= 184

id-TransportInformation								ProtocolIE-ID ::= 185

id-LHN-ID											ProtocolIE-ID ::= 186

id-AdditionalCSFallbackIndicator					ProtocolIE-ID ::= 187

id-TAIListForRestart								ProtocolIE-ID ::= 188

id-UserLocationInformation							ProtocolIE-ID ::= 189

id-EmergencyAreaIDListForRestart					ProtocolIE-ID ::= 190

id-KillAllWarningMessages							ProtocolIE-ID ::= 191

id-Masked-IMEISV									ProtocolIE-ID ::= 192

id-eNBIndirectX2TransportLayerAddresses				ProtocolIE-ID ::= 193

id-uE-HistoryInformationFromTheUE					ProtocolIE-ID ::= 194

id-ProSeAuthorized									ProtocolIE-ID ::= 195

id-ExpectedUEBehaviour								ProtocolIE-ID ::= 196

id-LoggedMBSFNMDT									ProtocolIE-ID ::= 197

id-UERadioCapabilityForPaging						ProtocolIE-ID ::= 198

id-E-RABToBeModifiedListBearerModInd				ProtocolIE-ID ::= 199

id-E-RABToBeModifiedItemBearerModInd				ProtocolIE-ID ::= 200

id-E-RABNotToBeModifiedListBearerModInd				ProtocolIE-ID ::= 201

id-E-RABNotToBeModifiedItemBearerModInd				ProtocolIE-ID ::= 202

id-E-RABModifyListBearerModConf						ProtocolIE-ID ::= 203

id-E-RABModifyItemBearerModConf						ProtocolIE-ID ::= 204

id-E-RABFailedToModifyListBearerModConf				ProtocolIE-ID ::= 205

id-SON-Information-Report							ProtocolIE-ID ::= 206

id-Muting-Availability-Indication					ProtocolIE-ID ::= 207

id-Muting-Pattern-Information						ProtocolIE-ID ::= 208

id-Synchronisation-Information						ProtocolIE-ID ::= 209

id-E-RABToBeReleasedListBearerModConf				ProtocolIE-ID ::= 210

id-AssistanceDataForPaging							ProtocolIE-ID ::= 211

id-CellIdentifierAndCELevelForCECapableUEs			ProtocolIE-ID ::= 212

id-InformationOnRecommendedCellsAndENBsForPaging	ProtocolIE-ID ::= 213

id-RecommendedCellItem								ProtocolIE-ID ::= 214

id-RecommendedENBItem								ProtocolIE-ID ::= 215

id-ProSeUEtoNetworkRelaying							ProtocolIE-ID ::= 216

id-ULCOUNTValuePDCP-SNlength18						ProtocolIE-ID ::= 217

id-DLCOUNTValuePDCP-SNlength18						ProtocolIE-ID ::= 218

id-ReceiveStatusOfULPDCPSDUsPDCP-SNlength18			ProtocolIE-ID ::= 219

id-M6Configuration									ProtocolIE-ID ::= 220

id-M7Configuration									ProtocolIE-ID ::= 221

id-PWSfailedECGIList								ProtocolIE-ID ::= 222

id-MME-Group-ID										ProtocolIE-ID ::= 223

id-Additional-GUTI									ProtocolIE-ID ::= 224

id-S1-Message										ProtocolIE-ID ::= 225

id-CSGMembershipInfo								ProtocolIE-ID ::= 226

id-Paging-eDRXInformation							ProtocolIE-ID ::= 227

id-UE-RetentionInformation							ProtocolIE-ID ::= 228

id-UE-Usage-Type									ProtocolIE-ID ::= 230

id-extended-UEIdentityIndexValue					ProtocolIE-ID ::= 231

id-RAT-Type											ProtocolIE-ID ::= 232

id-BearerType										ProtocolIE-ID ::= 233

id-NB-IoT-DefaultPagingDRX							ProtocolIE-ID ::= 234

id-E-RABFailedToResumeListResumeReq					ProtocolIE-ID ::= 235

id-E-RABFailedToResumeItemResumeReq					ProtocolIE-ID ::= 236

id-E-RABFailedToResumeListResumeRes					ProtocolIE-ID ::= 237

id-E-RABFailedToResumeItemResumeRes					ProtocolIE-ID ::= 238

id-NB-IoT-Paging-eDRXInformation					ProtocolIE-ID ::= 239

id-V2XServicesAuthorized							ProtocolIE-ID ::= 240

id-UEUserPlaneCIoTSupportIndicator 					ProtocolIE-ID ::= 241

id-CE-mode-B-SupportIndicator 						ProtocolIE-ID ::= 242

id-SRVCCOperationNotPossible						ProtocolIE-ID ::= 243

id-NB-IoT-UEIdentityIndexValue 						ProtocolIE-ID ::= 244

id-RRC-Resume-Cause									ProtocolIE-ID ::= 245

id-DCN-ID											ProtocolIE-ID ::= 246

id-ServedDCNs					 					ProtocolIE-ID ::= 247

id-UESidelinkAggregateMaximumBitrate 				ProtocolIE-ID ::= 248

id-DLNASPDUDeliveryAckRequest						ProtocolIE-ID ::= 249

id-Coverage-Level 									ProtocolIE-ID ::= 250

id-EnhancedCoverageRestricted						ProtocolIE-ID ::= 251

id-UE-Level-QoS-Parameters							ProtocolIE-ID ::= 252

id-DL-CP-SecurityInformation						ProtocolIE-ID ::= 253

id-UL-CP-SecurityInformation						ProtocolIE-ID ::= 254

id-extended-e-RAB-MaximumBitrateDL					ProtocolIE-ID ::= 255

id-extended-e-RAB-MaximumBitrateUL					ProtocolIE-ID ::= 256

id-extended-e-RAB-GuaranteedBitrateDL				ProtocolIE-ID ::= 257

id-extended-e-RAB-GuaranteedBitrateUL				ProtocolIE-ID ::= 258

id-extended-uEaggregateMaximumBitRateDL				ProtocolIE-ID ::= 259

id-extended-uEaggregateMaximumBitRateUL				ProtocolIE-ID ::= 260

id-NRrestrictioninEPSasSecondaryRAT					ProtocolIE-ID ::= 261

id-UEAppLayerMeasConfig								ProtocolIE-ID ::= 262

id-UE-Application-Layer-Measurement-Capability		ProtocolIE-ID ::= 263

id-SecondaryRATDataUsageReportList					ProtocolIE-ID ::= 264

id-SecondaryRATDataUsageReportItem					ProtocolIE-ID ::= 265

id-HandoverFlag										ProtocolIE-ID ::= 266

id-E-RABUsageReportItem								ProtocolIE-ID ::= 267

id-SecondaryRATDataUsageRequest						ProtocolIE-ID ::= 268

id-NRUESecurityCapabilities							ProtocolIE-ID ::= 269

id-UnlicensedSpectrumRestriction					ProtocolIE-ID ::= 270

id-CE-ModeBRestricted								ProtocolIE-ID ::= 271

id-LTE-M-Indication									ProtocolIE-ID ::= 272

id-DownlinkPacketLossRate							ProtocolIE-ID ::= 273

id-UplinkPacketLossRate								ProtocolIE-ID ::= 274

id-UECapabilityInfoRequest							ProtocolIE-ID ::= 275

id-serviceType										ProtocolIE-ID ::= 276

id-AerialUEsubscriptionInformation					ProtocolIE-ID ::= 277

id-Subscription-Based-UE-DifferentiationInfo		ProtocolIE-ID ::= 278

id-EndIndication									ProtocolIE-ID ::= 280

id-EDT-Session										ProtocolIE-ID ::= 281

id-CNTypeRestrictions								ProtocolIE-ID ::= 282

id-PendingDataIndication							ProtocolIE-ID ::= 283

id-BluetoothMeasurementConfiguration				ProtocolIE-ID ::= 284

id-WLANMeasurementConfiguration						ProtocolIE-ID ::= 285

id-WarningAreaCoordinates							ProtocolIE-ID ::= 286

id-NRrestrictionin5GS								ProtocolIE-ID ::= 287

id-PSCellInformation								ProtocolIE-ID ::= 288

id-LastNG-RANPLMNIdentity							ProtocolIE-ID ::= 290

id-ConnectedengNBList								ProtocolIE-ID ::= 291

id-ConnectedengNBToAddList							ProtocolIE-ID ::= 292

id-ConnectedengNBToRemoveList						ProtocolIE-ID ::= 293

id-EN-DCSONConfigurationTransfer-ECT				ProtocolIE-ID ::= 294

id-EN-DCSONConfigurationTransfer-MCT				ProtocolIE-ID ::= 295

id-IMSvoiceEPSfallbackfrom5G						ProtocolIE-ID ::= 296

id-TimeSinceSecondaryNodeRelease					ProtocolIE-ID ::= 297

id-RequestTypeAdditionalInfo						ProtocolIE-ID ::= 298

id-AdditionalRRMPriorityIndex						ProtocolIE-ID ::= 299

id-ContextatSource									ProtocolIE-ID ::= 300

id-IAB-Authorized									ProtocolIE-ID ::= 301

id-IAB-Node-Indication								ProtocolIE-ID ::= 302

id-IAB-Supported									ProtocolIE-ID ::= 303

id-DataSize											ProtocolIE-ID ::= 304

id-Ethernet-Type									ProtocolIE-ID ::= 305

id-NRV2XServicesAuthorized							ProtocolIE-ID ::= 306

id-NRUESidelinkAggregateMaximumBitrate				ProtocolIE-ID ::= 307

id-PC5QoSParameters									ProtocolIE-ID ::= 308

id-IntersystemSONConfigurationTransferMCT			ProtocolIE-ID ::= 309

id-IntersystemSONConfigurationTransferECT			ProtocolIE-ID ::= 310

id-IntersystemMeasurementConfiguration				ProtocolIE-ID ::= 311

id-SourceNodeID										ProtocolIE-ID ::= 312

id-NB-IoT-RLF-Report-Container						ProtocolIE-ID ::= 313

id-UERadioCapabilityID								ProtocolIE-ID ::= 314

id-UERadioCapability-NR-Format						ProtocolIE-ID ::= 315

id-MDTConfigurationNR								ProtocolIE-ID ::= 316

id-DAPSRequestInfo									ProtocolIE-ID ::= 317

id-DAPSResponseInfoList								ProtocolIE-ID ::= 318

id-DAPSResponseInfoItem								ProtocolIE-ID ::= 319

id-NotifySourceeNB									ProtocolIE-ID ::= 320

id-eNB-EarlyStatusTransfer-TransparentContainer		ProtocolIE-ID ::= 321

id-Bearers-SubjectToEarlyStatusTransfer-Item		ProtocolIE-ID ::= 322

id-WUS-Assistance-Information						ProtocolIE-ID ::= 323

id-NB-IoT-PagingDRX									ProtocolIE-ID ::= 324

id-TraceCollectionEntityURI							ProtocolIE-ID ::= 325

id-EmergencyIndicator								ProtocolIE-ID ::= 326

id-UERadioCapabilityForPaging-NR-Format 			ProtocolIE-ID ::= 327

id-SourceTransportLayerAddress						ProtocolIE-ID ::= 328

id-lastVisitedPSCellList							ProtocolIE-ID ::= 329

id-RACSIndication									ProtocolIE-ID ::= 330

id-PagingCause										ProtocolIE-ID ::= 331

id-SecurityIndication									ProtocolIE-ID ::= 332

id-SecurityResult										ProtocolIE-ID ::= 333

id-E-RABSecurityResultItem							ProtocolIE-ID ::= 334

id-E-RABSecurityResultList							ProtocolIE-ID ::= 335

id-RAT-Restrictions									ProtocolIE-ID ::= 336

id-UEContextReferenceatSourceeNB					ProtocolIE-ID ::= 337

id-LTE-NTN-TAI-Information  						ProtocolIE-ID ::= 33

8

END

-- ASN1STOP

>>> END OF CHANGE <<<